3GPP TSG-RAN WG3 Meeting #125bis	R3-245277

Hefei, China, 14 – 18 October, 2024

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.300) MRO Enhancements for LTM and CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM and CHO with candidate SCG(s) in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #125 [1]:

- Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

- Take the existing detection mechanism descriptions as baseline.

- For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

- CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

The agreements achieved in RAN3 #124 [2]:

- RAN3 to prioritize MCG LTM over SCG LTM.

- RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

- RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

The agreements achieved in RAN2 #127 [3]:

- Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

- -introduce a new field in RLF report to indicate the LTM recovery cell id.

- RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

- We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

- RA-based access will not be a new triggering condition for the SHR report.

- HO interruption time will not be a new triggering condition for the SHR report

- Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

- We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

- We will not log interruption time in SON reports (e.g. SHR) for LTM.

- We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

- Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

The agreements achieved in RAN2 #126 [4]:

- We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

- Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures.

- Log the LTM cell ID upon performing recovery an LTM candidate cell.

- Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

### 2.1.2 Near Failure Monitoring for MCG LTM

Main benefits of the LTM shall be seen in avoiding the complete reset of L3, avoiding RACH procedure which shall result into reduction of the interruption time. The interruption time is significantly impacted with RACH procedure. In case the TA of the candidate target LTM cell is not included into Cell Switch Command (MAC CE) the UE will initiate the legacy RACH procedure to access the target candidate cell. Despite the procedure can be monitored by means of T304 timer, but the quality in terms of having it in a RACH-less manner is not considered. However, it is essential to get information whether the cell change occurred RACH-less which is key for LTM. It resulted into proposal to define new SHR triggers specific to LTM near failure cases related to RACH-based LTM by some of companies. However, during the RAN2 # 126 meeting it was agreed not to use this as a new trigger event for SHR for LTM. As we think this conclusion in RAN2 was made quickly and not considering all the aspects we propose to discuss it again in RAN3.

Proposal 1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

On the other hand, in case the RACH-less cell change is triggered, i.e. MAC-CE msg contains TA, the first UL transmission is done in RLC AM. So, it may require several RLC retransmissions to get the transmission through.

Proposal 2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

### 2.1.4 Connection Failure Monitoring for MCG LTM

#### 2.1.4.1 CU and DU LTM Optimization

During the RAN3 # 124 meeting the following was agreed:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

During the RAN3 # 125 meeting the following was agreed:

- For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

- CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

Based on the above agreements the gNB-CU is responsible for optimizing LTM candidate cells. Two options are possible to optimize the LTM candidate cell list:

- UE based solution

- Network based solution

LTM compared to CHO supports subsequent LTM and therefore the UE stores LTM configuration also after successful LTM execution. UE can therefore report LTM candidate cell list also in scenario when RLF occurred shortly after successful LTM execution.  On the other hand, as Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

Observation 1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

As the info on LTM candidate cells is available in the network (gNB-CU) it is proposed not to ask UE to monitor this information.

Proposal 3: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list.

Also based on the above agreements the gNB-DU determines the target cell to which LTM is executed and the timing of LTM execution (e.g., based on L1 measurements). The gNB-DU is intended to provide such optimization based on RLF report reported from gNB-CU. There is already ACCESS and MOBILITY INDICATION sent from gNB-CU to gNB-DU which includes the RLF Report Container and will automatically include also the enhancements of the RLF Report in Rel-19 for LTM which was agreed in RAN3 # 125 meeting.

There may be plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which oversees source DU. Despite the TA was reported in Cell Switch Command, but it failed to attempt the target LTM cell and had to switch RACH based access maybe an info which may be reported in the RLF report.

Proposal 4: RAN3 agrees the study to identify issues related to Early TA Acquisition procedure and on the extension of the parameters in RLF like invalid or missing TA.

#### 2.1.4.2 Stage 2 on Connection failure due to intra-system mobility for LTM

RAN2 has already agreed on failure and near failure scenarios already in RAN2#125bis :

For LTM MRO, RAN2 considers the following three connection failure cases:

-	Too late LTM

-	Too early LTM

-	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

-	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

-	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

-	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

-	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

-	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

-	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Minor corrections and rewording could be applied to the scenarios to reflect the fact that only the cell where the UE initially attempts the radio connection re-establishment to, either via re-establishment or LTM recovery, is of interest for root cause analysis. More specifically, the scenarios definitions could be updates as proposed already in                R2-2408380.

A more correct description of the LTM scenarios would be as follows:

Too Late LTM

Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell and attempts LTM recovery, which fails, then attempts re-establishment .

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

Too Early LTM

Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

Case 2b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell which is the initial source cell and attempts LTM recovery, which fails, then attempts re-establishment..

Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell, which is also an LTM candidate cell.

LTM to wrong cell

Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell other than source or target cell and attempts LTM recovery, which fails, and then attempts re-establishment .

Case 3c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

Based on these scenarios, the detection mechanism in 38.300 could be updated as follows:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/ the cell UE attempts LTM recovery  is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

Proposal 5: RAN3 agrees the TP for BL CR for TS 38.300.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #125 [1]:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

The agreements achieved in RAN3 #124 [2]:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

The agreements achieved in RAN2 #127 [3]:

UE includes following information in RLF report:

Time information regarding condition fulfilment for CHO with candidate SCGs. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

Measurement results of PCells and PSCells.

The agreements achieved in RAN2 #126 [4]:

- RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

### 2.2.3 Time Between Fulfilment of the Associated Conditions

During the RAN3 # 125 meeting all the companies agreed the following:

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

In case of CHO with candidate SCG(s), it may happen that if one of the execution conditions is not configured properly, the entire execution will be delayed as the configuration cannot be executed partially. Even the CHO with candidate SCG(s) is at the end successfully completed, the delayed execution may have negative impact on end user when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier. If execution completion fails to either both or one of them the end user will perceive it as a complete drop or as degradation of provided QoS in case of SCG execution completion failure. Even there is a general agreement between the companies on the need to monitor the time between fulfilment the associated conditions the details of such monitoring and optimization steps to reduce this time have not been discussed yet. In the remaining part of this chapter, we focus on near failure monitoring as it is more probable having both conditions fulfilled in this scenario than in connection failure scenario.

Example 1

Let us consider a UE is connected to PCell 1 and PSCell 1. The UE is configured with CHO with candidate SCG(s) configuration PCell 2 and PSCell 2.  Let’s further consider the UE starts evaluating the execution conditions of candidate PCell 2 and PSCell 2 simultaneously and after some time the CPC execution conditions of the candidate PSCell 2 are fulfilled first and then with some delay also the CHO execution conditions for candidate PCell 2 are fulfilled. Then for near failure monitoring from end user perspective when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier we see the following possible scenarios. The scenarios primarily focus on CHO with CPC. It is mentioned where it is applicable also for CHO with CPA.

1a) during the wating for fulfilment of the CHO execution conditions, the T310 on SCG of PSCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

1b) the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

1c) during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and also T310 on MCG for PCell 1 is started. The T310 time value both for MCG and SCG is above the configured T310 threshold to trigger the SHR and SPR, respectively. This case is applicable only for CPC not for CPA.

1d) during the wating for fulfilment of the CHO execution conditions the T310 on MCG for PCell 1 is started. The T310 is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

1e) during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable.

Observation 2: The scenario 1a when during the wating for fulfilment of the second CHO execution conditions the T310 on SCG of PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR may require the optimization of the CHO execution conditions.

Observation 3: The scenario 1b when the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SPR requires the legacy optimization of the CPAC execution conditions.

Observation 4: The scenario 1c when during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR and SHR may require the optimization of the CHO execution conditions.

Observation 5: The scenario 1d when during the wating for fulfilment of the CHO execution conditions T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR requires the optimization of the CHO execution conditions.

Observation 6: The scenario 1e when during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Example 2

Let us consider a UE is connected to PCell 1 and PSCell 1. The UE is configured with CHO with candidate SCG(s) configuration PCell 2 and PSCell 2.  Let’s further consider the UE starts evaluating the execution conditions of candidate PCell 2 and PSCell 2 simultaneously and after some time the CHO execution conditions of the candidate PCell 2 are fulfilled first and then with some delay also the CPC execution conditions for candidate PSCell 2 are fulfilled. Then for near failure monitoring from end user perspective when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier we see the following possible scenarios. The scenarios primarily focus on CHO with CPC. It is mentioned where it is applicable also for CHO with CPA.

2a) during the wating for fulfilment of the CPAC execution conditions the T310 on MCG of PCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

2b) The T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. The T310 time value is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

2c) during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and also T310 on SCG for PSCell 1 is started. The T310 time value both for MCG and SCG is above the configured T310 threshold to trigger the SPR and SHR, respectively. This case is applicable only for CPC not for CPA.

2d) during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

2e) during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable.

Observation 7: The scenario 1a when during the wating for fulfilment of the second CPAC execution conditions the T310 on MCG of PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR may require the optimization of the CPAC execution conditions.

Observation 8: The scenario 2b when the T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SHR requires the legacy optimization of the CHO execution conditions.

Observation 9: The scenario 2c when during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR and SPR may require the optimization of the CPAC execution conditions.

Observation 10: The scenario 2d when during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR requires the optimization of the CPAC execution conditions.

Observation 11: The scenario 2e when during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Note: In some of the above scenarios for simplicity only T310 is considered but it may be extended also to T312.

Proposal 6: RAN3 agrees to consider the above scenarios 1a,b,c,d,e and 2a,b,c,d,e as a baseline to study improvement of the UE reports for near failure monitoring for UEs with CHO with Candidate SCG(s) only configured.

### 2.2.4 Connection Failure Monitoring for CHO with Candidate SCG(s)

#### 2.2.4.1 Optimization of the association of the PCell and PSCell

During the RAN3 # 125 meeting the following possible new IEs of the RLF report were marked as FFS:

The RLF report includes:

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

In Rel-18, CHO with candidate SCG(s) was specified for NR-DC and the following is captured in 37.340:

- A CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met.

- The UE starts evaluating the execution conditions for candidate PCell(s) and candidate PSCell(s) simultaneously upon receiving the CHO with candidate SCG(s) configuration

- UE stops evaluating the execution conditions once a PCell change or a PSCell change is triggered.

- The UE does not execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met

Further the following is captured in 38.331 as an extraction from the Section 5.3.5.13.5

------

- The UE shall:

1>           if more than one pair of triggered PCell and associated triggered PSCell exist:

2>           select one of the triggered PCell(s) and the associated triggered PSCell(s) as the selected cells for conditional reconfiguration execution;

1>           else if only one pair of triggered PCell and associated triggered PSCell exists:

2>           consider the triggered PCell and the associated triggered PSCell as the selected cells for conditional reconfiguration execution;

-------

The UE may experience a failure (either RFL or SCG failure) while being configured with CHO with candidate SCG(s), more specifically one of the following scenarios may happen:

- CHO execution condition of a candidate PCell is fulfilled but CPAC execution condition of the associated candidate PSCell is not fulfilled or vice versa; or,

- neither CHO execution condition of a candidate PCell is fulfilled nor CPAC execution condition of the associated candidate PSCell is fulfilled.

- Both CHO execution condition of a candidate PCell and CPAC execution condition of the associated candidate PSCell are fulfilled but consequent execution completion will fail to either both or one of them

- Both CHO execution condition of a candidate PCell and CPAC execution condition of the associated candidate PSCell are fulfilled, consequent execution completion for both is successful but shortly after that there is SCG or MCG failure or both.

These failure scenarios above are now captured by the agreed cases [R3-244223]: scenario a) by cases 2 and 3, scenario b) by case 1, scenario c) by case 7 and scenario d) by case 8 all of which have been agreed to be in scope of the WI.

Observation 12: For the case 7 and 8 when both conditions from the same association are fulfilled before the RLF is encountered either during the execution phase or shortly after execution completion as “Identifier of candidate PCell and PSCell” the ones of the pair selected by UE based on 38.331 shall be reported within the RLF report or SCGFailureInformation message.

Proposals 7: RAN3 agrees for the case when both conditions are fulfilled from the same association before the RLF is encountered either during the execution phase or shortly after execution completion to report within the RLF report or SCGFailureInformation message as “Identifier of candidate PCell and PSCell” the ones of the pair selected by UE based on 38.331.

Cases 2 and 3 may be further divided based on Qualcomm R3-244430 in RAN3 # 125 meeting into two sub-cases:

- There is not any other PSCell/PCell of another association with fulfilled CPAC/CHO execution conditions. For example, for case 2, UE is configured with the CHO with candidate SCG configurations i.e., (PCell X1, PSCell Y1), (PCell X1, PSCell Y2), (PCell X2, PSCell Y3). Let’s consider that CHO execution conditions for PCell X1 were fulfilled but no one of other associations PSCell Y2 and PSCell Y3 were fulfilled.

- There is at least one other PSCell/PCell of another association with fulfilled CPAC/CHO execution conditions. For example, for case 2, UE is configured with the CHO with candidate SCG configurations i.e., (PCell X1, PSCell Y1), (PCell X1, PSCell Y2), (PCell X2, PSCell Y3). Let’s consider that CHO execution conditions for PCell X1 were fulfilled and at least one of other associations PSCell Y2 and PSCell Y3 were fulfilled. Let us consider both were fulfilled as an example.

For the sub-case II) it may be beneficial for operator to identify such scenario. However based on example, reporting it as a list of PSCell Y2 and Y3 identifiers with fulfilled CPAC execution conditions from another associations may not be that beneficial as some selection within them is needed for proper optimization. Instead to provide PSCell Y2 and Y3 related measurements within the RLF report or SCGFailureInformation message is recommended. Based on the measurements the operator may be able to select the PSCell Y2 and Y3 with fulfilled CPAC execution conditions from associations different from the association of the PCell X1 in the proper order. Based on that the operator may optimize the execution conditions meaning that in the association with the fulfilled CHO execution conditions for PCell X1 the PSCell Y1 may be replaced with PSCell Y2 and/or PSCell Y3.

Observation 13: For the case when CHO/CPAC execution condition of a candidate PCell/PSCell is fulfilled but CPAC/CHO execution condition of the associated candidate PSCell/PCell is not fulfilled but there is other PSCell(s)/PCell(s) from different association(s) with fulfilled execution conditions it is recommended to report PSCell(s)/PCell(s) measurements within the RLF report or SCGFailureInformation message.

Proposals 8: RAN3 agrees for the case when CHO/CPAC execution condition of a candidate PCell/PSCell is fulfilled but CPAC/CHO execution condition of the associated candidate PSCell/PSCell is not fulfilled but there is other PSCell(s)/PSCell(s) from different association(s) with fulfilled execution conditions to report PSCell(s)/PCell(s) measurements within the RLF report or SCGFailureInformation message. Addition of such measurements already agreed by RAN2.

#### 2.2.4.2 Study on failure cases of CHO with Candidate SCG(s)

For all the agreed failure cases, the RLF and/or SCGFailureInformation could be enhanced with additional information for root cause analysis.

For cases 1, 2 and 3, depending on which leg experienced the failure, the UE may log the following information in the RLF report/ SCGFailureInformation message:

- An indication of the first fulfilled event /execution condition (CHO or CPAC) (if any) – this is fully in accordance with current RAN2 and RAN3 agreements

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not – this will help the network identify how close to fulfillment the event was and gives a good indication on possible corrective actions

Proposal 9: In case RLF on MCG or SCG is detected before execution of CHO w/ candidate SCG, the RLF report or SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event /execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

For cases 7 and 8, where failure is experienced during HO execution or shortly after successful execution, additional information regarding the order and time between execution condition fulfilment may aid the network in determining the appropriate corrective actions. If the CHO execution condition was fulfilled first, while waiting for CPAC execution condition to also be fulfilled (especially if for a long time), may lead to artificially selecting the wrong PCell. In this case the CPAC execution condition needs to be corrected. If the situation is reversed and the CPAC execution condition is fulfilled first and UE has to wait for a long time until CHO execution condition is also fulfilled before execution the handover, it is clear that this is a CHO execution condition parametrization issue.

Proposal 10: In case HOF during execution or RLF shortly after the successful execution of CHO w/ candidate  is detected on the MCG or SCG, the RLF report/ SCGFailureInformation  should be enhanced with the following information:

- An indication of the first fulfilled event /execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

The last agreed case, case 9, assumes RLF on MCG or SCG is experienced due to the execution conditions for candidate PCell and the associated candidate PSCell not being met simultaneously. We believe that such a case where there is toggling of the fulfilment on one or both of the conditions is very unlucky (seems to indicate spotty coverage) and can be in the end reduced to case 2 or 3 depending on the final state of the two conditions before RLF detection. It is unclear if more in depth knowledge on the history of the fulfilment status of the conditions would be of use to the network or if a different corrective action than in case 2/3 can be deduced.

Observation 14: Case 9 can be in fact reduced to case 2 or 3 and does not seem to offer any significant useful insight to the network.

Proposal 11: Remove case 9 from consideration as it can be covered by case 2 and 3.

# 3	Conclusions

We have the following observations and proposals:

Proposal 1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

Proposal 2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

Observation 1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

Proposal 3: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list.

Proposal 4: RAN3 agrees the study to identify issues related to Early TA Acquisition procedure and on the extension of the parameters in RLF like invalid or missing TA.

Proposal 5: RAN3 agrees the TP for BL CR for TS 38.300.

Observation 2: The scenario 1a when during the wating for fulfilment of the second CHO execution conditions the T310 on SCG of PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR may require the optimization of the CHO execution conditions.

Observation 3: The scenario 1b when the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SPR requires the legacy optimization of the CPAC execution conditions.

Observation 4: The scenario 1c when during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR and SHR may require the optimization of the CHO execution conditions.

Observation 5: The scenario 1d when during the wating for fulfilment of the CHO execution conditions T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR requires the optimization of the CHO execution conditions.

Observation 6: The scenario 1e when during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Observation 7: The scenario 1a when during the wating for fulfilment of the second CPAC execution conditions the T310 on MCG of PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR may require the optimization of the CPAC execution conditions.

Observation 8: The scenario 2b when the T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SHR requires the legacy optimization of the CHO execution conditions.

Observation 9: The scenario 2c when during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR and SPR may require the optimization of the CPAC execution conditions.

Observation 10: The scenario 2d when during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR requires the optimization of the CPAC execution conditions.

Observation 11: The scenario 2e when during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Proposal 6: RAN3 agrees to consider the above scenarios 1a,b,c,d,e and 2a,b,c,d,e as a baseline to study improvement of the UE reports for near failure monitoring for UEs with CHO with Candidate SCG(s) only configured.

Observation 12: For the case c) and d) when both conditions are fulfilled before the RLF is encountered either during the execution phase or shortly after execution completion as “Identifier of candidate PCell and PSCell” the ones of the pair selected by UE based on 38.331 shall be reported within the RLF report.

Proposals 7: RAN3 agrees for the case when both conditions are fulfilled before the RLF is encountered either during the execution phase or shortly after execution completion the report within the RLF report as “Identifier of candidate PCell and PSCell” the ones of the pair selected by UE based on 38.331.

Observation 13: For the case when CHO/CPAC execution condition of a candidate PCell/PSCell is fulfilled but CPAC/CHO execution condition of the associated candidate PSCell/PCell is not fulfilled but there is other PSCell(s)/PCell(s) from different association(s) with fulfilled execution conditions it is recommended to report PSCell(s) and PCell(s) measurements within the RLF report or SCGFailureInformation message.

Proposals 8: RAN3 agrees for the case when CHO/CPAC execution condition of a candidate PCell/PSCell is fulfilled but CPAC/CHO execution condition of the associated candidate PSCell/PSCell is not fulfilled but there is other PSCell(s)/PSCell(s) from different association(s) with fulfilled execution conditions to report PSCell(s and /PCell(s) measurements within the RLF report or SCGFailureInformation message. Addition of such measurements already agreed by RAN2.

Proposal 9: In case RLF on MCG or SCG is detected before execution of CHO w/ candidate SCG, the RLF report or SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event /execution condition (CHO or CPAC) (if any)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

Proposal 10: In case HOF during execution or RLF shortly after the successful execution of CHO w/ candidate  is detected on the MCG or SCG, the RLF report/ SCGFailureInformation  should be enhanced with the following information:

- An indication of the first fulfilled event /execution condition (CHO or CPAC)

- Time duration between the two execution conditions being fulfilled

Observation 14: Case 9 can be in fact reduced to case 2 or 3 and does not seem to offer any significant useful insight to the network.

Proposal 11: Remove case 9 from consideration as it can be covered by case 2 and 3.

# References

[1] Report of 3GPP TSG RAN3 meeting #125

[2] Report of 3GPP TSG RAN3 meeting #124

[3] Report of 3GPP TSG RAN2 meeting #127

[4] Report of 3GPP TSG RAN2 meeting #126

# 4	TP for TS 38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

First change, ommited text not changed

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/ the cell UE attempts LTM recovery  is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/ the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM  execution until connection failure .

Remaining text not changed