3GPP TSG-RAN WG3 #125bis	R3-245280

He Fei, China, 14th – 18th October 2024

Agenda item:		13.2 Support for inter-CU LTM

Title:		discussion of Rel-19 inter-CU LTM issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, we continue to discuss and proposal to progress the project.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-242356 (target: RAN #109) [TU: 1 ( 1, 1, 1, 1)] QUOTA: 3

13.1. General Time plan, skeletons, BLs

13.2. Support for inter-CU LTM Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: As secondary priority, support the case where CU is acting as SN and MN is unchanged As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. RAN3#125: Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure

2. Discussion

2.1	LTM Preparation

2.1.2	LTM configuration update for initial preparation

Question: Handover request with single LTM candidate cell or multiple LTM candidate cells?

For the inter-gNB-CU LTM, the source gNB-CU initiates Handover Request message per LTM candidate cell to the candidate gNB-CU, the candidate gNB-CU then sends the F1: UE Context Setup Request message (as per cell) to the candidate gNB-DU. After admission control and resource configuration in the candidate gNB-DU, the F1: UE Context Setup Response message is sent from the candidate gNB-DU, the candidate gNB-CU then responds with a Handover Request Acknowledge message with the LTM configuration of the candidate cell to the source gNB-CU.

It will be simple and natural to follow the Rel-18 Intra CU LTM in F1AP UE Context Setup procedure, i.e. per cell per message.

Proposal 1: The Handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

LTM Configuration ID of target candidate cell

For the inter-CU LTM, the configuration information exchanged between the source CU to the target CU, here we need to confirm the LTM Configuration ID is allocated by the source CU or target CU. The LTM Configuration ID is the RRC LTM-CandidateID which indicates the index of the candidate configuration of the target cell, it may be easy just for the source CU to allocate the LTM Configuration ID and the target CU simply follow and then forward to the candidate cells in the target DUs. This is because if the candidate CU assigns the LTM configuration ID then it will cause a unnecessary confirmation in the source gNB e.g. to align with other candidate gNB.

Proposal 2: To confirm that the LTM Configuration ID is allocated by the source CU and transfer from the source CU to candidate CU, target CU simply forward to the candidate cells in candidate DUs.

2.1.2	LTM configuration update for initial preparation

Modification from source CU to candidate CU:

There is a two round trip request message between source gNB and the candidate gNB(s) for the initial preparation, the first round is the to request the LTM candidate cell configuration and the second round is to update the CSI related configurations if some of the candidate cell(s) reject the LTM request.

For the first-round request message, RAN3 already agreed to reuse the current Handover Request message and Handover Request Acknowledge message.

For the second-round request message, RAN3#125 has agreed to “Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.”

Proposal 3: TP for new LTM Configuration Update is proposed.

LTM Modification from candidate CU to source CU:

Whether need to have modification from candidate CU? One motivation of the LTM modification from candidate CU to source CU before LTM Execution phase, may be because the situation in candidate is changed e.g. the load status of the candidate cell becomes worse. If this will be main and only motivation, then the candidate CU can trigger cancel request towards the source CU. Consider the LTM is controlled mainly in source CU, we would think there is no need for Modification from candidate CU to source CU.

Proposal 4: No need to introduce LTM modification from candidate CU to source CU, if only request to release the prepared cells, the candidate CU can trigger cancel request towards the source CU.

2.2	LTM Execution

2.2.1	LTM Cell Switch Notification message

It has been agreed in RAN3#124 to introduce LTM Cell Switch Notification procedure, and in RAN3#125 it was agreed to change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

2.2.2	TA Information Transfer message

In the RAN3#124 meeting, it was already agreed to introduce a new procedure on Xn interface to transfer the TA information. But whether it is a UE associated message or non-UE associated message is still FFS, and in RAN3#125 meeting there is still no consensus. A simple way is to align with Rel-18 LTM in F1, no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 5: in order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

2.3	Candidate-initiated LTM Cancellation

For the Source-initiated LTM Cancellation, in RAN3#124 is has been agreed to reuse the Handover Cancel message for the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

For the Candidate-initiated LTE Cancellation, in RAN3#125 meeting it was agreed to have “Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation.”, but for the signalling message, we need to “Down select from Option1 and Option3”

- Option 1: Reuse CHO Cancel

- Option 2: Rename CHO Cancel

- Option 3: Introduce new procedure

The motivation to have LTM cancellation from candidate CU to source CU, may be because the situation in candidate is changed e.g. the load status of the candidate cell becomes worse and becomes no further capacity for LTM purpose.

If existing signalling procedure can be reused, it would be preferable to do so. The existing Conditional Handover Cancel procedure is not so complicated, which is easy to be reused.

Proposal 6: to use existing Conditional Handover Cancel procedure for LTM cancel from candidate CU to source CU.

2.4	LTM Release

RAN3#124 meeting has the following FFS: FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Last RAN3#125 this issue was not treated.

The condition of this issue is after the LTM Completion Phase i.e. after the UE has been cell switched to target CU, the ex-serving cells (of the ex-serving CU) can be kept for subsequent LTM. However at the time the UE successfully access to the new serving cell, if the new serving CU decides that there is no need to keep the ex-serving CU, whether to use UE Context Release message?

For the legacy handover procedure, after the target gNB has completed Path Switch towards CN, it initiates UE Context Release towards the source gNB. If we will follow this procedure sequence, then it would be natural to use UE Context Release procedure. There is also one other possibility to use Handover Cancel procedure later. We simply just need to decide one principle, e.g. Handover Cancel procedure is used for the source gNB (or source CU) to cancel some of prepared Cells that was requested by the source gNB, the UE Context Release procedure is used to release the UE context of a gNB (i.e. release all prepared cells).

Proposal 7: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

2.5	Support of subsequent LTM

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, and data forwarding addresses, etc) may need to sync up among candidate gNBs.

In Rel-18 intra-CU inter-DU LTM in F1, the following note is captured in stage 2 (38.401):

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

For the Rel-19 inter-CU LTM, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via LTM Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

Considering to align with Rel-18 Intra-CU case that “Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.”, it would be then feasible to do the same, i.e. all three Option 1, 2 and 3 may be done by implementation.

Proposal 8: To align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3 may be done by implementation.

3. Conclusion and proposals

Proposal 1: The Handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

Proposal 2: To confirm that the LTM Configuration ID is allocated by the source CU and transfer from the source CU to candidate CU, target CU simply forward to the candidate cells in candidate DUs.

Proposal 3: TP for new LTM Configuration Update is proposed.

Proposal 4: No need to introduce LTM modification from candidate CU to source CU, if only request to release the prepared cells, the candidate CU can trigger cancel request towards the source CU.

Proposal 5: in order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 6: to use existing Conditional Handover Cancel procedure for LTM cancel from candidate CU to source CU.

Proposal 7: To reuse the existing XnAP UE Context Release procedure at the source gNB if no LTM candidate cell(s) exist.

Proposal 8: To align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3 may be done by implementation.

# References

[1] RP-242356 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-244718, Summary of offline for R19 LTM procedure

[3] R3-244855, (BL CR to 38.300) Inter-CU LTM basic procedure

Annex: ONLY for Reference.

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s).

Editor’s Note: FFS on whether one candidate cell or a list of candidate cells is contained in the message.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB.

6.	The source gNB requests LTM configuration update to the candidate gNB(s).

Editor’s Note: Details are FFS on step 6.

7.	The candidate gNB(s) sends the response (LTM CONFIGURATION UPDATE ACKNOWLEDGE) to the source gNB.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early TA acquisition to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB.

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s Note: Details are FFS on above note.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Details are FFS on step 20.

TS38.423 8.2.9 Conditional Handover Cancel 8.2.9.1 General The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover or an already prepared conditional reconfiguration. The procedure uses UE-associated signalling. 8.2.9.2 Successful Operation Figure 8.2.9.2-1: Conditional Handover Cancel, successful operation The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover or the conditional reconfiguration by means of an appropriate cause value. At the reception of the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved for candidate cells associated to the UE-associated signalling identified by the Source NG-RAN node UE XnAP ID IE and the Target NG-RAN node UE XnAP ID IE. If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released. If the Conditional Reconfigurations To Be Cancelled List IE is included in the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall, if supported, consider that only the resources reserved for the cell(s) identified by the included NG-RAN CGI(s) are about to be released. 8.2.9.3 Unsuccessful Operation Not applicable. 8.2.9.4 Abnormal Conditions If the CONDITIONAL HANDOVER CANCEL message refers to a context that does not exist, the source NG-RAN node shall ignore the message. If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the CONDITIONAL HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the source NG-RAN node shall ignore those non-associated candidate cells. 9.1.1.13 CONDITIONAL HANDOVER CANCEL This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared conditional handover or an already prepared conditional reconfiguration. Direction: target NG-RAN node source NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; ignore. Source NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source NG-RAN node.; YES; reject. Target NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the target NG-RAN node.; YES; reject. Cause; M; 9.2.3.2; YES; ignore. Candidate Cells To Be Cancelled List; 0 .. <maxnoofCellsinCHO>; YES; reject. >Target Cell ID; M; Target Cell Global ID 9.2.3.25; –. Conditional Reconfigurations To Be Cancelled List; 0..1; Includes CHO-only (with or without SCG) and CHO with candidate PSCell(s); YES; reject. >Conditional Reconfigurations To Be Cancelled Item; 1 .. < maxnoofPSCellCandidates>; –. >>Target PCell ID; M; Target Cell Global ID 9.2.3.25; –. >>Target PSCell ID; O; NR CGI 9.2.2.7; If this IE is absent, indicates CHO only (with or without SCG); –. Range bound; Explanation. maxnoofCellsinCHO; Maximum no. cells that can be prepared for a conditional handover. Value is 8.. maxnoofPSCellCandidates; Maximum no. conditional reconfigurations that the UE can store. Value is 8. The total of maxnoofCellsinCHO and maxnoofPSCellCandidates should not exceed 8..