3GPP TSG RAN WG3 #125bis					                 R3-245283

Hefei, China, October 14th – 18th, 2024

Source:	NEC

Title:	Discussion on CLI Handling Support

Agenda item:	19.2

Document for:   Discussion and decision

1 Introduction

In the RAN #102 meeting, a WI on the evolution of NR duplex operation was approved. The objectives of the WI related to RAN3 work are listed as follows [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

The following RAN1 agreement was reached in the RAN1#118 meeting, a LS is coming to RAN3 [2].

Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB. Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

This contribution provides NEC views on information exchange among gNBs for CLI mitigation.

2 Discussion

As per the agreement given by RAN1, the information below may be exchanged among gNBs for gNB-gNB CLI handling through Xn and between gNB-CU and gNB-DU through F1:

- Semi-static cell-specific SBFD time and frequency location configuration

- Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Strongest DL beam information

- CLI-mitigation request

2.1 gNB-to-gNB over Xn

The procedure and workflow for gNB -gNB CLI mitigation and handling can be described as follows:

- Step 1: gNB1 sends semi-static cell-specific SBFD time and frequency location configuration information and measurement resource configuration information, i.e., SSB and/or periodic NZP CSI-RS, to gNB2.

- Step 2: Upon reception of NZP CSI-RS /SSB, gNB2 completes gNB1-gNB2 CLI measurement and feedback on the measurement result, such as strongest DL beam information, or sends CLI-mitigation request through backhaul (i.e., Xn) to gNB1. This may also happen further down via F1 from gNB-CU to gNB-DU under gNB-DU1 in split architecture. Alternatively, gNB2 applies a CLI handling scheme according to the CLI measurement result, e.g., power boosting, changing the transmission resource/direction, or transmitted traffic type/signals.

- Step 3(optional): gNB1 applies an interference mitigation scheme, such as power backoff, scheduling adjustment after receiving the strongest DL beam information, or CLI-mitigation request.

Figure 1: Signalling of CLI mitigation over Xn.

Proposal 1:

Some information may be exchanged among gNBs for gNB-gNB CLI handling through Xn , and the information includes at least one of the following:

- Semi-static cell-specific SBFD time and frequency location configuration

- a set of one or more periodic NZP CSI-RS resources or SSB

- Strongest DL beam information(a CSI-RS Resource index or an SSB index)

- CLI-mitigation request

The same information is also exchange between gNB-CU and gNB-DU through F1.

2.2 Signalling design and for CLI Handling

At least the information for CLI handling between gNB and gNB, as in proposal 1, needs to be exchanged over Xn through Non UE Associated signalling messages. It needs to be evaluated whether existing Xn signalling procedure e.g. NG-RAN node Configuration Update, or Resource Statis Reporting Initiation / Reporting procedures can be reused. In a different approach, it would be easier to introduce new Xn signalling procedures as it will also limit impact on other existing function. Similarly to F1 signalling procedure in CU-DU split architecture.

Proposal 2:

For the CLI handling, it is proposed to further discuss for the information exchange for CLI handling over Xn between gNBs and F1 between CU and DU.

2.3 Information Exchange for CLI Handling

As mentioned in Proposal 1 above, those information exchanges for CLI handling are further discussed.

2.2.1 Semi-static cell-specific SBFD time and frequency location configuration

This IE contains the SBFD time and frequency location configuration information used by gNB for CLI mitigation, and it is necessary for a gNB to do CLI measurements. The SBFD time and frequency location configuration IE includes the configuration of SBFD symbols position and frequency locations of the SBFD UL subband and DL subband. For the configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are included

-	A starting slot index

-	A starting symbol index within the starting slot

-	An ending slot index

-	An ending symbol index within the ending slot

For the configuration of frequency locations of the SBFD UL subband and DL subband, starting RB and bandwidth for each subband can be configured. One or two SBFD DL subbands can be configured for a cell. This information can be the same as what is configured for UE by RRC.

If the neighboring gNB configuration has the same SBFD time and frequency location as UE, the gNB-to-gNB CLI can be avoided as the uplink and downlink transmission scheduling of the two gNBs are the same.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SBFD time location configuration | M |  | 9.3.3.x | This IE indicates the SBFD subband time position. |
| SBFD frequency location configuration | M |  | 9.3.3.x | This IE indicates the SBFD subband frequency position. |

2.2.2 Measurement resource configuration

For gNB-gNB CLI mitigation and handling support, CLI measurement resource configuration IE can be defined, and it is used by the gNB to complete inter-gNB CLI measurement. The candidate values can be a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) or SSB resources (relevant IE in 38.331 is SSB-Configuration).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Measurement resource configuration | O |  | ENUMERATED (a set of one or more periodic NZP CSI-RS resources or SSB) | The RS resource for gNB-gNB CLI measurement |

2.2.3 Strongest DL beam information

The strongest DL beam information IE can be defined, and it is used by the gNB to handle inter-gNB CLI. The candidate value can be a CRI or an SSB index. NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. The SSB index is an absolute index value among the SSB(s) for which the configuration is provided to a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Strongest DL beam information | O |  | ENUMERATED (a CSI-RS Resource index or an SSB index) | Strongest DL beam information |

2.2.4 CLI-mitigation request

CLI-mitigation request IE can be defined. It is used by the victim gNB to send a CLI-mitigation request to the aggressor gNB to do CLI-mitigation. The aggressor gNB may choose to respond to said request accordingly.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI-mitigation request | O |  | 9.3.3.x | request CLI-mitigation |

Proposal 3:

RAN 3 should consider including the listed IEs for gNB-gNB CLI handling or mitigation.

3 Conclusion

Based on the discussion in the previous sections, this contribution is concluded with the following proposals:

Proposal 1:

Some information may be exchanged among gNBs for gNB-gNB CLI handling through Xn or F1, and the information includes at least one of the following:

- Semi-static cell-specific SBFD time and frequency location configuration

- a set of one or more periodic NZP CSI-RS resources or SSB

- Strongest DL beam information(a CSI-RS Resource index or an SSB index)

- CLI-mitigation request

Proposal 2:

For the CLI handling, it is proposed to further discuss for the information exchange for CLI handling over Xn between gNBs and F1 between CU and DU.

Proposal 3:

RAN 3 should consider including the listed IEs for gNB-gNB CLI handling or mitigation.

4 References

[1] RP-234035, ‘New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’ CMCC.

[2] R3-245006 (R1-2407533)	LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation