3GPP TSG-RAN WG3 Meeting #125-bis	R3-245293

Hefei, China, 14th – 28th October 2024

Agenda Item:		11.2

Source:		NEC

Title:	(TP to TS38.423) General considerations on Enhancements for AI/ML for NG-RAN

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a new WI on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN was approved based on the study results from SI and the detailed objectives are captured as below [1]:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

In this contribution, we provide our considerations on the work tasks in this new WI, including stage 2 impact, stage 3 impact and open issues.

# Discussion

During the study phase on Enhancements for AI/ML for NG-RAN, two new use cases i.e. Slicing and CCO were studied, including deployment scenario, input/output/feedback and potential standard impact. Besides, three Rel-18 leftovers i.e. Mobility optimization for NR-DC, Split architecture support and Continuous MDT collection, were justified by the study and will continue the potential solutions discussion.

The study conclusions were captured in TR38.743 [2] and we should take study results as baseline during the normative phase.

## 2.1	Stage 2 impact

As captured in the new WID [1], AI/ML-based Slicing and AI/ML based CCO are agreed to be specified in Rel-19, so TS38.300 needs to be updated to include these two new use cases for AI/ML for NG-RAN.

Proposal 1: Add the above two new use cases, i.e. Slicing and CCO, in the stage 2 specification TS38.300.

Regarding the locations for AI/ML Model Training and AI/ML Model Inference, besides what we already agreed in Rel-18, the following were also agreed to be possible for CU-DU architecture:

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP).

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

Proposal 2: To add the above deployment solutions in the stage 2 specifications, including TS38.300 and TS38.401.

For the two Rel-19 new use cases, compared with what was specified during Rel-18, the following information are listed to be exchanged over Xn interface.

AI/ML based Network Slicing

- Predicted radio resource status per slice

- Predicted slice available capacity

AI/ML based CCO

- Future CCO State together with an associated coverage modification cause

- Timing Information

Proposal 3: To add the above new information that can be configured to be reported by an NG-RAN node in the stage 2 specification TS38.300.

Draft CRs to TS38.300 and TS38.401, mirroring the proposals above, are available in R3-245294 and R3-245295 respectively.

Proposal 4: To agree the draft CRs on Enhancements for AI/ML for NG-RAN to TS38.300 in R3-245294 and to TS38.401 in R3-245295.

## 2.2	Stage 3 impact

As agreed in the study phase, for the two new use cases, the existing procedures (Data Collection Reporting Initiation procedure and Data Collection Reporting procedure) will be reused and enhanced to collect predicted information between gNBs. Therefore, the new information as listed in section 2.1 need to be added in DATA COLLECTION UPDATE message. Besides, DATA COLLECTION REQUEST message also needs to be updated accordingly to initiate the information reporting.

Proposal 5: To update the existing DATA COLLECTION UPDATE message and DATA COLLECTION REQUEST message to support new required information collected between gNBs.

Below we analyses the new information need to be added in DATA COLLECTION UPDATE message one by one.

- Predicted radio resource status per slice

The existing Radio Resource Status IE can include a Slice Radio Resource Status List as below which indicates the usage of the PRBs per slice for all traffic in Downlink and Uplink. Therefore this IE can be reused, but the semantics description of this IE needs to be updated to include Slice Radio Resource Status List IE.

| >>Slice Radio Resource Status List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Slice Radio Resource Status Item |  | 1..< maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>S-NSSAI Radio Resource Status List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Radio Resource Status Item |  | 1..<maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>>>>Slice DL GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice UL GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice DL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of DL PRBs available per cell for the slice if all the resources the slice could access were usable. | – |  |
| >>>>>>Slice UL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of UL PRBs available per cell for the slice if all the resources the slice could access were usable. | – |  |

- Predicted slice available capacity

The existing Slice Available Capacity IE as below can be reused.

#### 9.2.2.55	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Available Capacity |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI Available Capacity List |  | 1 |  |  |
| >>S-NSSAI Available Capacity Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Available Capacity Value Downlink | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity . Slice Available Capacity Value Downlink should be measured on a linear scale. |
| >>>Slice Available Capacity Value Uplink | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity. Slice Available Capacity Value Uplink should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

- Future CCO State together with an associated coverage modification cause

- Timing Information

The Timing information is only referred to when the future CCO state needs to be applied, so it should be carried together with Future CCO state.

A new Future CCO State IE group can be added. As for the detailed information in Future CCO State IE, we can refer to the existing Coverage Modification Item IE as below, but a Timing Information needs to be newly added.

| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |

In DATA COLLECTION REQUEST message, new values need to be added in Report Characteristics for Data Collection IE for Predicted Slice Available Capacity and Future CCO State.

TP to TS38.423, mirroring the above analysis, is available in Annex.

Proposal 6: To agree the TP to TS38.423 in Annex.

For stage 3 impacts of Rel-18 leftovers, please refer to R3-245296 and R3-245297.

## 2.3	Open issues

For the following issues, the initial analysis had been done and possible solutions had been discussed, but we still need further discussion on the details in WI.

- Exact level of finer granularity UE performance for Slicing

- F1 impact for CCO (whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information?)

- The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP.

- Potential solutions addressing the two problems for Continuous MDT

# Proposals

In this contribution, we provide our general considerations on the new WI on Enhancements for AI/ML for NG-RAN and the following proposals are made.

Proposal 1: Add the above two new use cases, i.e. Slicing and CCO, in the stage 2 specification TS38.300.

Proposal 2: To add the above deployment solutions in the stage 2 specifications, including TS38.300 and TS38.401.

Proposal 3: To add the above new information that can be configured to be reported by an NG-RAN node in the stage 2 specification TS38.300.

Proposal 4: To agree the draft CRs on Enhancements for AI/ML for NG-RAN to TS38.300 in R3-245294 and to TS38.401 in R3-245295.

Proposal 5: To update the existing DATA COLLECTION UPDATE message, DATA COLLECTION REQUEST message and DATA COLLECTION RESPONSE message to support new required information collected between gNBs.

Proposal 6: To agree the TP to TS38.423 in Annex.

# Reference

[1] RP-242358, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[2] TR38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

# Annex TP to TS38.423

<Start of 1st change>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity " of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Future CCO State IE, if the eleventh bit, "Future CCO State" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

<Start of 2nd change>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Future CCO State Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<Start of 3rd change>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.2.3.1 |  |  |  | YES |  | ignore |  |
| NG-RAN node1 Measurement ID |  | M |  |  |  | INTEGER (1..4095,...) |  | Allocated by NG-RAN node1 |  | YES |  | reject |  |
| NG-RAN node2 Measurement ID |  | M |  |  |  | INTEGER (1..4095,...) |  | Allocated by NG-RAN node2 |  | YES |  | reject |  |
| Cell Measurement Result for Data Collection List |  |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Cell Info Result for Data Collection Item |  |  |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |  | – |  |  |  |
| >>Cell ID |  | M |  |  |  | Global NG-RAN Cell Identity 9.2.2.27 |  | Indicates an NR Cell Identity. |  | – |  |  |  |
| >>Predicted Radio Resource Status |  | O |  |  |  | Radio Resource Status 9.2.2.50 |  | The IE only includes the SSB Area Radio Resource Status List IE and Slice Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. |  | – |  |  |  |
| >>Predicted Number of Active UEs |  | O |  |  |  | Number of Active UEs 9.2.2.62 |  |  |  | – |  |  |  |
| >>Predicted RRC Connections |  | O |  |  |  | RRC Connections 9.2.2.56 |  |  |  | – |  |  |  |
| UE Associated Info Result List |  |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >UE Associated Info Result Item |  |  |  | 1 .. < maxnoofUEReports > |  |  |  |  |  | – |  |  |  |
| >>UE Assistant Identifier |  | M |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | NG-RAN node UE XnAP ID allocated by NG-RAN node1. |  | – |  |  |  |
| >>UE Performance |  | O |  |  |  | 9.2.3.179 |  |  |  | – |  |  |  |
| >>Measured UE Trajectory |  | O |  |  |  | 9.2.3.182 |  | It contains information about cells that a UE has connected to. |  | – |  |  |  |
| Node Associated Info Result |  |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Energy Cost |  | O |  |  |  | INTEGER (0..10000,…) |  | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. |  | - |  | - |  |
| Predicted Slice Available Capacity |  | O |  |  |  | Slice Available Capacity 9.2.2.55 |  |  |  | YES |  | ignore |  |
| Future CCO State List |  |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Future CCO State List Item |  |  |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |  | – |  |  |  |
| >>Global NG-RAN Cell Identity |  |  |  |  |  | M |  |  |  |  |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| >>Timing Information | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates when to apply the coverage modification. |  |  |

<Start of 4th change>

### 9.3.4	PDU Definitions

<Unchanged part omitted >

BAPAddress,

S-CPAC-Request,

SK-COUNTER,

RegistrationRequestForDataCollection,

ReportCharacteristicsForDataCollection,

ReportingPeriodicityForDataCollection,

NodeAssociatedInfoResult,

SLPositioning-Ranging-Services-Info,

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

FutureCCOState-List

FROM XnAP-IEs

<Unchanged part omitted >

id-sk-Counter,

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-PredictedSliceAvailableCapacity,

id-FutureCCOState-List,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-CellMeasurementResultForDataCollection-List	CRITICALITY ignore	TYPE CellMeasurementResultForDataCollection-List		PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional }|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional },

{ ID id-PredictedSliceAvailableCapacity				CRITICALITY ignore	TYPE SliceAvailableCapacity	PRESENCE optional },

{ ID id-FutureCCOState-List							CRITICALITY ignore	TYPE FutureCCOState-List	PRESENCE optional },

...

}

<Start of 5th change>

### 9.3.5	Information Element definitions

<Unchanged part omitted >

FreqDomainSlotHSNAconfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

FutureCCOState-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF FutureCCOState-List-Item

FutureCCOStateList-Item ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

cellCoverageState				INTEGER (0..63, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- This IE shall be present if the Cell Deployment Status Indicator IE is present.

sSB-Coverage-Modification-List	SSB-Coverage-Modification-List,

coverageModificationCause		CoverageModificationCause				OPTIONAL,

timingInformation				TimingInformation						Optional,

iE-Extension			ProtocolExtensionContainer { { FutureCCOStateList-Item-ExtIEs } } OPTIONAL,

...

}

FutureCCOStateList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

<Start of 6th change>

### 9.3.7	Constant definitions

<Unchanged part omitted >

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-PredictedSliceAvailableCapacity																	ProtocolIE-ID ::= xx1

id-FutureCCOState-List																			ProtocolIE-ID ::= xx2

END

-- ASN1STOP

<End of change>