3GPP TSG-RAN WG3 Meeting #125-bis	R3-245299

Hefei, China, 14th – 28th October 2024

Agenda Item:		17.2

Source:		NEC

Title:			Support on-demand SSB operation

Document for:		Discussion and decision

# Introduction

One objective on on-demand SSB (OD-SSB) is captured in WID [1] as follows:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

In last RAN3 meeting, we had some discussions on Xn impact and F1 impact but no conclusion due to lack of relevant agreements in other WGs. The following issues were listed FFS:

No Xn impact?

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement.

F1 impact?

- NES gNB-CU can indicate NES gNB-DU about the cells which are allowed to apply on-demand SSB?

- On-demand SSB operation is decided by CU or DU?

- Cell On-demand SSB status exchange?

In this contribution, we discuss this issues and other potential RAN3 impacts.

# Discussion

## RAN1 agreements

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Below are the agreements made in RAN1#118 meeting that may influence RAN3 discussion.

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

## RAN2 agreements

Below are the agreements made in RAN2#127 meeting that may influence RAN3 discussion.

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

## RAN3 impacts

## Xn impact

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

In last meeting, one issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? Considering the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. Besides, RAN2 is also working on how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration and their initial assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell.

Therefore, from the above two points of view, no Xn impact is foreseen for OD-SSB SCell operation for now. But this issue can be kept FFS in case RAN1 or RAN2 reaching any new agreements that will change the above working assumptions.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

## F1 impact

RAN1 agreed to Support on-demand SSB SCell operation triggered by gNB and so for split architecture, we need to discuss which node, e.g. gNB-CU or gNB-DU, can decide OD-SSB configuration and trigger OD-SSB transmission.

As in legacy, gNB-DU decides the SSB configuration, OD-SSB configuration can still be decided by gNB-DU. Besides, considering in legacy, after CA is set up, gNB uses MAC CE (SCell Activation/Deactivation) to activate or deactivate data transmission for SCell, gNB-DU is the suitable node to trigger OD-SSB transmission.

Proposal 2: gNB-DU decides OD-SSB operation configuration and triggers OD-SSB transmission.

Based on RAN2 agreement “RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration”, gNB-DU needs to sends the OD-SSB configuration to gNB-CU, e.g via GNB-DU CONFIGURATION UPDATE message.

Proposal 3: gNB-DU sends the OD-SSB configuration of SCells to gNB-CU.

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

Proposal 4: gNB-CU can provide suggested SSB operation parameters to gNB-DU to assist the on-demand SSB SCell operation.

# Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

Proposal 2: gNB-DU decides OD-SSB operation configuration and triggers OD-SSB transmission.

Proposal 3: gNB-DU sends the OD-SSB configuration of SCells to gNB-CU.

Proposal 4: gNB-CU can provide suggested SSB operation parameters to gNB-DU to assist the on-demand SSB SCell operation.

# Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR