3GPP TSG-RAN WG3 Meeting #125-bis	R3-245300

Hefei, China, 14th – 28th October 2024

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a way forward on on-demand SIB1 (OD-SIB1) was made, i.e. support Case-2 as part of normative work on OD SIB1 for Rel-19 NES, and the WID was updated accordingly as below [1]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In RAN3#125 meeting, the following agreements were achieved for case 2 of OD SIB1:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

This contribution will further discusses the RAN3 impact of OD-SIB1 based on the above study scope and agreements.

# Discussion

## UL WUS provision

In case 2, a UE receives UL WUS configuration of a NES cell from Cell A, and then the UE sends an UL WUS to the NES cell to request it to transmit SIB1 for a specified duration. Once receiving the UL WUS, the NES cell can send SIB1 in the legacy way and for one or more legacy SIB1 periodicity.

Before the NES cell starts on demand SIB1 or when the UL WUS configuration changes, the NES cell needs to provide its UL WUS configuration to cell A. RAN3 has already reached the following agreement on inter-node exchange of UL WUS configuration from NES cell to Cell A.

UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

Regarding to the procedure for UL WUS config exchange over Xn interface, the existing NG-RAN node Configuration Update procedure, which is used to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn interface, can be reused for UL WUS configuration exchange, as shown in figure 1.

Proposal 1: NES cell sends UL WUS configuration to Cell A in NG-RAN NODE CONFIGURATION UPDATE message.

As we already agreed, upon the reception of the UL WUS configuration, Cell A decides whether it can support UL WUS configuration provision for the NES cell and then provides the feedback in the Acknowledge message.

Figure 1: NG-RAN node Configuration Update procedure

Proposal 2: Cell A provides feedback on whether it can support UL WUS configuration provision for the NES cell in NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

In case of there are multiple Cell A for a single NES cell, which should be a rare case if under proper cell planning, the NES cell can choose one Cell A for UL WUS configuration provision. If the 1st chosen Cell A reject the request, then the NES cell can try another Cell A. The same procedure in figure 1 is still applicable in this case.

Proposal 3: In case of there are multiple Cell A for a single NES cell, NES cell can choose on Cell A for UL WUS configuration provision.

Regarding the detailed IEs that need to be added in the NG-RAN node Configuration Update message, we can discuss after RAN2 defining the UL WUS configuration parameters.

According to the agreement below, NES Cell DU needs to send the UE WUS configuration to NES Cell CU.

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU.

The existing GNB-DU CONFIGURATION UPDATE message and F1 SETUP REQUEST message can be reused to exchange UL WUS configuration NES Cell DU to NES Cell CU.

Proposal 3: NES Cell DU sends UL WUS configuration to NES Cell CU in GNB-DU CONFIGURATION UPDATE message and F1 SETUP REQUEST message.

## OD SIB1 activation/deactivation

After the NES cell provide its UL WUS configuration to cell A, the NES cell can update Cell A if the current UL WUS configuration is changed or released. Meanwhile, the NES cell can activate or deactivate (switch to legacy way) OD SIB transmission at any time. If the NES cell is in SIB1 broadcast, the UE can receive SIB1 in legacy way; if the NES cell is in OD SIB1, the UE will request cell A for the UL WUS configuration first and then trigger the NES cell to send SIB1. Therefore, we think, if this activate/deactivate switch is dynamic and frequent, the NES cell may not need to informs cell A over Xn about this activate/deactivate status change.

However, considering there might be a case that is multiple NES cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES cells under that Cell A can be activate OD SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 4: Cell A can provide guidance to NES cell about OD SIB1 activation/deactivation over Xn.

In split architecture, gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES cells and neighbor cells, so we prefer gNB-CU to decide OD SIB1 activate/deactivate of a NES cell, or at least gNB-CU should provide guidance to gNB-DU on OD SIB1 activation/deactivation of a NES cell.

Proposal 5: gNB-CU decides or guides gNB-DU on the OD SIB1 activation/deactivation of a NES cell, and sends the request to gNB-DU.

# Proposals

In this contribution, we analysed OD SIB1 transmission of a NES cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: NES cell sends UL WUS configuration to Cell A in NG-RAN NODE CONFIGURATION UPDATE message.

Proposal 2: Cell A provides feedback on whether it can support UL WUS configuration provision for the NES cell in NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 3: In case of there are multiple Cell A for a single NES cell, NES cell can choose on Cell A for UL WUS configuration provision.

Proposal 4: Cell A can provide guidance to NES cell about OD SIB1 activation/deactivation over Xn.

Proposal 5: gNB-CU decides or guides gNB-DU on the OD SIB1 activation/deactivation of a NES cell, and sends the request to gNB-DU.

# Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR