3GPP TSG-RAN WG3 Meeting #125bis	R3-245307

Hefei, China, 14th – 18th Oct 2024

Title: 	(TP for TR38.769) Text description for AIoT Signalling and Procedures

Source: 	ZTE Corporation, Huawei, NEC, Nokia, CATT, China Telecom, CMCC

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #125 meeting, we have agreed with the flow chart for AIoT Signalling and Procedures.

However, although the corresponding text description for the flow chart has been discussed in the SoD R3-244703, it is not agreed.

This paper proposes to agree the corresponding text description for the flow chart.

- Text Proposal

============================Start of Change==========================

6.4.2	Signaling and Procedures for Topology 1

6.4.2.1 Candidate procedures for AIoT Inventory for Topology 1

Figure 6.4.2.1-1: Message flow for AIoT Inventory in Topology 1

1.  The AIoT CN sends an Inventory request message to the AIoT RAN node.

2. The AIoT RAN node sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4/4b. After receiving inventory result from the AIoT device(s), the AIoT RAN node may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 2: The Step 4/4b may happen in parallel with Step 3.

6.4.3	Signaling and Procedures for Topology 2

6.4.3.1 Candidate procedures for AIoT Inventory for Topology 2

Figure 6.4.3.1-1: Message flow for AIoT Inventory in Topology 2 (if RRC-based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled gNB.

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

Editor’s Note: Whether the AIoT-enabled gNB can select AIoT-enabled UE(s) is FFS.

2. The AIoT-enabled gNB sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled gNB may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s).

4/4b. After receiving inventory result reported from the AIoT enabled UEs, the AIoT-enabled gNB may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 3: The Step 4/4b may happen in parallel with Step 3.

Figure 6.4.3.1-2: Message flow for AIoT Inventory in Topology 2 (if NAS/UP based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled UE(s).

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

Editor’s Note: Whether the AIoT-enabled gNB can select AIoT-enabled UE(s) is FFS.

2. The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled UE(s) may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s).

4/4b. After receiving inventory result from the AIoT device(s), the AIoT enabled UE(s) may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 3: The Step 4/4b may happen in parallel with Step 3.

============================End of Change==========================