3GPP TSG-RAN WG3 Meeting #125-bis	R3-245311

Hefei, China, 14-18 Oct 2024

Agenda item:	10.3.1

Source:	Nokia

Title:	Solution to avoid restarting MRO in NTN deployments

Document for:	Discussion

# 1	Introduction

At RAN3 #125, the discussion on MRO for NTN deployments focused on the failure cases to address. While important, the problem of restarting MRO whenever cell moves is still not discussed. Therefore, in this paper, we remind the description of the issue related to moving cells: hot to avoid restarting MRO analysis for given area whenever cell moves over?

# 2	Discussion

This discussion has been presented originally in [1].

As discussed in [2], in classic networks, MRO monitors executed handovers and enables reporting failed or nearly-failed executions to the source nodes. Those, after accumulating necessary statistics, can possibly apply necessary action. MRO also helps filtering out failures that are not caused by the mobility settings and thus avoid unnecessary corrections. This mechanism has always been based on the assumption that the access network is stable: cells are provided from a fixed locations and can collect information from mobile users that roam over their serving areas as long as it is needed (the only considered dynamism was related to relatively small cell border changes or cell switching off).

In case of NTN, the cell moves in a perfectly predictable manner while UE mobility can be considered negligible. Handovers are therefore based not on radio signal reporting but rather on timing. The latter does not mean a handover cannot fail: due to temporary conditions (e.g. clouds, rain) radio propagation may change and thus timed handover may fail. This may require MRO to adjust the condition to prepare and trigger inter-satellite handover. The existing MRO mechanism may not be sufficient, either, in case of moving cells.

Proposal 1: MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.).

In typical terrestrial network deployment, such temporary propagation changes would be handled by MRO by collecting statistics from UEs in the area and then modifying the mobility setting so that it would adapt to the changed conditions. This, however, will fail in NTN deployment: any MRO adaptation will get invalid as soon as the cell moves away (in fact, a cell may not even have time to collect the statistics). And the next cell over the concerned area will have to start the same MRO adaptation anew, while UEs will suffer.

For the cells handled within the same gNB (in case of regenerative NTN architecture), the change in the mobility configurations (e.g. in the condEvent D1) may be directly shared internally and applied for the other cells which may cover the same geographical area in the future time.

The figure below provides a complex view of an area covered by LEO satellite via couple of cells. For simplicity a problematic scenario that may lead into a change in condEvent D1 is represented via a cloud attenuation in the area covered by orange and blue cell in the point of time T0. As the LEO satellite moves in time, in point of time T1 different set of cells will cover the same geographical area impacted with the cloud attenuation. However, as the cells that cover the area are handled with the same satellite and considering also within the same gNB then internally within the gNB the information related to change in the condEvent D1 may be applied to relevant set of cells covering the area impacted with cloud attenuation in the point in time T1.

The situation is being become more complicated in case the LEO satellite is being handled with another gNB on the earth or when the problematic area is being covered with cells handled by another satellite belong to another gNB. In case of regenerative NTN architecture the gNB is located directly in satellite side and thus such problem may occur each time the problematic area is being covered with cells of different satellite.

Observation 1: In case of moving cells and local propagation changes, each cell has to start similar MRO analysis anew, which limits the effectiveness of the MRO mechanism.

There is the Mobility Setting Change procedure which enables one node to inform a neighbour about modified mobility triggering point. The procedure has the trigger information, which tells it is due to MRO adaptation. The neighbour may, in theory, attempt to apply the same modification towards the next cell in the chain when the next cell border moves over the area. This will, however, create more problems than benefits: the Mobility Setting Change operates on cell- or SSB-scale, which, in case of NTN, is too wide.

Observation 2: The existing Mobility Setting Change procedure is insufficient to transfer effectively information about MRO adaptations in NTN deployments.

The information that needs to be provided to the neighbour is either location of the UE(s) for which adaptation of the mobility setting is applied, or a list of UEs for which special triggering was applied must be provided. In the former case, the new serving cell can apply the change immediately for UEs in the given location, while in the latter it may apply it to the same UEs when the serving cell changes again (it may also apply it to other UEs in the same area).

The former approach has the problem of defining the location to be addressed. It may therefore be simpler to list the affected UEs and the applied setting change. The procedure can be used during CHO, after CHO preparation, when the affected UEs can be identified with UE IDs of both, the source and the target. Since the local problem may concern in a similar manner multiple UEs, the information shall be provided as a list.

Using a single procedure may be better than informing about the change per each UE separately (e.g. during HO preparation), because it can separate the feature from the HO and enable providing the information also after HO preparation, if needed.

A simplified message flow for the proposed solution is presented below:

Proposal 2: The Mobility Setting Change procedure is enhanced to provide per-UE adaptation of the CHO configuration to the next cell serving the area where similar MRO adaptation is needed.

# 3	Conclusions

In this paper, we remind the discussion on the deficiencies of the classic MRO mechanism in case of moving cells, as it happens in NTN deployments. First, we remind the proposal from [1]:

Proposal 1: MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.).

Then, we move forward and make following observations:

Observation 1: In case of moving cells and local propagation changes, each cell has to start similar MRO analysis anew, which limits the effectiveness of the MRO mechanism.

Observation 2: The existing Mobility Setting Change procedure is insufficient to transfer effectively information about MRO adaptations in NTN deployments.

They enable us to make the final proposal:

Proposal 2: The Mobility Setting Change procedure is enhanced to provide per-UE adaptation of the CHO configuration to the next cell serving the area where similar MRO adaptation is needed.

Based on the above, we propose a TP to XnAP in the Annex.

# References

- R3-244066, RAN3 #125

- R3-241559, RAN3 #123-bis

# Annex: Text proposal to TS 38.423

First change, ommited text not changed

### 8.4.9	Mobility Settings Change

#### 8.4.9.1	General

This procedure enables an NG-RAN node to negotiate the handover trigger settings with a peer NG-RAN node controlling neighbouring cells.

The procedure uses non UE-associated signalling.

#### 8.4.9.2	Successful Operation

Figure 8.4.9.2-1: Mobility Settings Change, successful operation

NG-RAN node1 initiates the procedure by sending the MOBILITY CHANGE REQUEST message to NG-RAN node2.

Upon receipt, NG-RAN node2 shall evaluate if the proposed NG-RAN node2 handover trigger modification may be accepted. If NG-RAN node2 is able to successfully complete the request it shall reply with MOBILITY CHANGE ACKNOWLEDGE message.

If the NG-RAN node1 SSB Offset Information IE is included in the MOBILITY CHANGE REQUEST, the NG-RAN node2 should take into account the included value of the SSB Offset for UE measurements received for the SSB Area indicated by the SSB Index IE.

If the NG-RAN node2 Proposed SSB Offset Information IE is included in the MOBILITY CHANGE REQUEST, the NG-RAN node2 shall, if supported, evaluate if the proposed value of SSB Offset IE may be accepted for the SSB Area indicated by the SSB Index IE. If NG-RAN node2 is able to successfully complete the request it shall reply with MOBILITY CHANGE ACKNOWLEDGE message.

If the UE CHO Offset for NTN List IE is included in the MOBILITY CHANGE REQUEST, the NG-RAN node2 may take the mobility settings provided there into account for UEs being handed over identified with the Source NG-RAN node UE XnAP ID IE and the Target NG-RAN node UE XnAP ID IE.

#### 8.4.9.3	Unsuccessful Operation

Figure 8.4.9.3-1: Mobility Settings Change, unsuccessful operation

If the requested parameter modification is refused by NG-RAN node2, or if NG-RAN node2 is not able to complete the procedure, NG-RAN node2 shall send the MOBILITY CHANGE FAILURE message with the Cause IE set to an appropriate value. NG-RAN node2 may include the Mobility Parameters Modification Range IE in the MOBILITY CHANGE FAILURE message, for example in cases when the proposed change is out of the permitted range.

NG-RAN node2 may include the SSB Offset Modification Range IE in the MOBILITY CHANGE FAILURE message, for example in cases when the proposed change is out of the permitted range.

#### 8.4.9.4	Abnormal Conditions

If the Source NG-RAN node UE XnAP ID IE and the Target NG-RAN node UE XnAP ID IE do not point to any prepared CHO, the NG-RAN node2 shall ignore the information for the given UE context.

Next change, ommited text not changed

#### 9.1.3.22	MOBILITY CHANGE REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate adaptation of mobility parameters.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node1 Mobility Parameters | O |  | Mobility Parameters Information 9.2.2.60 | Configuration change in NG-RAN node1 cell | YES | reject |
| NG-RAN node2 Proposed Mobility Parameters | M |  | Mobility Parameters Information 9.2.2.60 | Proposed configuration change in NG-RAN node2 cell | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| SSB Offsets List |  | 0..1 |  |  | YES | ignore |
| >SSB Offsets Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>NG-RAN node1 SSB Offset Information | O |  | SSB Offset Information 9.2.2.77 | Configuration change in NG-RAN node 1 SSB | – |  |
| >>NG-RAN node2 SSB Offset Information | M |  | SSB Offset Information 9.2.2.77 | Proposed configuration change in NG-RAN node2 SSB | – |  |
| UE CHO Offset for NTN List |  | 0..1 |  |  | YES | ignore |
| > UE CHO Offset for NTN Item |  | 1 .. <maxnoofPreparedCHOs> |  |  | – |  |
| >>Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| >>Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| >>NG-RAN node1 SSB Offset Information | O |  | SSB Offset Information 9.2.2.77 | Configuration change in NG-RAN node 1 SSB | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofPreparedCHOs | Maximum no. of prepared CHOs towards NG-RAN node2. Value is FFS. |

Remaining text not changed