3GPP TSG-RAN3 #125-bis	R3-245318

Hefei, China, 14 – 18 October 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	(TP for TS 38.300) Stage 2 signallings for inter-CU LTM

Document for:	Approval

# 1	Introduction

In this contribution we propose a TP for TS 38.300, based on R3-245316.

# 2	Annex I

# TP for TS 38.300

--------------------------------------------------------START changes-------------------------------------------------------------

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s).

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB.

6.	The source gNB may send the LTM CONFIGURATION UPDATE message  the candidate gNB(s) containing the information for subsequent LTM or for updating the configurations of candidate cells.

.

7.	The candidate gNB(s) sends the response LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early TA acquisition to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB which includes the TA values, and the associated PRACH resource information.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s Note: Details are FFS on above note.

19. The target gNB sends the LTM CONFIGURATION UPDATE message to the candidate gNB(s) in which it signals its allocated NG-RAN node UE XnAP ID.

20. The candidate gNB(s) sends the LTM CONFIGURATION UPDATE message to the target gNB in which it signals its allocated NG-RAN node UE XnAP ID in the response.

NOTE x: Step 19 and 20 may be triggered after step 16 by implementation.

21.	The UE sends an RRCReconfigurationComplete message to the target gNB.

22. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

.

--------------------------------------------------------END changes-------------------------------------------------------------