3GPP TSG-RAN3 Meeting #123bis	R3-245351

Hefei, China, 14th – 18th Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1270 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Cell DTX/DRX for NES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18 NES, the technique of cell DTXDRX is specified over air interface, however, the discussion in RAN3 has not finalized the specification for supporting cell DTXDRX over network interfaces. In split architecture, gNB-DU decides to use cell DTX/DRX and parameters of cell DTX/DRX. The gNB-DU can also decide to release cell DTX/DRX. When a NES cell is using cell DTX/DRX, it would be necessary to remove some unsuitable UEs for cell DTX/DRX (e.g., UEs not capable of cell DTX/DRX) from the cell. However, it is the gNB-CU to control the access/removal of UE. So, gNB-DU needs to indicate the information of cell DTX/DRX configuration (as well as release) to gNB-CU. Besides access control, the information is necessary for gNB-CU to determine the real PRB usage of the NES cell. It can be used in conjunction with the resource status report from gNB-DU. Also, the information is beneficial on interference elimination between different cells within the NES gNB. F1 impacts: The information of cell DTX/DRX can be indicated over F1 when the cell DTX/DRX is configured, modified or released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Cell DTXDRX Info List IE which informs the information of cell DTXDRX is introduced to the F1 SETUP REQUEST message and the GNB-DU CONFIGURATION UPDATE message. In the F1 setup procedure and the gNB-DU configuration update procedure, the behaviors for gNB-CU receiving the Cell DTXDRX Info List IE is added. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, will impact the gNB-DU configuration update procedure and the F1 setup procedure. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this change, the gNB-CU will not be able to perform access control or derive the resource status when the cell applies cell DTX/DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.2.4.2, 9.2.1.4, 9.2.1.7, 9.3.1.x (new), 9.3.1.y (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.423 CR 1379 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Revision#1 update the front page and remove the changes to 8.2.5.2 and 9.2.1.10 based on Revision 0 (R3-240185) - Compared to Revision#0, presence of Cell DTXDRX Type IE in Cell DTXDRX Status Required IE is changed from mandatory to optional in Revision#1 - Compared to Revisoin#1, Revision#2 add the change to 8.2.3.2 and 9.2.1.4, modify the change to 8.2.4.2, 9.2.1.7, 9.4.4, 9.4.5 and 9.4.7. |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Cell DTX/DRX Info List IE is included in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take the information in the Cell DTXDRX Info IE into account for the cell[, e.g., with respect to performing access control to the cell, deriving the resource status of the cell or eliminating interference between different serving cells].

-------------------------------------------Next change----------------------------------------------

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Cell DTX/DRX Info List IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take the information in the Cell DTXDRX Info IE into account for the cell[, e.g., with respect to access control to the cell, determining the resource status of the cell or eliminating interference between different serving cells].

-------------------------------------------Next change----------------------------------------------

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information associated to an F1-C interface instance.

NOTE:	If a TNL association is shared among several F1-C interface instances, several F1 Setup procedures are issued via the same TNL association after that TNL association has become operational.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) |  | YES | ignore |
| gNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| gNB-DU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-node. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor gNB-ID | O |  | Global gNB ID 9.3.1.305 | The Global gNB ID of a mobile IAB-node’s RRC-terminating IAB donor. This IE is only present if the mobile IAB-node’s RRC terminating IAB-donor-CU is different from the gNB-CU receiving this message. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Cell DTXDRX Info List |  | 0..1 |  |  | YES | ignore |
| >Cell DTXDRX Info Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| >>Cell DTXDRX Info | M |  | 9.3.1.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

-------------------------------------------Next change----------------------------------------------

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| >Cells Status Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Service Status | M |  | 9.3.1.68 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-DU ID | O |  | 9.3.1.9 |  | YES | reject |
| gNB-DU TNL Association To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-DU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | reject |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. | - |  |
| >>TNL Association Transport Layer Address gNB-CU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Cell DTXDRX Info List |  | 0..1 |  |  | YES | ignore |
| >Cell DTXDRX Info Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| >>Cell DTXDRX Info | M |  | 9.3.1.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

-------------------------------------------Next change----------------------------------------------

#### 9.3.1.x	Cell DTXDRX Info

#### This IE contains the information of cell DTX/DRX for a cell served by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell DTXDRX Config | O |  | 9.2.2.y | Indicates the cell DTX or DRX of the concerned cell is configured and parameters of cell DTX or DRX. |
| Released Indication | O |  | ENUMERATED (true, ...) | Indicates the cell DTX or DRX configuration of the concerned cell is released. |

#### 9.3.1.y	Cell DTXDRX Config

This IE indicates the parameters of cell DTX/DRX configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE On-Duration Timer | M |  |  | Corresponds to cellDTXDRX-onDurationTimer in CellDTXDRX-Config IE, as defined in TS 38.331 [10]. |
| >SubMilliSeconds |  |  |  |  |
| >>SubMilliSeconds Timer | M |  | INTEGER (1..31) |  |
| >MilliSeconds |  |  |  |  |
| >>MilliSeconds Timer | M |  | ENUMERATED (ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, ... ) |  |
| CHOICE Cycle Start Offset | M |  |  | Corresponds to cellDTXDRX-CycleStartOffset in CellDTXDRX-Config IE, as defined in TS 38.331 [10]. |
| >ms10 |  |  |  |  |
| >>Offset ms10 | M |  | INTEGER (0..9) |  |
| >ms20 |  |  |  |  |
| >>Offset ms20 | M |  | INTEGER (0..19) |  |
| >ms32 |  |  |  |  |
| >>Offset ms32 | M |  | INTEGER (0..31) |  |
| >ms40 |  |  |  |  |
| >>Offset ms40 | M |  | INTEGER (0..39) |  |
| >ms60 |  |  |  |  |
| >>Offset ms60 | M |  | INTEGER (0..59) |  |
| >ms64 |  |  |  |  |
| >>Offset ms64 | M |  | INTEGER (0..63) |  |
| >ms70 |  |  |  |  |
| >>Offset ms70 | M |  | INTEGER (0..69) |  |
| >ms80 |  |  |  |  |
| >>Offset ms80 | M |  | INTEGER (0..79) |  |
| >ms128 |  |  |  |  |
| >>Offset ms128 | M |  | INTEGER (0..127) |  |
| >ms160 |  |  |  |  |
| >>Offset ms160 | M |  | INTEGER (0..159) |  |
| >ms256 |  |  |  |  |
| >>Offset ms256 | M |  | INTEGER (0..255) |  |
| >ms320 |  |  |  |  |
| >>Offset ms320 | M |  | INTEGER (0..319) |  |
| >ms512 |  |  |  |  |
| >>Offset ms512 | M |  | INTEGER (0..511) |  |
| >ms640 |  |  |  |  |
| >>Offset ms640 | M |  | INTEGER (0..639) |  |
| >ms1024 |  |  |  |  |
| >>Offset ms1024 | M |  | INTEGER (0..1023) |  |
| >ms1280 |  |  |  |  |
| >>Offset ms1280 | M |  | INTEGER (0..1279) |  |
| >ms2048 |  |  |  |  |
| >>Offset ms2048 | M |  | INTEGER (0..2047) |  |
| >ms5120 |  |  |  |  |
| >>Offset ms5120 | M |  | INTEGER (0..5119) |  |
| >ms10240 |  |  |  |  |
| >>Offset ms10240 | M |  | INTEGER (0..10239) |  |
| Slot Offset | M |  | INTEGER (0..31) | Corresponds to cellDTXDRX-SlotOffset in CellDTXDRX-Config IE, as defined in TS 38.331 [10]. |
| Config Type | M |  | ENUMERATED (dtx, drx, dtxdrx, ...) | Corresponds to cellDTXDRXconfigType in CellDTXDRX-Config IE, as defined in TS 38.331 [10]. |

-------------------------------------------Next change----------------------------------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

Cell-DTXDRX-Info-List,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Cell-DTXDRX-Info-List,

id-Recommended-SSBs-for-Paging-List,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-DU-ID						CRITICALITY reject	TYPE GNB-DU-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE GNB-DU-Name						PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE GNB-DU-Served-Cells-List			PRESENCE optional	}|

{ ID id-GNB-DU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name			CRITICALITY ignore	TYPE Extended-GNB-DU-Name				PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-gNB-ID	CRITICALITY reject	TYPE GlobalGNB-ID					PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation	CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-Cell-DTXDRX-Info-List			CRITICALITY ignore	TYPE Cell-DTXDRX-Info-List				PRESENCE optional	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-Related-Info	CRITICALITY reject	TYPE RRC-Terminating-IAB-Donor-Related-Info	         PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation    CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		      PRESENCE optional	}|

{ ID id-Cell-DTXDRX-Info-List					CRITICALITY ignore	TYPE Cell-DTXDRX-Info-List									PRESENCE optional	},

...

}

Served-Cells-To-Add-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Add-ItemIEs } }

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Modify-ItemIEs } }

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Delete-ItemIEs } }

Cells-Status-List	::= SEQUENCE (SIZE(0.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Status-ItemIEs } }

Dedicated-SIDelivery-NeededUE-List::= SEQUENCE (SIZE(1.. maxnoofUEIDs))	OF ProtocolIE-SingleContainer { { Dedicated-SIDelivery-NeededUE-ItemIEs } }

GNB-DU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-DU-TNL-Association-To-Remove-ItemIEs } }

Served-Cells-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Add-Item		CRITICALITY reject	TYPE	Served-Cells-To-Add-Item				PRESENCE mandatory	},

...

}

Served-Cells-To-Modify-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Modify-Item			CRITICALITY reject	TYPE		Served-Cells-To-Modify-Item							PRESENCE mandatory	},

...

}

Served-Cells-To-Delete-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Delete-Item				CRITICALITY reject	TYPE		Served-Cells-To-Delete-Item					PRESENCE mandatory	},

...

}

Cells-Status-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Status-Item				CRITICALITY reject	TYPE		Cells-Status-Item					PRESENCE mandatory	},

...

}

Dedicated-SIDelivery-NeededUE-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Dedicated-SIDelivery-NeededUE-Item		CRITICALITY ignore	TYPE	Dedicated-SIDelivery-NeededUE-Item				PRESENCE mandatory	},

...

}

GNB-DU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-DU-TNL-Association-To-Remove-Item		CRITICALITY reject	TYPE	 GNB-DU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### -- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CapacityValue::= SEQUENCE {

capacityValue				INTEGER (0..100),

sSBAreaCapacityValueList	SSBAreaCapacityValueList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CapacityValue-ExtIEs} } OPTIONAL

}

CapacityValue-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer { { Cause-ExtIEs} }

}

Cause-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

CUtoDUTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF CUtoDUTAInformation-Item

CUtoDUTAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

tagIDPointer			TagIDPointer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CUtoDUTAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CUtoDUTAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CSIResourceConfiguration ::= SEQUENCE  {

cSIResourceConfigToAddModList 			OCTET STRING		OPTIONAL,

cSIResourceConfigToReleaseList			OCTET STRING		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} }	OPTIONAL

}

CSIResourceConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXOnDurationTimer ::= CHOICE {

subMilliSeconds 		INTEGER (1..31),

milliSeconds    		ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600,

...},

choice-extension		ProtocolIE-Single-Container { {CellDTXDRXOnDurationTimer-ExtIEs} }

}

CellDTXDRXOnDurationTimer-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CycleStartOffset ::= CHOICE {

ms10					INTEGER(0..9),

ms20					INTEGER(0..19),

ms32					INTEGER(0..31),

ms40					INTEGER(0..39),

ms60					INTEGER(0..59),

ms64					INTEGER(0..63),

ms70					INTEGER(0..69),

ms80					INTEGER(0..79),

ms128					INTEGER(0..127),

ms160					INTEGER(0..159),

ms256					INTEGER(0..255),

ms320					INTEGER(0..319),

ms512					INTEGER(0..511),

ms640					INTEGER(0..639),

ms1024					INTEGER(0..1023),

ms1280					INTEGER(0..1279),

ms2048					INTEGER(0..2047),

ms2560					INTEGER(0..2559),

ms5120					INTEGER(0..5119),

ms10240					INTEGER(0..10239),

choice-extension		ProtocolIE-Single-Container { {CycleStartOffset-ExtIEs} }

}

CycleStartOffset-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXConfig ::= SEQUENCE {

onDurationTimer							CellDTXDRXOnDurationTimer,

cycleStartOffset						CycleStartOffset,

slotOffset                					INTEGER (0..31),

configType								ENUMERATED {dtx, drx, dtxdrx},

iE-Extensions							ProtocolExtensionContainer { { CellDTXDRXConfig-ExtIEs} } OPTIONAL,

...

}

CellDTXDRXInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-DTXDRX-Info-Item ::= SEQUENCE {

nRCGI		NRCGI,

cellDTXDRXConfig				CellDTXDRXConfig,

releaseIndication				ENUMERATED (true, ...),

iE-Extension					ProtocolExtensionContainer { { Cell-DTXDRX-Info-Item-ExtIEs} } OPTIONAL,

...

}

Cell-DTXDRX-Info-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-DTXDRX-Info-List ::= SEQUENCE (SIZE (1.. maxCellingNBDU)) OF Cell-DTXDRX-Info-Item

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

id-TAInformation-List								ProtocolIE-ID ::= 837

id-NonIntegerDRXCycle								ProtocolIE-ID ::= 838

id-PointA											ProtocolIE-ID ::= 839

id-SCS-SpecificCarrier								ProtocolIE-ID ::= 840

id-NR-PCI											ProtocolIE-ID ::= 841

id-PeerUE-ID											ProtocolIE-ID ::= 842

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-Cell-DTXDRX-Info-List							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------