3GPP TSG-RAN WG3 Meeting #125b	R3-245360

Hefei, CN, 14 - 18 Oct, 2024

Agenda Item:	14.2

Source:	Huawei

Title:	(TP for TS 38.413 and TS 38.300) Support MBS broadcast service

Document for:	discussion

# 1	Introduction

In order to indicate the intended MBS service area via NG interface, the last RAN3#125 meeting agreed to turn the work assumption into agreements, and would further check other options.

Turn WA into agreement:

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

At the same time, RAN2#127 meeting made the following agreements.

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

Taking above into account, this paper provides our further considerations and proposals.

# 2	Discussion

As quoted above the last RAN2 meeting agreed the following geographical area formats:

- a (set of) circles represented by reference location and radius, or

- a (set of) polygon(s)

It is RAN3 work to pass the intended service area from the AMF to the gNB so as to finally pass such area information to UEs. According to current NGAP specification, the MBS service area information is included in the MBS broadcast session related procedure, where the MBS service area information could be TA list or cell list.

Observation: The intended broadcast service area is defined by a geographical area represented by a (set of) reference Location and radius or by a (set of) polygon(s) over the Uu interface.

#### 9.3.1.208 MBS Service Area This IE contains the MBS service area. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Session Type; M. >location independent. >>MBS Service Area Information; M; 9.3.1.209. >location dependent. >>MBS Service Area Information List; 1. >>>MBS Service Area Information Item; 1..<maxnoofMBSServiceArea Information>. >>>>MBS Area Session ID; M; 9.3.1.207. >>>>MBS Service Area Information; M; 9.3.1.209. Range bound; Explanation. maxnoofMBSServiceAreaInformation; Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256.. #### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. Range bound; Explanation. maxnoofCellsforMBS; Maximum no. of cells allowed within one MBS Service Area. Value is 8192.. maxnoofTAIforMBS; Maximum no. of TAs allowed within one MBS Service Area. Value is 1024..

## 2.1	Intended MBS broadcast service area

The previous RAN3-125 meeting agreed that Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN. Under some circumstances, the mapped cell ID may not work well for MBS broadcast service. From the RAN2 agreements above regarding the intended areas over the Uu interface, these areas may not exactly coincide with mapped cell ID area. For example, if this area is represented as a set of polygons, it is not easily considered as one mapped cell area.

Hence, a new geographic area information should be defined over NG interface, and its formats can refer to RAN2 definitions. In this case the gNB may perform any mapping from the service areas over NG interface to service areas over the Uu interface.

- A new geographic area (its encoding is referring to RAN2 specification) can be defined for MBS broadcast service area.

The previous RAN3-125 meeting also agreed the following texts in the BLCR, with an editorial’s note.

### 16.14.5 NG-RAN signalling The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported: - The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information; - The Cell Identity used for Paging Optimization in NG interface; - The Cell Identity used for Area of Interest; - The Cell Identity used for PWS. The Cell Identity used for MBS broadcast service may be a Mapped Cell ID. Editor’s Note: The wording for the above text for using Mapped Cell ID for MBS may be further clarified.

The main reason of the above wording is that there were some comments that the NR CGI included in the MBS Service Area Information could also be a Uu Cell, e.g., used for the GEO case to cover a large fixed geographic area.

In order to differentiate the Uu Cell and mapped cell, there are two options over the NG interface below:

- Option 1: To introduce a CGI type IE with codepoint “mapped” or a new mapped CGI cell list

- Option 2: To use the new geographic area as the Uu Cell

We consider option 2 is pretty simple, without any additional indication of mapped cell, which is aligned with good practice of NTN in the past releases.

- The new geographic area can be used if to support the Uu Cell as the intended MBS broadcast area (i.e., without any explicit “mapped” CGI type or mapped CGI list).

In addition, the existing TAI list could also be reused to indicate the intended broadcast service, which is even bigger than Uu cells, e.g., covering multiple Uu cells. This is already supported by the specification.

- RAN3 to confirm that the TAI list can be used as intended MBS broadcast service area.

Note that multiple options can be supported, and it is up to the CN to indicate the intended service area to the gNB.

## 2.2 Stage 2 TP and Stage 3 TP

Based on the discussion in section 2.1, the Cell Identity used for MBS broadcast service could be regarded as mapped cell ID for NTN, and the editorial note can be removed. And it is suggested to add a separate paragraph in stage 2 to indicate that the MBS broadcast service is supported over the intended service area. And the intended service area can be in a form of cell ID list, TAI list, or geographic area list or any of combinations as defined in TS 38.413 [26].

In addition, for NGAP, a new MBS Service Area Geographic Area List IE can be introduced in the 9.3.1.209	MBS Service Area Information, for which its format can refer to TS 38.331, with FFS to be further checked with other WGs. It is suggested to add a description of MBS service area regarding NTN NG RAN signalling in stage 2 TP.

- RAN3 to agree the NGAP TP and the stage 2 TP provided in the Annex.

## 2.3 MBS broadcast procedures

With the satellite movement, the MBS broadcast session resources need to be setup or removed in time. Typically, when the satellite covers the intended service area, the MBS broadcast session can be setup, otherwise, it can be released.

Currently, the related MBS Broadcast Session Management Messages over the NGAP can at least include:

- Broadcast Session Setup procedure: To request the NG-RAN node to setup MBS session resources for a broadcast MBS session

- Broadcast Session Release procedure: To release the MBS session resources related to a previously established broadcast MBS session

- Broadcast Session Release Required procedure: To trigger the AMF to release the MBS session resources related to a previously established broadcast MBS session.

It seems to us that the existing broadcast session procedures can be used for NTN to setup or release the broadcast session, without any further changes.

- RAN3 to agree that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

# 3	Conclusion

In this paper, we provide the following observations and proposals:

Observation: The intended broadcast service area is defined by a geographical area represented by a (set of) reference Location and radius or by a (set of) polygon(s) over the Uu interface.

- A new geographic area (its encoding is referring to RAN2 specification) can be defined for MBS broadcast service area.

- The new geographic area can be used if to support the Uu Cell as the intended MBS broadcast area (i.e., without any explicit “mapped” CGI type or mapped CGI list).

- RAN3 to confirm that the TAI list can be used as intended MBS broadcast service area.

- RAN3 to agree the NGAP TP and the stage 2 TP provided in the Annex.

- RAN3 to agree that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

# Annex1	Text Proposal for TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  | - |  |
| >NR CGI | M |  | 9.3.1.7 |  | - |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  | - |  |
| >TAI | M |  | 9.3.3.11 |  | - |  |
| MBS Service Area Geographic Area [FFS] | O |  | OCTET STRING | Includes the [FFS] IE, as defined in TS 38.331 [18]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |

Editor’s Note: The MBS Service Area Geographic Area IE needs to be further reformatted, and the details are pending to RAN2.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	Text Proposal for TS 38.300 on top of R3-244856

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

The MBS broadcast service is supported over the intended service area. The intended service area can be in a form of cell ID list, TAI list, or the geographic area or any of combinations as defined in TS 38.413 [26].

Editor’s Note: The above paragraphs may be further reformulated.

END OF CHANGES

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>