3GPP TSG-RAN2 Meeting # 127bis 	R3-245364

# Hefei, P. R. China,  14 – 18 October, 2024

Title: 	Clarification on TRP ID for SRS Bandwidth Aggregation

Source: 	Huawei, Nokia, CMCC, CUC, ZTE

Agenda item:	9.2

Document Type:	Discussion

# 1. Introduction

This paper attempt to clarify and progress the discussion on TRP ID management e.g. for SRS bandwidth aggregation.

The RAN3 specification is not clear on whether the identification of a physical TRP hosting multiple cells should be with a single TRP ID with a list of TRP Information type e.g. per cell or multiple TRP ID. This “freedom” gives uncertainty on location possible location of the TRP ID, then extra constraint on LMF implementation on TRP selection, computation, etc …

In this discussion we focus on TRPID list encoding without further considerations.

# 2. Discussion

In Rel-16, a TRP within a NG-RAN node is uniquely identified by a TRP ID, which could further be associated with cell characteristics such as PCI, ARFCN, and CGI, as shown in the table in Clause 9.2.25 of NRPPa.

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.88 |  | YES | ignore |
| >>Common TA Parameters | M |  | 9.2.89 |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |

A TRP supporting BW aggregation, may be associated with multiple cells deployed in different frequencies, each associated with a PCI/ARFCN and/or CGI, as shown in Figure 1.

CGI X

CGI Y

Physical TRP

Freq 1

Freq 2

Figure 1 Illustration of multiple cells within a single physical TRP for a bandwidth aggregation

Observation 1: A physical TRP within a NG-RAN node supporting BW aggregation may be associated with multiple cells deployed in different frequencies.

RAN3 could specify this information by two ways/options:

Option 1: The TRP information for a physical TRP supporting bandwidth aggregation is constructed using two distinct TRP IDs, each associated with its own PCI/ARFCN and/or CGI.

CGI X/TRP ID A

CGI Y/TRP ID B

TRP ID X

NG-RAN node

Server

TRP ID X/Y (NRPPa)

TRP ID Y

Figure 2 Illustration of multiple TRP IDs within a single physical TRP

In this regard, the physical TRP is split into two logical TRPs at the LMF, as illustrated by Figure 2, and LMF should be able to identify them being collated using the same Geographical Coordinates.

An example of encoding of this option1 could results as:

TRPInformationTypeItem ::= 1

nrPCI ::==1,

nG-RAN-CGI::==1,

arfcn::==1,

TRPInformationTypeItem ::= 2

nrPCI ::==2,

nG-RAN-CGI::==2,

arfcn::==2,

Option 2: The TRP information for a physical TRP supporting bandwidth aggregation is constructed using a single TRP ID, but it can be associated with multiple PCI/ARFCN and/or CGI defining a list of TRP Information types.

In this regard, the physical TRP is a single logical TRP, with a single TRP ID, at the LMF with a single Geographical Coordinates. The TRP Information Type IE list, could associated multiple cells to this single logical TRP ID. The LMF must then be aware that this TRP is hosting multiple cells.

We should note:

- That the TRP Information Type IE indeed provide a list of NR PCI, NR CGI NR ARCFN, but the choice structure does not allow an explicit linkage of the PCI information with CGI, ARCFN, etc …

- That the cell ID introduced in e.g. the MEASUREMENT REQUEST (NRPPa 9.1.4.1) to associate the cell ID with each TRP ID, with the purpose to simplify the processing and management in the gNB, instead of distinguishing multiple cells under a single TRP. The reporting specified in the MEASUREMENT REQUEST is not matching with the option2.

An example of encoding of this option2 could results as:

TRPInformationTypeItem ::= 1

nrPCI ::==1,

nG-RAN-CGI::==1,

arfcn::==1,

TRPInformationTypeItem ::= 1

nrPCI ::==2,

nG-RAN-CGI::==2,

arfcn::==2,

Therefore, Option 1 should be a better option to enable the logical TRP ID, which is align with the original design for NRPPa.

Proposal: RAN3 to confirm and minute officially (MCC) that a TRP ID is associated with no more than one PCI value, ARFCN value, and NR CGI value.

# 3. Conclusion

We kindly ask RAN3 to discuss and agree the following proposal:

Proposal: RAN3 to confirm and minute officially (MCC) that a TRP ID is associated with no more than one PCI value, ARFCN value, and NR CGI value.