3GPP TSG-RAN3 Meeting # 125bis 	R3-245365

# Hefei, P. R. China,  14th – 18th October, 2024

Title:	Correction on Measurement Time Window

Agenda Item:	9.2

Source:	Huawei, CMCC, CUC, Ericsson

Document for:	Discussion

# 1	Introduction

Last RAN3 the discussion progress but we postponed again, due to further internal checking request the Time Windows management for UL-TDOA and Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only or not.

# 2	Discussion

In RAN3#125 the offline discussion concluded (Summary of Offline discussion R3-244658):

Summary of offline discussion:

Discuss whether measurements outside the measurement window(s) are allowed?

If Time Window Information Measurement List is included, all the configured measurement should be performed within the window(s)?

Feature of positioning measurement, on using of measurement time windows (including start time) may misalign with RAN1/RAN2, further check.

We kindly ask RAN3 to clarify whether the UL-TDOA and Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only or like UE (and RAN2) is specified the UL-TDOA and Multi-RTT and UL SRS resources could occur in different time windows, which need to be clarified by reference to stage 2. Please note this should be enable by two parallel messages from the LMF.

We do believe that from network side the UL-TDOA and Multi-RTT and UL SRS resources could occur in different time windows.

It is also possible to have both in same time windows, which also needs clarifications.

We are also thinking that a start time is beneficial to have clear indication signalled rather than an interpretation.

More details on RAN2 reference and possible change are provided in cover page of the CR provide in Annex.

# Conclusion

We kindly ask RAN3 to clarify the Time Windows behaviour, and agreed the CR proposed in Annex A and B

# 4	Annex A: CR for NRPPa

3GPP TSG-RAN3 Meeting # 125bis 	R3-24xxxx

# Hefei, P. R. China,  14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | <CR#> | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Measurement Time Window |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, CUC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following agreement can be found in RAN1 parameter list R1-2312708 on measurement time window(s): LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s). Each time window is defined with the following parameters: • The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time • The duration of the time window, which is given by a number of consecutive slots/symbols o Periodicity: Periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455, can be optionally provided. For a time window, LMF can optionally request gNB to perform UL measurements for UL-TDOA and Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only.. The Measurement time window(s) introduced for Rel-18 CPP, according to RAN1, are also applicable to perform the legacy measurements, i.e., UL measurements for DL-TDOA and Multi-RTT within indicated time window(s) only. In addition, last meeting, RAN2 (LPP) has introduced a new IE for the measurement time window configuration to specify what measurements the UE shall be performed within the configured time window, not necessarily same as above. nr-MeasurementsToPerformInTimeWindow-r18 BIT STRING { rstd (0), ueRxTx (1), rsrp (2), rsrpp (3), rscp (4), rscpd (5) } (SIZE(1..16)) OPTIONAL, -- Need ON .... Based on the agreements for LPP, we think similarly to SRS, the gNB may be indicated by the LMF to perform legacy UL measurements for UL-TDOA and multi-RTT on indicated UL SRS resources occurring within the dedicated time window(s). Note: this require two parallel messages from the LMF with the specifications of each time windows. According to RAN1, the parameters for each time window includes the start of the time window. The parameter is currently missing is clause 9.2.91 due to the presence of SFN and Slot number in Measurement Request message, which are introduced by R3-204970 for sp-SRS to indicate the SRS transmission. Therefore, the exiting IEs cannot be reused and an additional parameter is needed to indicate the start of the measurement time window. Then It is proposed to associate the Time Window Information Measurement List IE with a start time and a list of associated Positioning SRS Resource ID when the UL-TDOA and multi-RTT measurement are defined. This allows to fulfilled the RAN1 requirement to have the others measurement with the same or different Time windows as the CPP measurements. |
| 1 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Time Window Information Measurement List IE to enable the LMF to request the gNB to perform measurements for DL-TDOA or Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only or not. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it affects the behaviour of the NG-RAN to perform UL measurements. This CR has an impact under protocol & functional |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN1 parameter list. Impossible to perform like LPPa measurement in different time windows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.1, 9.2.91, 9.3.5, 9.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.473 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//****** Beginning of Change ******//

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the indicated time window(s) within which to perform the positioning measurements. If the Time Window Start IE is included in the Time Window Information Measurement Item IE, the NG-RAN node shall, if supported, consider this as an indication to perform all applicable measurements, as specified in TS 38.305 [18], at the indicated window start time.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

#### 8.5.1.3	Unsuccessful Operation

Figure 8.5.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

#### 8.5.1.4	Abnormal Conditions

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message but the NG-RAN node is unable to provide the measurement results within the indicated time, the NG-RAN node shall, if supported, respond with a MEASUREMENT FAILURE message with an appropriate cause value.

//****** Unchanged Text Omitted ******//

### 9.2.91	Time Window Information Measurement List

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time Window Information Measurement List |  | 1 |  |  | - |  |
| >Time Window Information Measurement Item |  | 1..<maxnoofTimeWindowMeas> |  |  | - |  |
| >>CHOICE Time Window Duration | M |  |  | Duration of time window with start time given by the System Frame Number IE and Slot Number IE. | - |  |
| >>>Slots |  |  |  |  |  |  |
| >>>>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  | - |  |
| >>Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |  |  |
| >>Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, 20480, 40960, 61440, 81920, 368640, 737280, 1843200, …) | Unit: Milli-seconds | - |  |
| >>Time Window Start |  | 0..1 |  |  | YES | ignore |
| >>>System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| >>>Slot Number | M |  | INTEGER(0..79) |  | - |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindowMeas | Maximum no of Time Window for measurement. Value is 16. |

//****** Unchanged Text Omitted ******//

### 9.3.5	Information Element definitions

//****** Unchanged Text Omitted ******//

id-ReportingGranularitykminus1AdditionalPath,

id-ReportingGranularitykminus2AdditionalPath,

id-ReportingGranularitykminus3AdditionalPath,

id-ReportingGranularitykminus4AdditionalPath,

id-ReportingGranularitykminus5AdditionalPath,

id-ReportingGranularitykminus6AdditionalPath,

id-MeasuredResultsAssociatedInfoList,

id-PointA,

id-NR-PCI,

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-TimeWindowStart

//****** Unchanged Text Omitted ******//

TimingReportingGranularityFactorExtended ::=INTEGER(-6..-1, ...)

TimeWindowStart ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotNumber				SlotNumber,

iE-Extension			ProtocolExtensionContainer { { TimeWindowStart-ExtIEs} }	OPTIONAL,

...

}

TimeWindowStart-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowInformation-Measurement-List ::= SEQUENCE (SIZE (1.. maxnoofTimeWindowMeas)) OF TimeWindowInformation-Measurement-Item

TimeWindowInformation-Measurement-Item ::= SEQUENCE {

timeWindowDurationMeasurement		TimeWindowDurationMeasurement,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicityMeasurement	TimeWindowPeriodicityMeasurement		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-Measurement-Item-ExtIEs} }	OPTIONAL,

...

}

TimeWindowInformation-Measurement-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-TimeWindowStart	CRITICALITY ignore EXTENSION timeWindowStart	PRESENCE optional}

,

...

}

//****** Unchanged Text Omitted ******//

### 9.3.8	Container definitions

//****** Unchanged Text Omitted ******//

id-ReportingGranularitykminus1AdditionalPath					ProtocolIE-ID ::= 147

id-ReportingGranularitykminus2AdditionalPath					ProtocolIE-ID ::= 148

id-ReportingGranularitykminus3AdditionalPath					ProtocolIE-ID ::= 149

id-ReportingGranularitykminus4AdditionalPath					ProtocolIE-ID ::= 150

id-ReportingGranularitykminus5AdditionalPath					ProtocolIE-ID ::= 151

id-ReportingGranularitykminus6AdditionalPath					ProtocolIE-ID ::= 152

id-MeasuredResultsAssociatedInfoList							ProtocolIE-ID ::= 153

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-TimeWindowStart												ProtocolIE-ID ::= xx3

//****** End of Change ******//

# 5	Annex B: CR for F1AP

3GPP TSG-RAN3 Meeting # 125bis 	R3-24xxxx

# Hefei, P. R. China,  14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | <CR#> | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Measurement Time Window |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC; CUC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following agreement can be found in RAN1 parameter list R1-2312708 on measurement time window(s): LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s). Each time window is defined with the following parameters: • The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time • The duration of the time window, which is given by a number of consecutive slots/symbols o Periodicity: Periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455, can be optionally provided. For a time window, LMF can optionally request gNB to perform UL measurements for UL-TDOA and Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only.. The Measurement time window(s) introduced for Rel-18 CPP, according to RAN1, are also applicable to perform the legacy measurements, i.e., UL measurements for DL-TDOA and Multi-RTT within indicated time window(s) only. In addition, last meeting, RAN2 (LPP) has introduced a new IE for the measurement time window configuration to specify what measurements the UE shall be performed within the configured time window, not necessarily same as above. nr-MeasurementsToPerformInTimeWindow-r18 BIT STRING { rstd (0), ueRxTx (1), rsrp (2), rsrpp (3), rscp (4), rscpd (5) } (SIZE(1..16)) OPTIONAL, -- Need ON .... Based on the agreements for LPP, we think similarly to SRS, the gNB may be indicated by the LMF to perform legacy UL measurements for UL-TDOA and multi-RTT on indicated UL SRS resources occurring within the dedicated time window(s). Note: this require two parallel messages from the LMF with the specifications of each time windows. According to RAN1, the parameters for each time window includes the start of the time window. The parameter is currently missing is clause 9.2.91 due to the presence of SFN and Slot number in Measurement Request message, which are introduced by R3-204970 for sp-SRS to indicate the SRS transmission. Therefore, the exiting IEs cannot be reused and an additional parameter is needed to indicate the start of the measurement time window. Then It is proposed to associate the Time Window Information Measurement List IE with a start time and a list of associated Positioning SRS Resource ID when the UL-TDOA and multi-RTT measurement are defined. This allows to fulfilled the RAN1 requirement to have the others measurement with the same or different Time windows as the CPP measurements. |
| 1 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Time Window Information Measurement List IE to enable the LMF to request the gNB to perform measurements for DL-TDOA or Multi-RTT on indicated UL SRS resources occurring within indicated time window(s) only or not. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it affects the behaviour of the NG-RAN to perform UL measurements. This CR has an impact under protocol & functional |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN1 parameter list. Impossible to perform like LPPa measurement in different time windows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1.334, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.455 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//****** Beginning of Change ******//

### 8.13.3	Positioning Measurement

#### 8.13.3.1	General

The purpose of the Positioning Measurement procedure is to allow the gNB-CU to request one or more TRPs in the gNB-DU to perform and report positioning measurements. The procedure uses non-UE-associated signalling.

#### 8.13.3.2	Successful Operation

Figure 8.13.3.2-1: Positioning Measurement procedure: successful operation

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT REQUEST message to the gNB-DU, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The gNB-DU node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the gNB-DU shall reply with the POSITIONING MEASUREMENT RESPONSE message including the Positioning Measurement Response List IE.

If the Positioning Report Characteristics IE is set to "OnDemand", the gNB-DU shall return the corresponding measurement results in the Positioning Measurement Result List IE in the POSITIONING MEASUREMENT RESPONSE message, and the gNB-CU shall consider that this reporting has been terminated by the gNB-DU.

If the Measurement Beam Information Request IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall include the Measurement Beam Information IE in the Positioning Measurement Result IE of the POSITIONING MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the Measurement Result IE in the POSITIONING MEASUREMENT RESPONSE message, the gNB-CU may use it for further signalling. If the Measurement Quality IE includes the Zenith Quality IE, the gNB-CU may use it for further signalling.

If the System Frame Number IE and/or the Slot Number IE are included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Measurement Characteristics Request Indicator IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the POSITIONING MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, consider this as the indicated time window(s) within which to perform the positioning measurements. If the Time Window Start IE is included in the Time Window Information Measurement Item IE, the gNB-DU shall, if supported, consider this as an indication to perform all applicable measurements, as specified in TS 38.305 [42], at the indicated window start time.

Interaction with the Positioning Measurement Report procedure:

If the Positioning Report Characteristics IE is set to "Periodic", the gNB-DU shall initiate the corresponding measurements, and it shall reply with the POSITIONING MEASUREMENT RESPONSE message without including any measurement results in the message. The gNB-DU shall then periodically initiate the Positioning Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, return the corresponding measurement results in the POSITIONING MEASUREMENT RESPONSE message within the indicated time.

If the Positioning Report Characteristics IE is set to "Periodic" and the Positioning Measurement Amount IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take it into account for sending the POSITIONING MEASUREMENT REPORT message.

#### 8.13.3.3	Unsuccessful Operation

Figure 8.13.3.3-1: Positioning Measurement procedure: unsuccessful operation

If the gNB-DU is unable to configure any of the requested positioning measurements for any of the TRPs in the TRP Measurement Request List IE of the POSITIONING MEASUREMENT REQUEST message, it shall respond with a POSITIONING MEASUREMENT FAILURE message.

#### 8.13.3.4	Abnormal Conditions

If the gNB-DU receives a POSITIONING MEASUREMENT REQUEST message containing an LMF Measurement ID corresponding to an ongoing positioning measurement, it shall consider the procedure as failed and initiate local error handling.

//****** Unchanged Text Omitted ******//

#### 9.3.1.334	Time Window Information Measurement List

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time Window Information Measurement List |  | 1 |  |  | - |  |
| >Time Window Information Measurement Item |  | 1..<maxnoofTimeWindowMeas> |  |  | - |  |
| >>CHOICE Time Window Duration | M |  |  | Duration of time window with start time given by the System Frame Number IE and Slot Number IE. | - |  |
| >>>Slots |  |  |  |  |  |  |
| >>>>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  | - |  |
| >>Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |  |  |
| >>Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, 20480, 40960, 61440, 81920, 368640, 737280, 1843200, …) | Unit: Milli-seconds | - |  |
| >>Time Window Start |  | 0..1 |  |  | YES | ignore |
| >>>System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| >>>Slot Number | M |  | INTEGER(0..79) |  | - |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindowMeas | Maximum no of Time Window of Measurements. Value is 16. |

### 9.4.5	Information Element Definitions

//****** Unchanged Text Omitted ******//

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PeerUE-ID,

id-MeasBasedOnAggregatedResources,

id-SIB23-message,

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-MaxDataBurstVolume,

id-TimeWindowStart,

//****** Unchanged Text Omitted ******//

TimingReportingGranularityFactorExtended ::=INTEGER(-6..-1,...)

TimeWindowStart ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotNumber				SlotNumber,

iE-Extension			ProtocolExtensionContainer { { TimeWindowStart-ExtIEs} }	OPTIONAL,

...

}

TimeWindowStart-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowInformation-Measurement-List ::= SEQUENCE (SIZE (1.. maxnoofTimeWindowMea)) OF TimeWindowInformation-Measurement-Item

TimeWindowInformation-Measurement-Item ::= SEQUENCE {

timeWindowDurationMeasurement		TimeWindowDurationMeasurement,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicityMeasurement	TimeWindowPeriodicityMeasurement		OPTIONAL,

-- This IE shall be present if the Time Window Type IE is set to the value “periodic”.

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-Measurement-Item-ExtIEs} }	OPTIONAL,

...}

TimeWindowInformation-Measurement-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TimeWindowStart	CRITICALITY ignore EXTENSION timeWindowStartMeasurement	PRESENCE optional}

,

...

}

//****** Unchanged Text Omitted ******//

### 9.4.7	Constant Definitions

//****** Unchanged Text Omitted ******//

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-E-CID-MeasuredResultsAssociatedInfoList			ProtocolIE-ID ::= 834

id-XR-Bcast-Information								ProtocolIE-ID ::= 835

id-MaxDataBurstVolume 								ProtocolIE-ID ::= 836

id-TAInformation-List								ProtocolIE-ID ::= 837

id-NonIntegerDRXCycle								ProtocolIE-ID ::= 838

id-PointA											ProtocolIE-ID ::= 839

id-SCS-SpecificCarrier								ProtocolIE-ID ::= 840

id-NR-PCI											ProtocolIE-ID ::= 841

id-PeerUE-ID										ProtocolIE-ID ::= 842

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-TimeWindowStart												ProtocolIE-ID ::= xx3

//****** End of Change ******//