3GPP TSG RAN WG3 Meeting #125bis	R3-245373

Hefei, China, 14 - 18 October, 2024
Agenda Item:	14.3

Source:    TCL.

Title:         Discussion on NTN regenerative architecture

Document for:	Discussion and Decision

# Introduction

As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support NTN architecture with 5G system functions on board of satellite (i.e., regenerative payloads) [1]. The support of NTN architecture with 5G system functions of regenerative payloads provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. On this regard, during last RAN2 meeting, there was a discussion on NG Interface management and Feeder link switch issue, and the following agreements has been made.

Agreements:

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

This report discusses NG interface management and feeder link switch and their impacts on UE NG related procedures.

# Discussion

3GPP is currently discussing the regenerative payload architecture of NTN satellite access in both SA and RAN WGs As part of Release 19, it is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support non-terrestrial network architecture with 5G system functions on board of satellite (i.e., regenerative payloads). In such an architecture the gNB can be deployed on-board of a satellite terminating the Uu radio interface toward the UE (via the service link) and terminating the NG network interface toward the 5GC (via the feeder link). Additionally, different gNBs hosted by two regenerative satellite payloads may often be interconnected to each other’s by the mean of an inter-satellite link (ISL) interface for exchanging of Xn protocol messages. Under such architecture, unlike terrestrial networks (TN) where NG interfaces is presumed to be always available, in regenerative NTN access, NG logical interfaces may often be disconnected, switched, suspended, or updated due to satellite movement or the switching of the  feeder link between gNB and AMF [2, 3, 4]. For example, if a UE is accessing from an NTN access with gNB on-board, there will always be a single NG logical link for such a UE towards its selected AMF/UPF (Figure 1). If a feeder link switching occurred (Figure 2), UE’s NG link related procedures such as initial UE message, initial UE context setup message exchange between UE serving gNB and selected AMF will be impacted.

Figure 1: Example of Regenerative NTN access with Feeder Link Switching

An NG disconnection due to a feeder change may lead to an interruption of UE of exchanging of UE NG related procedure such as initial UEs message, initial UE context setup message and/or the paging message. Such an interruption may impact  RRC resumption, RRC reestablishment , UE triggered state transition from RRC IDLE to RRC CONNECTED and/or network triggered RRC IDLE to RRC CONNECTED state transition via paging. One option to address this issue while maintaining UE RRC connection procedures and guarantying that AMF will not be changed for the UEs during the feeder link switch (setup/removal) is to allow enchaining of UEs NG related procedure such as initial UE message, initial UE context setup as a part of NG setup/removal signalling. Another alternative way, If the failed link is non-recoverable (e.g., after time or a number of tries), the AMF or satellite gNB may perform an exchanging of information about the other available XN, NG links, or XN and NG links combination to find the sets of active links to reroute UEs NG associated procedures over such links so that the AMF is may not need to be changed for the UEs (Figure 3).

Figure 2: Procedures of connection procedures during NG disconnection:

During the feeder link switch, to guarantee that AMF is not changed for the UEs, we kindly ask RAN3 to consider discussing the following proposals:

- Proposal 1: Discuss containing UEs NG related procedure such as initial UE message, initial UE context setup as a part of NG setup/removal signalling to guarantee that AMF will not be changed for the UEs during the feeder link switch,.

- Proposal 2: Study exchanging information regarding the availability of neighbouring XN and NG to allow rerouting UEs NG associated procedures over other NG/XN links in case that NG link interconnecting to its selected AMF is temporary switched/disconnect.

# Conclusion

In this contribution, we have discussed NG interface management and feeder link switch and their impacts on UE NG related procedures. Based on the discussion we proposed to discuss the following proposals

- Proposal 1: Discuss containing UEs NG related procedure such as initial UE message, initial UE context setup as a part of NG setup/removal signalling to guarantee that AMF will not be changed for the UEs during the feeder link switch,.

- Proposal 2: Study exchanging information regarding the availability of neighbouring XN and NG to allow rerouting UEs NG associated procedures over other NG/XN links in case that NG link interconnecting to its selected AMF is temporary switched/disconnect.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Maastricht, The Netherlands, March 18-22, 2024.

- R3-243017,” LS on Support of Regenerative-based Satellite Access” SA2 (VIVO), Fukuoka, Japan, 20 - 24 May, 2024.

- R2-2405156” Regenerative payload for NR NTN” Samsung. Fukuoka, Japan, May 20th-24th, 2024

- R2-2404799” Considerations on regenerative payload in NTN” Fukuoka, Japan, May 20th-24th, 2024.