3GPP TSG-RAN WG3 Meeting #125bis	R3-245383

Hefei, China, Oct. 14th – Oct. 18th, 2024

Agenda Item:	12.2

Source: 	Canon Research Centre France

Title:	Discussion on enhancements for WAB

Document for:	Discussion

# Introduction

In this paper, we discuss some aspects related to Wireless Access Backhaul (WAB), based on the TR 38.799 on WAB [2], the WID on additional topological enhancements [3] and on agreements/notes from the previous RAN3 meeting [1].

# Discussion

## WAB unaware BH RAN nodes

A WAB node may benefit from connecting to a WAB-aware BH RAN node, especially in terms of resource management.

However, in order to ease up the adoption of WAB deployment in 5G systems, it is essential for a WAB node to be allowed to serve UEs while the co-located WAB-MT is connected to a legacy BH RAN node, i.e., a BH RAN node which is not WAB-aware.

Indeed, despite the WAB-specific services which may be offered by WAB-aware BH RAN nodes, a WAB-MT may not always be in range of such enhanced RAN nodes. For instance, this would greatly depend on the trajectory of a mobile WAB node, which may be unpredictable, or on the actual deployment of WAB-aware BH RAN nodes.

A WAB-MT may thus be handed over to a legacy BH RAN node (i.e., a RAN node which is not WAB-aware) when it is no longer in range of a WAB-aware BH RAN node, which would hence prevent, or at least limit, service interruptions at the UEs served by the WAB-gNB. In this case, the legacy BH RAN node would serve the WAB MT as a legacy UE, while the WAB-MT would keep its already-established PDU sessions, and the co-located WAB-gNB would keep on serving UEs. If the legacy BH RAN node is not able to accommodate the whole traffic associated to the served UEs, some minimum level of service may still be considered.

Proposal 1: A WAB-MT may be allowed to connect to either a WAB-aware BH RAN node or to a legacy BH RAN node.

Proposal 2: A WAB-gNB may be allowed to serve UEs when the BH RAN node serving the co-located WAB-MT is not WAB-aware.

## WAB support indication by WAB-aware BH RAN node

In order to benefit from WAB-specific services provided by a WAB-aware BH RAN node, a WAB node should be capable of prioritizing such RAN nodes when performing cell selection, at network integration or when performing connection reestablishment.

Similarly, when performing the handover of a WAB-MT, a WAB-aware BH RAN node should preferably select a target BH RAN node that is also WAB-aware.

It is therefore essential for a WAB-aware RAN node to advertise its ability to support WAB-specific features and services through both the Uu interface and, when available, the Xn interface.

For instance, a WAB support indication may be broadcast by the WAB-aware BH RAN node, for instance using SIB1 signalling, so that a WAB-MT can consider a cell where such WAB support indication is broadcast as a preferred candidate cell for further cell selection.

Besides, the WAB support indication may be exchanged between NG-RAN nodes over the Xn interface, for instance at the establishment of the Xn connection. The Xn Setup Request and Xn Setup Response messages may then include such WAB support indication.

Proposal 3a: A WAB-aware RAN node advertises a WAB support indication using for instance SIB1 signalling.

Proposal 3b: A WAB-aware RAN node may advertise a WAB support indication over the Xn interface, e.g., at Xn setup.

## WAB indication by WAB node

A WAB capability information may be used at the integration of the WAB node to the network, but also at WAB-MT handover or at WAB-MT connection reestablishment after a connection failure. For instance, a WAB-aware target or new BH RAN node may use the WAB capability indication to accept or reject a handover/reestablishment request when the UE performing such request is a WAB-MT, e.g., for multihop avoidance purpose.

In order to inform a WAB-aware BH RAN node and a BH AMF that a UE is actually the MT component of a WAB node, a WAB capability indication may therefore be introduced in the WAB-MT’s UE capability information.

The addition of a new WAB-MT type in the UE capability may be an alternative to the broadcast by the WAB-gNB of either a WAB cell indication or a barring indication for managing multiple hop avoidance

Nevertheless, the broadcast of a WAB cell indication by the WAB-gNB may be of interest. For instance, if a WAB-gNB broadcasts a WAB cell indication in SIB1, the WAB-MT of a WAB node in the vicinity node may avoid selecting this WAB cell when performing cell (re)selection. Also, the WAB-MT of a WAB node in the vicinity may refrain from performing measurements and associated reporting of WAB cells to its BH RAN node.

Proposal 4a: A WAB indication is introduced in the UE capability of a WAB-MT.

Proposal 4b: A WAB cell indication may be broadcast by a WAB-gNB using SIB1 signalling.

## Xn Connections

Between WAB-gNB and BH RAN node

RAN3 agreed that the establishment of Xn connections between a WAB-gNB and BH RAN nodes or between a WAB-gNB and surrounding RAN nodes is supported. It can follow legacy procedures, but addition of new information in these legacy procedures is not precluded.

For instance, to support coordination for resource multiplexing between a WAB-gNB and a (WAB-aware) BH RAN node, the BH RAN node serving the WAB-MT should have the knowledge of which WAB-MT is collocated with the WAB-gNB while the WAB-gNB should know the BH RAN node serving the WAB-MT it is collocated with.

Thus, some Xn procedures, such as the Xn setup procedure, may be enhanced to support the indication of the co-location of WAB-MT/WAB-gNB in a WAB-node and/or the identification of a BH RAN node serving a WAB-MT.

Moreover, one should consider that the Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 5a: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to support the sharing of some WAB-MT/WAB-gNB co-location information.

Proposal 5b: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to allow a WAB-gNB to identify the BH RAN node serving a WAB-MT.

Proposal 5c: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Between WAB-gNBs

In the context of two WAB-gNBs on top of two different vehicles having completely different trajectories, the interest of establishing Xn connection between these WAB-gNBs would be limited. In this respect, the objectives of Rel-19 WID [3] include how to avoid setting up Xn between WAB-gNBs.

However, when considering vehicles embedding several WAB nodes – e.g., a cruise ship or a train – establishing Xn connections between the WAB-gNBs bound to a same vehicle would become relevant. For instance, for the served UEs moving inside the vehicle, the handover from one WAB-gNB to another could be facilitated by having Xn connection between the source and target WAB-gNBs. Thus, it seems interesting not to preclude the establishment of Xn connections between WAB-gNBs as such connections may be relevant for some WAB deployments.

To allow or prevent the setup of an Xn connection between two WAB-gNBs, it is proposed to have an optional WAB indication shared between WAB-gNBs during the Xn setup procedure, for instance in the Xn setup request, Xn setup response, and/or Xn setup failure messages, to indicate that the sender of the message is a WAB-gNB.

Thus, a WAB-gNB receiving a Xn setup request with the WAB indication may reject the request, and send an Xn setup failure message, which includes the WAB indication. Besides, in case the WAB-gNB positively responds to the Xn setup request, the WAB indication is included in the Xn setup response message. Then, if the WAB-gNB that had initiated the Xn setup wants to avoid the multiple hop situation, it may immediately remove the Xn interface with the Xn removal procedure.

Proposal 6a: An optional WAB indication is shared between two WAB-gNBs when performing the Xn setup procedure.

Proposal 6b: An optional WAB indication may be added to Xn setup request message, the Xn setup response message and/or the Xn setup failure messages

Proposal 6c: The WAB indication shared during the Xn setup procedure can be used to avoid setting up Xn connection between WAB-gNBs.

## NG Connection Removal and Suspension

The Rel-19 WID [3] for WAB includes a note indicating that NG connection management should take the NTN conclusion into account. For Rel-19 NTN, the current conclusions for NG connection removal and suspension after RAN3#125 meeting [1] are the followings:

- NG Removal/Setup can be used for Regenerative Payload NG interface management.

- Further discussions on benefits to introduce NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companies to have 2 methods for NG interface management.

As RAN3 agreed that the NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed, we can conclude that NG removal can also be used in the WAB context (as done in the NTN context). Thus, a NG removal procedure can be specified through coordination between NTN and topological enhancement work items.

Proposal 7: NG removal procedure is conjointly specified in NTN and Topological Enhancement work items.

In addition, when considering a WAB node having a “loop” trajectory, i.e., where it is expected that the WAB node moves along a predefined path, while coming back to its starting point on a periodic basis, the existing NG connection(s) with AMF(s) may be suspended and resumed later. For some WAB nodes, there may be no strong constraints on hardware resources (as it is the case for satellites), and there would be no issue to keep in memory some parameters related to the suspended NG connections. Thus, even though NG suspend/resume is finally not supported in Rel-19 NTN, it may still be considered in the WAB context.

Proposal 8: NG suspend/resume will be considered and discussed in the scope of WAB’s work item once the conclusion is available in NTN work item.

# Conclusion

In this contribution, we have discussed some WAB-related subjects about WAB indications, Xn connection management, NG connection management. Finally, we have made the following proposals:

Proposal 1: A WAB-MT may be allowed to connect to either a WAB-aware BH RAN node or to a legacy BH RAN node.

Proposal 2: A WAB-gNB may be allowed to serve UEs when the BH RAN node serving the co-located WAB-MT is not WAB-aware.

Proposal 3a: A WAB-aware RAN node advertises a WAB support indication using for instance SIB1 signalling.

Proposal 3b: A WAB-aware RAN node may advertise a WAB support indication over the Xn interface, e.g., at Xn setup.

Proposal 4a: A WAB indication is introduced in the UE capability of a WAB-MT.

Proposal 4b: A WAB cell indication may be broadcast by a WAB-gNB using SIB1 signalling.

Proposal 5a: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to support the sharing of some WAB-MT/WAB-gNB co-location information.

Proposal 5b: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to allow a WAB-gNB to identify the BH RAN node serving a WAB-MT.

Proposal 5c: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 6a: An optional WAB indication is shared between two WAB-gNBs when performing the Xn setup procedure.

Proposal 6b: An optional WAB indication may be added to Xn setup request message, the Xn setup response message and/or the Xn setup failure messages

Proposal 6c: The WAB indication shared during the Xn setup procedure can be used to avoid setting up Xn connection between WAB-gNBs.

Proposal 7: NG removal procedure is conjointly specified in NTN and Topological Enhancement work items.

Proposal 8: NG suspend/resume will be considered and discussed in the scope of WAB’s work item once the conclusion is available in NTN work item.

# References

[1] Chair notes at RAN3#125 meeting

[2] RP-241925 - TR 38.799, Version 2.1.0

[3] RP-242395 - New WID on additional topological enhancements for NR