3GPP TSG-RAN WG3 Meeting #125bis	R3-245392

Hefei, China, 14 – 18 October, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	(TP for TS 38.300) Discussion on WAB related procedures

Document for:	Discussion

1	Introduction

The previous RAN plenary meeting agreed the new WID for topology enhancement, which includes the objectives of WAB part [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows:

-	Specifications for the support of WAB including [RAN3]:

-	Support of a WAB-node including a WAB-gNB and a WAB-MT.

-	Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s).

-	Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

-	Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

-	Network integration procedures for WAB nodes.

-	Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes.

-	Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

-	UE’s ULI that reflect the WAB node’s location.

-	The handling of:

-	PCI collision avoidance.

-	Reconfiguration of TAC and RANAC on WAB-gNBs.

-	Mechanisms to avoid multi-hop WAB topology.

-	Radio-resource coordination between access and backhaul links.

-	NG connection management.

NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB.

NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions.

NOTE 3: No impact on the UE.

NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed.

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase.

NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase.

NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

This contribution further dicusses on the mobility, authorization, integration, resource multiplexing and parameters configuration issues for WAB.

2	Discussion

2.1	WAB-node mobility support

2.1.1 How to support UE’s CN change during the mobility of WAB-node

In the RAN3 #125 meeting, companies discussed the solutions for how to support the UE’s AMF change during the WAB-node mobility. The following options were captured in the TR 38.799[2]:

- Option A: Solution with two logical WAB-gNBs.

In this solution, two logical gNBs are activated simultaneously in the WAB-node. The source logical gNB and target logical gNB respectively connects to the old AMF and new AMF. Since the two logical gNBs connect to different AMFs, the UE handover between the cells served by the two logical gNBs should be NG-based handover, and the serving AMF is re-located during the NG-based handover.

- Option B1:  Single WAB-gNB with a single cell using mobility registration update due to TAC change.

In this option, the WAB-gNB establishes a new NG connection towards the new AMF and concurrently maintains NG connections to both AMFs. The WAB-gNB reports a new TAC only to the new AMF. The WAB-gNB initiates the change of the UE’s AMF by updating the SI with the new TAC. When the UE detects the new TAC in the SI broadcast, it initiates the Mobility Registration Update procedure as defined in clause 4.2.2.2.3 of TS 23.502. After all UEs have been migrated to the new AMF, the NG connection between the WAB-gNB and the initial AMF(s) can be removed.

Fig 1. Peocedure to support UE’s AMF change based on option B1

- Option B2: Single WAB-gNB with two cells with different TACs, using NG-based HO

In this option, the procedures defined in clause 4.3.4.2.2.1 of TR38.799[3] can be reused with the difference that the new cell(s) and the old cell(s) are served by the same WAB-gNB, i.e., no new logical WAB-gNB needs to be instantiated. The gNB-ID part of the cell ID of the new cell is the same as that of the old cell. The WAB-gNB further has to report the new TAC only to the new AMF. This ensures that AMF reallocation can be achieved via the NG-based handover for RRC_CONNECTED UEs and via Mobility Registration Update for RRC_IDLE/RRC_INACTIVE UEs.

- Option B3: Single WAB-gNB single cell without TAC change.

In this option, during the AMF change, the WAB-gNB retains its TAC. When the WAB-gNB establishes an NG connection to the new AMF, the WAB-gNB indicates the TAC to the new AMF, and removes the TAC from the supported TAC list at the initial AMF. After this, the UE context transfer between the old and the new AMF is triggered, which requires enhancements in the core network (e.g., either the initial AMF or the new AMF can trigger UE context transfer for both RRC_CONNECTED and RRC_IDLE UEs, for example based on the GUAMI of the new or initial AMF, respectively).

During the study item, the conclusion from RAN3 about the solutions are listed as follows [3]

-	RAN3 concludes that the two-logical-gNB solution is feasible.

-	It remains to be verified, based on feedback from SA2 and, potentially, RAN2 if needed, whether the single-gNB solution is feasible and whether enhancements are needed.

Although RAN3 concludes that the two-logical-gNB solution is feasible, in our view, the handover-based solutions require that both the source cell and target cell should be visible to the UE at the same time, including option A and option B2. Then, it means that the two cells should share the resources allocated to this WAB-node, including the frequency resources, RF resources, baseband resources, etc. This will result in low resource utilization because only half of the available resources can be utilized by UE’s serving cell since the WAB-node needs to activate at least two cells at same time.

Regarding option B1, there was a comment during RAN3#125 meeting that how can the UE acquire the updated SI may have UE impact because the following Note in TS 38.331 “The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 and MBS broadcast if the UE can acquire it without disrupting unicast or MBS multicast data reception”. We do not observe any UE impact for the SI update, because it is already supported since LTE. For the RRC_CONNECTED UE, if the UE is receiving unicast data/MBS multicast data, it can acquire the updated SI after the data reception, no enhancement is needed. In addition, the DL data transmission is controlled by the network, it is feasible that the WAB-gNB stop to scheduling DL transmission for all the connected UEs if it is really so urgent to push all UEs to perform MRU when the TAC is updated.

Observation 1-1：With option A and option B2, if UE perfroms handover between two cells served by the WAB-node to support AMF relocation, the available resouces needs to be shared between the two cells and will leads to low resource utilization.

Observation 1-2：SA2 has concluded that the TAC broadcasted by the WAB-gNB will change if the AMF changes. The Option B3 does not align with SA2’s conclusion.

Observation 1-3：With Option B1, the network and UE behavior just follows legacy procedure of system information modification, no UE enhancement is needed.

The workflow of option 3 is given by Fig 1, and it is proposed:

- RAN3 send LS to SA2 to confirm the feasibility of the option B1, i.e., Single WAB-gNB with a single cell using mobility registration update due to TAC change.

2.1.2 Handling of WAB-gNB’s traffic during WAB-node mobility

Regarding the objective “Handling of WAB-gNB’s traffic during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes”, there are two possibilities. If the WAB-gNB’s traffic is protected by a tunnel between the WAB-node and another node which is deployed before or co-located with the termination of this traffic (e.g., IPsec tunnerl may be used between WAB-node and the SeGW, and tunnel is carried via the BH PDU session), the WAB-gNB’s IP address for the traffic is the inner IP address which is different from the IP address allocated to the WAB-MT during the PDU session management procedures. Then the BH PDU session change has no impact to the WAB-gNB’s traffic.

Another possibility is the WAB-gNB reuse the IP address allocated to the WAB-MT, if so, the WAB-gNB’s traffic should be redirected to the new IP address assigned to the WAB-MT during PDU session update. We think the redirection of the WAB-gNB’s traffic should follow the legacy procedures.

- During the mobility of WAB node, there is no impact on the the WAB-gNB’s traffic during WAB-node mobility if a tunnel carried via BH PDU session is used to protect the WAB-gNB’s traffic.

- If the WAB-gNB’s traffic reuse the WAB-MT’s IP address, the WAB-gNB’s traffic should be redirected to the new IP address using legacy procedures when the WAB-MT’s IP address is updated during mobility.

2.1.3 NG management for WAB-gNB during WAB-node mobility

Considering that the UE’s AMF may change during the WAB-node mobility, it is important to support the NG removal between the WAB-gNB and the original AMF after the AMF change. It is worth noting that the NG removal is also approved to be supported for the NTN topic, we think the general procedure should be designed for both NTN and WAB.

- RAN3 to design unified NG removal procedure for WAB and NTN.

2.1.4 Xn update for WAB-gNB during WAB-node mobility

As we discussed in the architecture part in [4], the WAB-node mobiliy will result in the dynamic change of the neighboring relationship for the WAB-gNB. Thus, it is essential for the WAB-gNB to update the Xn connection timely in order to support the UE mobility via the Xn connection.

Legacy ANR mechanism requires UE to be present in the overlap area of neighbour cells, and may be not be a time-effictive way in the WAB scenario where the cell neighboring relationship changes fast.

An alternative solution is that the BH-gNB can provides the information of neighboring gNBs (including the TNL address) to the WAB node, then the WAB-gNB can try to establish the Xn interface towards these neighboring gNBs directly. RAN3 can further investigate whether to use RRC signalling or Xn signalling for providing the neighboring gNBs’ information.

- The BH-gNB can provide the information of neighboring gNBs (including the TNL address) to the WAB node.

2.2	WAB-node integration

In TR 38.799, the network integration procedure of WAB-node is provided in clause 4.3.1. The current procedure is organized in good shape. During the WI, we would like to recommend RAN3 to take such content as start point, and capture the integration procedure in TS 38.300.

- In stage 2, RAN3 to capture the integration procedure for WAB as shown in the section 4.3.1 of TR 38.799.

2.3	WAB-node authorization

Since the authorization for the WAB-MT and WAB-gNB should be performed separately, we also need to consider two cases for the non-authorized handling, i.e., one case is WAB-MT becomes not authorized and another case is WAB-gNB is not authorized.

On one hand, the WAB-MT cannot continue to provide backhaul transmission for the WAB-gNB when the WAB-MT becomes non-authorized, so that the UP resources for the established PDU session(s) should be released. On the other hand, if the WAB-gNB is non-authorized, the WAB-gNB should not work as a gNB, e.g., the WAB-gNB needs to handover the UEs to other gNB, or release the NG/Xn connection. Under this circumstance, it is meaningless for the WAB-MT to maintain the UP resources because there is no CP/UP traffic from the WAB-gNB. Hence, the WAB-MT is required to inform the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time, then the UP resources for the established BH PDU session(s) of WAB-MT can be released.

- The UP resources for the established BH PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

2.4	Resource multiplexing and energy saving for WAB-node

For out-band mode the WAB-gNB and WAB-MT use different frequency, so that there is no interference between the WAB-gNB and the WAB-MT. For in-band mode, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated. For example, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the “Hard” resource configuration of the WAB-gNB.

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- Similar as IAB system, the BH-gNB should know the multiplexing capabilities of WAB-node, and the resource configuration of WAB-gNB. RAN3 to study Xn signalling for WAB-node resource coordination in normative phase.

Furthermore, as the Rel-18 introduced the cell DTX/DRX energy saving mechanism, the serving cell of the WAB-MT may be configured cell level DTX/DRX. It is meaningful the WAB-gNB set its own cells’ DTX/DRX referring to the serving cell’s DTX/DRX, e.g., it would be beneficial for the energy saving and reducing the storage cost that the WAB-gNB’s cells also turn to inactive when the seving cell becomes inactive. Since the cell DTX/DRX pattern is activated by L1 signalling, reporting and configuring via Xn looks not time efficient for such dynamics, and it is reasonable the WAB-gNB directly set its cells’ DTX/DRX pattern and/or activation status according to that of the serving cell. It is proposed,

- If WAB-node supports energy saving, the WAB-gNB determines the cell DTX/DRX based on the WAB-MT serving cell DTX/DRX.

2.5	Parameter configuration for WAB-node

The WID also includes the handling of PCI collision avoidance and the Reconfiguration of TAC and RANAC on WAB-gNBs. And according to the “NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB”, it can be observed that the legacy PCI assignment which includes both the centralized way and distributed way should be applicable to the WAB-gNB directly. And, as specified in TS 38.401, the PCI space can be partitioned between mobile IAB cells and stationary cells by implementation. So, it is straightforward to allow such implementation-based PCI space partition can also be supported for WAB cell. But there is no specification impact.

Regarding the TAC and RANAC reconfiguration of WAB-node, it should be the OAM to provide the configuration to the WAB-gNB, same as mobile IAB and legacy gNBs.

- The PCI, TAC, and RANAC of the WAB-gNB is (re-)configured by OAM, same as legacy.

- PCI space partition for mobile IAB is also applicable for the PCI collision avoidance of WAB-node, but up to implementation.

3	Conclusion

This paper provides mobility and other issues for the WAB-node. The proposals are as follows:

WAB-node mobility support

Observation 1-1：With option A and option B2, if UE perfroms handover between two cells served by the WAB-node to support AMF relocation, the available resouces needs to be shared between the two cells and will leads to low resource utilization.

Observation 1-2：SA2 has concluded that the TAC broadcasted by the WAB-gNB will change if the AMF changes. The Option B3 does not align with SA2’s conclusion.

Observation 1-3：With Option B1, the network and UE behavior just follows legacy procedure of system information modification, no UE enhancement is needed.

- RAN3 send LS to SA2 to confirm the feasibility of the option B1, i.e., Single WAB-gNB with a single cell using mobility registration update due to TAC change.

- During the mobility of WAB node, there is no impact on the the WAB-gNB’s traffic during WAB-node mobility if a tunnel carried via BH PDU session is used to protect the WAB-gNB’s traffic.

- If the WAB-gNB’s traffic reuse the WAB-MT’s IP address, the WAB-gNB’s traffic should be redirected to the new IP address using legacy procedures when the WAB-MT’s IP address is updated during mobility.

- RAN3 to design unified NG removal procedure for WAB and NTN.

- The BH-gNB can provide the information of neighboring gNBs (including the TNL address) to the WAB node.

WAB-node integration

- In stage 2, RAN3 to capture the integration procedure for WAB as shown in the section 4.3.1 of TR 38.799.

WAB-node authorization

- The UP resources for the established PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

Resource multiplexing and energy saving for WAB-node

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- Similar as IAB system, the BH-gNB should know the multiplexing capabilities of WAB-node, and the resource configuration of WAB-gNB. RAN3 to study Xn signalling for WAB-node resource coordination in normative phase.

- If WAB-node supports energy saving, the WAB-gNB determines the cell DTX/DRX based on the WAB-MT serving cell DTX/DRX.

Parameter configuration for WAB-node

- The PCI, TAC, and RANAC of the WAB-gNB is (re-)configured by OAM, same as legacy.

- PCI space partition for mobile IAB is also applicable for the PCI collision avoidance of WAB-node, but up to implementation.

4. Reference

- RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T.

- 3GPP TR 23.700-06 v 19.0.0, Study on architecture enhancements for vehicle-mounted relays - Phase 2.

- 3GPP TR 38.799 v 19.0.0, Study on additional topological enhancements for NR.

- R3-245391, (TPs for TS 38.300/38.413) Architecture and Access control for WAB, Huawei.

Annex A——TP for TS 38.300

Start of Change

### 4.X.Z	WAB-node integration procedure

Figure 4.X.Z-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

End of Change

Annex B——LS to SA2 on the WAB mobility

3GPP TSG-RAN WG3 Meeting #125-bis	TDoc <TDoc#>

Hefei, CN, 14 - 18 Oct, 2024

Title:	[DRAFT] LS on WAB mobility

Response to:

Release:	Rel-19

Work Item:	NR_WAB_5GFemto

Source:	Huawei [will be RAN3]

To:	SA2

Cc:

Contact person:	Yuanping Zhu

zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN3 discussed how to support UE’s CN change during the mobility of WAB-node, besides the two logical WAB-gNB based solution. There is also a single WAB-gNB based solution, which is described as follows

- Option B1:  Single WAB-gNB with a single cell using mobility registration update due to TAC change.

In this option, the WAB-gNB establishes a new NG connection towards the new AMF and concurrently maintains NG connections to both AMFs. The WAB-gNB reports a new TAC only to the new AMF. The WAB-gNB initiates the change of the UE’s AMF by updating the SI with the new TAC. When the UE detects the new TAC in the SI broadcast, it initiates the Mobility Registration Update procedure as defined in clause 4.2.2.2.3 of TS 23.502. After all UEs have been migrated to the new AMF, the NG connection between the WAB-gNB and the initial AMF(s) can be removed.

RAN3 want to ask SA2 to check the feasibility of the above option.

# 2. Actions:

# To SA2:

ACTION: RAN3 kindly asks SA2 to provide feedback on the feasibility of the option B1 for support the change of UE’s AMF during the WAB-node mobility .

# 3. Date of Next RAN3 Meetings:

RAN3#126	18th Nov – 22nd Nov 2024				Orlando, US

RAN3#127	17th Feb – 21st Feb 2024				Athens, GR