3GPP TSG-RAN WG3 Meeting #125bis	R3-245394

Hefei, China, 14 – 18 October, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for TS 38.300) Access control for NR Femto

Document for:	Discussion

1	Introduction

The previous RAN #105 meeting agreed the new WID for topology enhancement, which includes the objectives of femto part [1]:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

.

In this contribution, we mainly focus on the Access control of NR Femto.

2	Discussion

----------excerpt from TR 38.799----------

5.3	Access control

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

During the SI phase of the Topology enhancement, it concludes that “For Access control, NR Femto reuses the existing CAG functionalities, no stage3 impact has been identified.” And, in the TR 38.799[2], how to support the different access modes based on the existing CAG functionality has been clearly captured in clause 5.3.

According to the NOTE  10 in the WID, only stage 2 impact is expected for the access control part. So it is suggested that RAN3 to capture the access control content for NR femto in TS 38.300.

As captured in the TR 38.799, the existing CAG functionality can support open, hybrid and closed modes for NR Femto.

- RAN3 to agree the stage 2 TP to capture the access control part for NR Femto.

3	Conclusion

This paper provides analysis for the access control of NR femto, the following observation and proposal are obtained:

- Access control

- As captured in the TR 38.799, the existing CAG functionality can support open, hybrid and closed modes for NR Femto.

- Proposal 1:	RAN3 to agree the stage 2 TP to capture the access control part for NR Femto.

4. Reference

- RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T.

- 3GPP TR 38.799 v 19.0.0, Study on additional topological enhancements for NR.

Annex——TP for TS 38.300

Start of Change

4.X.Y	Access control for NR Femto

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

End of Change