3GPP TSG-RAN WG3 Meeting #125-bis 	R3-245400

# Hefei, China, 14. - 18.10.2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1199 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the usage of the UE History Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, CMCC, Deutsche Telekom, Verizon, NTT Docomo, ZTE, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the NGAP (and in XnAP), the description of UE History Information refers to “active state”. This is a description copied from X2AP, where it was added already in Rel-8. At the time, the LTE RRC had two possible states: RRC_IDLE and RRC_CONNECTED. Thus, the “active state” was clear. However, in NR, the term may be ambiguous: RRC has two states where UE may be “active”, the RRC_CONNECTED and the RRC_INACTIVE. The name of the latter suggests the UE is not “active”, but if looked at from the CN perspective, the CM-CONNECTED does include RRC_INACTIVE. Also, the RRC_INACTIVE, in some scenarios (when UE is “active”) does enable tracing cells that the UE uses. Thus, it may be unclear when exatly the UE History Information is to be collected. The original purpose of the UE History Information was to enable tracing ping-pong handovers. Thus, it can safely be assumed the term “active state” refers to the states where UE’s mobility is based on handovers. Also, the fact that it was introduced in LTE when the RRC_INACTIVE was not yet defined hints that the usage of the UE is limited to RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to clarify that the UE History Information is to be collected in RAN only when UE is in RRC_CONNECTED, it is proposed to replace “active” with “RRC_CONNECTED”. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only clarifies usage of the IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity concerning usage of the IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.95 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1381 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 9.3.1.95	UE History Information

This IE contains information about cells that a UE has been served by in RRC_CONNECTED state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >Last Visited Cell Information | M |  | 9.3.1.96 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |

Remaining text not changed