3GPP TSG-RAN WG3 Meeting #125-bis	R3-245404

Hefei, China, October 14-18, 2024

Agenda Item:	14.3

Source:	THALES

Title:	Discussion on interfaces mobility aspects for regenerative payload

Document for:	Discussion

# 1	Introduction

The approved work item justifications for NR NTN Evolution in Rel-19 [1] emphasizes that regenerative payloads architectures bring more flexibility in the NTN deployment besides transparent payload in Rel-17 and Rel-18:

4) Support non-terrestrial network architecture with 5G system functions on board the NTN vehicle (i.e. regenerative payloads). Support of non-terrestrial network architecture with 5G system functions on board the NTN vehicle (i.e. regenerative payloads) provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. Support of regenerative payload brings some benefits on radio resource handling in Uu, and radio resource coordination between the gNBs via the ISL. For the support of real time connectivity between 2 UEs and between network and UE via the space segment with/without ISL, a regenerative payload (5G system functions on board satellite) is required.

For that, the support of gNB on board is specified in TS 38.300 is one of the objectives defined in the WI:

4. Support of regenerative payload [RAN3, RAN2, RAN4] • Specify the support of gNB on board in TS 38.300 • Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] • Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

RAN3 will specify any necessary enhancements related to mobility Xn and NG interface.

In the study phase report to support integration of satellite components in the 5G architecture (phase 3) published in June 2024 [2], the SA2 concluded the following for the support of regenerative payload without workload mainly for RAN3:

The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

SA2 confirmed the reference architecture for the regenerative payload as defined in the adopted baseline CRs in RAN2 and RAN3. The gNB is embarked in the satellite vehicule (SV) and the 5GC is on the ground. The feeder link carries the NG to the ground. InterSatellite Link (ISL) should be used for NG in the context of NGSO constellation when the gateway (GW) is not in visibility to the SV.

In this paper, we discuss the NG and Xn interface mobility aspect for regenerative payload.

# 2	Discussion

In RAN2 and RAN3 was already agreed the following baseline architecture for regenerative payload:

Figure 1 - Overall illustration of an NTN with Regenerative Payload hosting a gNB

In this architecture, baseline architecture, the NG interface is carried via the feeder link between the gNB and the Core Network and the Xn between gNBs is carried via Inter-Satellite Link (ISL). Both interfaces are subjects to mobility due to satellite movements.

### NG Interface

In RAN3#125, it was agreed the following:

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companie to have 2 methods for NG interface management.

For the NG interface Setup, the NG Setup defined in [5]-§8.7.1 is re-used. The procedure is called after the the Transport Network Layer (TNL) association and is not associated to a UE. It is expected to be triggered at each feeder link switch. The TNL for NG Control Plane is built on IP transport and SCTP for reliability of signalling messages on top of IP. During the NG interface setup, the NG SETUP REQUEST message is sent by the gNB to the AMF. The NG SETUP REQUEST message [5]-§9.2.6.1 contains at least the Global RAN Node ID, the Supported TA List (TAC and PLMN Identity), the Default Paging DRX, all provided by the PDCP-CP entity. In response, the AMF setups a context associated to the gNB (e.g. UE context) and sends a NG SETUP RESPONSE message containing the AMF Name, the GUAMI name (Globally Unique AMF ID) and the PLMN Support List. The NG Setup request and NG Setup response are typically < 200 bytes. Two complete gNB-AMF RTT are necessary to establish the NG interface. The interruption time impact is limited to few milliseconds in NGSO and the overhead on the feeder link is very limited in terms of throughput.

Observation 1	The latency to setup the NG Setup interface is at least equal to two gNB-GTW-AMF RTTs and the signalling overhead is an order of few hundreds bytes

The reason why the gNB is eager to remove a NG interface is:

- The satellite is not yet covered by the gateway providing connectivity to the AMF through the feeder link and the ISLs are not used or available.

- The satellite changes its zone of service and need to re-select to a closer/more appropriate AMF corresponding to the new zone covered

From the UE point of view. In the first case, they remain under the cover of the same satellite and the AMF/UPF changed, it applies the legacy AMF (re-)selection procedure. The gNB should perform NG handover to change to an appropriate AMF, or initiate an UE Context Release towards the serving AMF. In the second case, the UE should move to another satellite that cover the area.

Proposal 1	From the UE point of view, the legacy procedures (NG Handover, AMF re-selection) can be re-used without more enhancements for NG interface mobility

For the NG interface Removal, there is no procedure defined in [5]. In terrestrial network, once setup, the NG interface remains up except if the link is cut. In this case, it is handled by the SCTP shutdown procedure and then, shut down the remaining NG interface. Since the gNB knows, based on its ephemeris and configuration when the feeder link will be down, it can trigger handover or release of its served UEs using legacy procedures and triggers an NG Removal procedure toward the AMF. The TP in [6] provide a NG Removal procedure with 3 messages, NG REMOVAL REQUEST, NG REMOVAL RESPONSE, and NG REMOVAL FAILURE. For the feeder link switchover, the AMF re-selection and handovers could be triggered before the loss of the source feeder link connectivity while for hard feeder link switch over, the UE AMF re-selection/handover is triggered after the feeder link switch.

Figure 2 - NG removal procedure

Proposal 2	Adopt the TP in R3-245179 as baseline for NG Removal procedure in TS 38.413

Figure 3 - The NG interface through ISL in the Regenerative Payload architecture

To be complete with the NG interface mobility, it is to be noted that, depending to the implementation of the constellation, the NG interface could be carried through ISL. The NG interface reliability through ISL is ensured by the IP packets (re)routing and the SCTP protocol, and is outside of the 3GPP scope.

Observation 2	The NG interface can be carried through ISL if no feeder link are available. No 3GPP impacts.

### Xn Interface

Xn interface support intra-NG-RAN mobility. The Xn interface could be supported by the ISL between two satellites. Typically, a satellite is equipped with 4 ISLs, two for satellites in intra-plane and two for satellites in neighbor planes. The ISL distance could go from hundreds to few thousands km. The switchover is typically hard switchover with an interruption time during repointing of the radio/optical head. The satellite relative movement may result in an Xn switch over, even during a handover, which may cause a failure in UE context acquisition. It could be handled by the TNL (i.e. SCTP) or NG handover procedure for Xn handover failure.

In the number of ISL switchover in the plane are limited (the relative position of the satellite remains likely the same), but could be important in the inter-plane. The Xn setup [7]-§8.4.1 and Xn removal [7]-§8.4.6. The procedure, likely to NG implies the establishment of the link (Tx/Rx head pointing), the establishment of the TNL (i.e. SCTP SETUP) and the XN SETUP REQUEST / XN SETUP RESPONSE. The establishment is performed in two RTT if we neglect the link establishment, i.e. up to 40 ms. The overhead for the gNB is limited by the fact that there is only 2 ISLs that present frequent switch over.

Figure 4 - Polar orbit LEO (extracted from DOI: 10.1016/j.aeue.2010.08.008 )

Let’s take a hypothetical example with a polar constellation with an important frequent ISL switch over in inter-plane as illustrated in Figure above.

- LEO-600 Set1

- 600 km with 30° elevation, meaning a 1860 km coverage per satellite

  - Speed of the satellite = 7.561 km/s

- 20 planes and 40 satellites per plane (minimum to cover all Earth, 800 satellites), inclination 85°

  - Intra-plane satellite distance ~1090 km

  - Inter-plane satellite distance at Equator ~2150 km

    - Satellite from a plane is connected to a satellite of the other plane for ~1100 km

  - Inter-plane satellite distance at the polar region boundary (=75°) ~280 km

    - Satellite from a plane is connected to a satellite of the other plane for ~300 km

In this simplified example, the frequency of ISL switch over at the equator is 145 sec and 40 sec at the poles. So the interruption time for Xn setup is negligible compared to the time the link is active.

Observation 3	The frequency of the ISL switch over may occur every minute or more, according to the network topology and should not lead to an overload of the gNB due to the signaling

Moreover, the satellite can prepare the UE context transfer and handover in advance, since the ISL service time is predictable. With regard of that, the risk of handover failure is limited. When an ISL is near to be done and all the mobility are done, the gNB can proceed to a Xn Removal to remove the interface between two gNB in a controlled manner, i.e. erasing any existing application level configuration data in the two node.

Observation 4	The ISL switch over are predictable by the gNB according to the topology provided by the OAM, so the gNB can prepare Xn mobility at ISL switch over event

Proposal 3	The legacy Xn setup and Xn removal are re-used for the Xn interface mobility handling without further enhancements

Figure 5 - Xn over the ground in regenerative payload architecture

According to the network topology, the Xn interface between two given satellites can benefit of the ground network, e.g. no direct route through multi-hop ISL. In this case, the feeder link is used to carry the Xn interface. The regenerative payload should be always connected to the core network through feeder link (possibly via ISL). It implies a long RTT and jitter at re-routing that is handled by the TNL (i.e. SCTP) but long RTT may affect the legacy Xn mechanism in Xn-C and Cn-U.

Observation 5	The ground network could be used when no direct ISL link is available between two given satellite according to the network mobility

Proposal 4		The specifications does not preclude the usage of the feeder link to carry Xn interface. No impacts on the RAN3.

# 3	Conclusion

In the previous sections we made the following observations and proposals:

<NG interface management>

Observation 1	The latency to setup the NG Setup interface is at least equal to two gNB-GTW-AMF RTTs and the signalling overhead is an order of few hundreds bytes

Proposal 1	From the UE point of view, the legacy procedures (NG Handover, AMF re-selection) can be re-used without more enhancements for NG interface mobility

Proposal 2	Adopt the TP in R3-245179 as baseline for NG Removal procedure in TS 38.413

Observation 2	The NG interface can be carried through ISL if no feeder link are available. No 3GPP impacts.

<Xn interface management>

Observation 3	The frequency of the ISL switch over may occur every minute or more, according to the network topology and should not lead to an overload of the gNB due to the signaling

Observation 4	The ISL switch over are predictable by the gNB according to the topology provided by the OAM, so the gNB can prepare Xn mobility at ISL switch over event

Proposal 3	The legacy Xn setup and Xn removal are re-used for the Xn interface mobility handling without further enhancements

Observation 5	The ground network could be used when no direct ISL link is available between two given satellite according to the network mobility

Proposal 4		The specifications does not preclude the usage of the feeder link to carry Xn interface. No impacts on the RAN3.

# References

RP-24187, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#105, 2024-09, Melbourne, AU.

TR 23.700-29, “Study on integration of satellite components in the 5G architecture; Phase 3”, SA#104, 2024-06, Shanghai, CN.

R3-243949, “Support for Regenerative Payload in NR NTN”.

R3-244031, “BL CR Support for Regenerative Payload in NR NTN”.

3GPP TS 38.413, “5G; NG-RAN; NG Application Protocol (NGAP)”, Rel-18.

R2-244108, “(TP for TS 38.413) Introduce NG Removal procedure”.

3GPP TS 38.423, “5G; NG-RAN, Xn Application Protocol (XnAP)”, Rel-18.