3GPP TSG-RAN WG3 Meeting #125bis	R3-245413

Hefei, China, 14th – 18th October, 2024

Title:	Discussion on the LS on terminology definitions for AI-ML in NG-RAN

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In SA5#156 meeting (August 2024) SA5 approved the LSout in [1], which is a reply to the LS in [2] that RAN3 sent to make SA5 aware of the fact that RAN3 agreed to introduce in Stage 2 specifications (TS 38.300 and TS 38.401) references to SA5 definitions for AI/ML Model Training and AI/ML Model Inference which follow the definition of “ML model training” and “AI/ML inference” as specified in clause 3.1 of TS 28.105, respectively.

In [1] SA5 informs RAN3 (and RAN2) that SA5 has recently discussed and agreed further clarifications to the terminologies in TS 28.105 and that the corresponding approved CRs to this specification have now been reflected in the v18.4.0 version of TS 28.105.

This contribution discusses the impact on the AI/ML-related definitions (i.e., AI/ML Model Training and AI/ML Model Inference) used by RAN3 in the context of the Rel-18 and Rel-19 discussions on AI/ML for NG-RAN in relation with the latest changes introduced by SA5 concerning the AI/ML terminology.

# 2	Discussion

In RAN3#124 meeting (May 2024) RAN3 agreed CRs to Rel-18 versions of TS 38.300 and TS 38.401 in [4] and [5], respectively, to provide definitions of “AI/ML Model Training” and “AI/ML Model Inference”. These terms, in facts, were used in RAN specifications for the Stage 2 description of the AI/ML for NG-RAN feature, without a proper definition which would have made the specifications unclear.

To this end, and considering the need to have an AI/ML framework as much harmonized as possible among 3GPP working groups actively working on this subject, RAN3 decided that it would have been better to refer to the AI/ML terminology as specified by SA5 in TS 28.105.

As can be deduced by checking clause 3.1 of TS 28.105, SA5 introduced several AI/ML-related definitions to cover basically all the management aspects of AI/ML models. But at least from RAN3 perspective, the relevant definitions were the ones that could have a good “matching” with the definitions of “AI/ML Model Training” and “AI/ML Model Inference” as agreed during the Rel-17 “Study on enhancement for Data Collection for NR and EN-DC” Study Item whose outcome is summarized in TR 37.817 [6].

Within the TR 37.817, RAN3 defined the following:

## 3.1 Terms **** skip unrelated text **** - AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference. - AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

while in clause 4.2 of TR 37.817 RAN3 defined the functions that performs “AI/ML Training” and “AI/ML Inference”, termed as “AI/ML Model Training” and “AI/ML Model Inference”, respectively, as part of the functional framework for the “RAN intelligence”. Then, for all use cases addressed during the Rel-17 Study Item phase, RAN3 captured the location of these functions based on the agreed deployment scenarios.

The deployment scenarios were specified (i.e., reflected from TR 37.817) in both Rel-18 initial versions of TS 38.300 and TS 38.401, were the terms “AI/ML Model Training” and “AI/ML Model Inference” were used but not defined. This issue was solved by introducing references to the following definitions of “ML model training” and “AI/ML inference” in both RAN specifications as they were defined in TS 28.105 v.18.3.0.:

## 3.1 Terms **** skip unrelated text **** ML model training: process performed by an ML training function to take training data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. **** skip unrelated text **** AI/ML inference: refers to the process of running a set of input data through a trained ML entity to produce set of output data, such as predictions.

As stated before, SA5 informed RAN3 (and RAN2) via the LSout in [1] that SA5 has revised the AI/ML-related terminologies in TS 28.105 and that the corresponding approved CRs to this specification have been reflected in the v18.4.0 version of TS 28.105 as follows:

## 3.1 Terms **** skip unrelated text **** ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. **** skip unrelated text **** AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions. NOTE 5: the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

By comparing the definitions of “ML model training” and “AI/ML inference” in v18.3.0 of TS 28.105 and the updated definitions in v18.4.0 of the same specification, it can be noted that the latter ones better reflect the definitions that RAN3 introduced in TR 37.817, especially with reference to the “AI/ML Model Training”. This because SA5 also clarified in the updated definition of “ML model training” that, because of this process, a trained ML model is generated, exactly as in the RAN3 definition in TR 37.817.

Observation 1: The updated definition of “ML model training” in TS 28.105 v18.4.0 better reflects the RAN3 understanding of “AI/ML Model Training” especially in regards of the outcome of the associated process (i.e., a trained ML model), hence it still makes sense to refer to this definition for the “AI/ML Model Training” in RAN specifications.

When it comes to the updated definition of “AI/ML inference”, also in this case the changes introduced by SA5 do not impact the RAN3 definition of “AI/ML Model Inference” as in TR 37.817. Instead, it can be noted that, in the updated AI/ML-related terminology, SA5 removed the definition of “ML entity” – which was used by SA5 in the “AI/ML inference” definition – and only kept “ML model”. From our perspective this change is useful as it removes any ambiguity that could derive from having “ML model” and “ML entity” being used in “ML model training” and “AI/ML inference” definitions, respectively. Therefore, by only mentioning “ML model” in such definitions improves the consistency and harmonizes the understanding of training and inference across working groups.

Observation 2: The updated definition of “AI/ML inference” in TS 28.105 v18.4.0 does not impact the RAN3 understanding of “AI/ML Model Inference”. Instead, it removes any ambiguity on the use of “ML entity” with respect to “ML model” as done in the “ML model training” definition, hence improving consistency across definitions. From RAN3 perspective it still makes sense to refer to this definition for the “AI/ML Model Inference” in RAN specifications.

In the definition of “AI/ML Inference” in TS 28.105, the NOTE5 was added in v18.4.0 by SA5 as follows:

## 3.1 Terms **** skip unrelated text **** AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions. NOTE 5: the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

In our view such NOTE5 results from a concern expressed in the RAN3 LSout in [2] due to the fact that SA5 refers to “ML” for training and “AI/ML” for inference without a clear motivation, while from RAN3 perspective the prefix “AI/ML” is used for both training and inference and, in the LSout, RAN3 asked to provide feedback on that.

To this end, our understanding is that SA5 clarified in NOTE5 that inference is a process involving at least a trained ML model along with “other AI enablers” if needed; therefore, to cover both cases as follows:

- inference based on a ML model only, as well as

- inference based on ML model and “other AI enablers”

SA5 uses the prefix “AI/ML” for the inference process.

It could be argued that the meaning of “other AI enablers” could be unclear and not captured in TS 28.105. However, if this is really an issue, it should be discussed and solved in SA5 or as part of the SA-level Rel-19 Study Item on “3GPP AI/ML Consistency Alignment” [7].

Observation 3: SA5 clarified in NOTE5 within the “AI/ML inference” definition about the differences in terminology concerning the use of the “ML” and “AI/ML” prefixes. This clarification does not have any impact on the decision from RAN3 to refer to relevant SA5 definitions for “AI/ML Model Training” and “AI/ML Model Inference” in RAN specifications.

Based on the above analysis and Observations we have the following proposal:

Proposal: RAN3 to confirm that, from RAN3 perspective, the updated definitions for “ML model training” and “AI/ML inference” in TS 28.105 are still valid to be used in RAN specifications as references for “AI/ML Model Training” and “AI/ML Model Inference”, respectively.

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: The updated definition of “ML model training” in TS 28.105 v18.4.0 better reflects the RAN3 understanding of “AI/ML Model Training” especially in regards of the outcome of the associated process (i.e., a trained ML model), hence it still makes sense to refer to this definition for the “AI/ML Model Training” in RAN specifications.

Observation 2: The updated definition of “AI/ML inference” in TS 28.105 v18.4.0 does not impact the RAN3 understanding of “AI/ML Model Inference”. Instead, it removes any ambiguity on the use of “ML entity” with respect to “ML model” as done in the “ML model training” definition, hence improving consistency across definitions. From RAN3 perspective it still makes sense to refer to this definition for the “AI/ML Model Inference” in RAN specifications.

Observation 3: SA5 clarified in NOTE5 within the “AI/ML inference” definition about the differences in terminology concerning the use of the “ML” and “AI/ML” prefixes. This clarification does not have any impact on the decision from RAN3 to refer to relevant SA5 definitions for “AI/ML Model Training” and “AI/ML Model Inference” in RAN specifications.

Proposal: RAN3 to confirm that, from RAN3 perspective, the updated definitions for “ML model training” and “AI/ML inference” in TS 28.105 are still valid to be used in RAN specifications as references for “AI/ML Model Training” and “AI/ML Model Inference”, respectively.

# 4	References

- R3-245023 (S5-244660), LS on terminology definitions for AI-ML in NG-RAN, NEC

- R3-243969, LS on terminology definitions for AI-ML in NG-RAN, Ericsson

- TS 28.105 “Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management”, Rel-18, v18.4.0 (2024-06)

- R3-243928, Stage 2 corrections for AI/ML for NG-RAN, Rel-18 CR to TS 38.300, Huawei, Nokia, Ericsson, InterDigital, NEC, Deutsche Telekom, Telecom Italia, CATT, Lenovo, ZTE, Samsung

- R3-243966, Correction on definition of AI/ML terminology, Rel-18 CR to TS 38.401, Samsung, ZTE, China Telecom, China Unicom, Telecom Italia, Huawei, Nokia, Deutsche Telekom, CMCC, CATT, Ericsson, NEC

- TR 37.817 “Study on enhancement for Data Collection for NR and EN-DC”, Rel-17

- SP-240970, Study on 3GPP AI/ML Consistency Alignment, Rel-19, Deutsche Telekom, AT&T, BT, CAICT, CATT, China Mobile, CKH IOD UK LIMITED, DISH Network, DSIT, ETRI, ICS, KDDI, KPN, KT corp., LG Uplus, MATRIXX Software, NEC, NTT DOCOMO, NVIDIA, Orange, Qualcomm, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telstra, T Mobile USA, UScellular, Verizon, vivo, Vodafone