3GPP TSG-RAN WG3 Meeting #125bis	R3-245414

Hefei, China, 14th – 18th October, 2024

Title:	Discussion on the Reply LS on improved KPIs involving end-to-end data volume transfer time analytics

Agenda Item:	8.3

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In SA5#156 meeting (August 2024) SA5 approved the LSout in [1], which is a reply to the LS sent by RAN3 in [2] summarizing the RAN3 evaluation of the newly introduced TS 28.558 “UE level measurements for 5G system” with respect to the previous TS 28.552 “5G performance measurements”, and some discrepancies between the contents of the two specifications were highlighted. Moreover, in [2], RAN3 provided some observations related to the usage of the 5G-S-TMSI as the measured UE identifier for UE level measurements in NG-RAN.

At the same time, since RAN3#124 meeting (May 2024), RAN3 is discussing how to properly derive UE-level UE performance metrics introduced as part of the Rel-18 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [3] and a clear conclusion is still missing.

In this contribution we provide our views on the SA5 LSout content in [1] as well as a way forward for the UE performance metrics definition.

# 2	Discussion

## 2.1 On misalignment between TS 28.558 and TS 28.552

In RAN3#124 meeting (May 2024) RAN3 sent the LS to SA5 in [2] following the request from SA5 in [4] where, in the ACTION to RAN2 and RAN3, SA5 asked to consider future update of the references in relevant RAN TSs (e.g., 37.320, 38.314) pointing to TS 28.558 rather than TS 28.552 as follows:

# 2 Actions **** Skip non-relevant text **** To RAN2, RAN3 ACTION: All of the NG-RAN UE level measurements defined in draft TS 28.558 have been already supported by the MDT for NR (TS 37.320). However, RAN specs (e.g., 37.320, 38.314) refer to TS 28.552 in the definitions of certain MDT measurements, SA5 would like to inform RAN2 and RAN3 that a proper reference for those measurements would be TS 28.558. SA5 kindly requests RAN2 and RAN3 to take this into account and consider future update of the references in relevant RAN TSs once the TS 28.558 is published.

Based on the above ACTION in [4], RAN3 evaluated v18.0.0 of the new SA5’s TS 28.588 published in April 2024 for the UE level measurements performed in NG-RAN – whose content in mainly inherited from TS 28.552 – and made a comparison with respect to the content in TS 28.552. The results of such evaluation were communicated to SA5 as excerpted from [2] and reported below:

1. Overall Description: RAN3 thanks SA5 for their LS. RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558: - TS 28.558 does not define M4 measurement. - There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases: - The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558. - The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558. - The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558. From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552. **** Skip non-relevant text ****

In the Reply LS in [1] SA5 informed RAN3 that the misalignment between TS 28.558 and TS 28.552 has been corrected in v18.1.0 of TS 28.558, where also the missing M4 measurement was added.

Observation 1: Following RAN3 indications, SA5 updated TS 28.558 to v18.1.0 to correct the misalignment between TS 28.552 and TS 28.558 v18.0.0 concerning some UE level measurements performed in NG-RAN, and introduced the missing M4 measurement as well.

## 2.2 On the usage of (5G-)S-TMSI as “Measured UE Identifier”

As part of the RAN3 evaluation of TS 28.558 v18.0.0, RAN3 noticed that, also for UE level measurements performed in NG-RAN, SA5 introduced a new field in the measurement template – field g) Measured UE Identifier – which indicates the type of the UE identifiers needs to be provided among IMSI, IMEI, SUPI, N4 Session ID or RAN UE ID etc. [5]:

Annex A (normative): Template for definitions of UE level measurements Following is the template for defining the UE level measurements for 5G system. Clause title: descriptive measurement name This is a descriptive name of the measurement type that is specified as clause X of the present document. a) Description This subclause contains an explanation of the measurement. **** Skip non-relevant text **** g) Measured UE Identifier This element indicates what type of the identifiers of the UE needs to be provided. The type of the UE identifier could be IMSI, IMEI, SUPI, N4 Session ID or RAN UE ID, etc., subject to the specific measurement. h) Purpose This optional clause aims at providing additional information on the interest of the measurement but is purely indicative.

Basically, for all UE level measurements performed in NG-RAN as specified in v18.0.0 of TS 28.558 clause 6.3, the (5G-)S-TMSI is used as “Measured UE Identifier”, including those ones performed by gNB-DU and gNB-CU-UP. Based on that, RAN3 in [2] also provided some observations on the actual feasibility of using the (5G-)S-TMSI as the measured UE identifier in UE level measurements in NG-RAN:

1. Overall Description: **** Skip non-relevant text **** Also, RAN3 made the following observations concerning the S-TMSI identifier: - “S-TMSI” should be modified to “5G-S-TMSI”. - S-TMSI is not available in the gNB-DU and in the gNB-CU-UP. - S-TMSI may not be always available in the gNB-CU-CP.

For the above observations, SA5 provided further indications in [1] on what the “Measured UE Identifier” is: it is the Trace Target sent in Trace Activation which, as specified in TS 28.662, is applicable to Trace, signalling-based MDT and 5GC UE level measurements collection only. Therefore, SA5 clarified that the “Measured UE Identifier” parameter specified in TS 28.558 v18.0.0 is not applicable to UE level measurements performed in NG-RAN, and set the “Measured UE Identifier” as “N/A” (Not Applicable) instead of “S-TMSI” for the NG-RAN UE level measurements in v18.1.0 of TS 28.558 as excerpted from [1]:

# 1 Overall description **** Skip non-relevant text **** For the following additional RAN3 observations on the S-TMSI identifier, Also, RAN3 made the following observations concerning the S-TMSI identifier: - “S-TMSI” should be modified to “5G-S-TMSI”. - S-TMSI is not available in the gNB-DU and in the gNB-CU-UP. - S-TMSI may not be always available in the gNB-CU-CP. **** Skip non-relevant text **** The bullet g specified in TS28.558 is the “Measured UE Identifier”. It is Trace Target which is sent in Trace Activation. As specified in TS 32.422, IMSI or IMEISV or IMEI-TAC or SUPI is used as Trace Target in signalling-based MDT activation procedure in 5GC and NG-RAN, and the Trace Target for management-based NR MDT activation has been defined in TS 28.622. So, the “Measured UE Identifier” parameter specified in TS28.558 is not applicable to NG-RAN UE level (MDT) measurements. Therefore, SA5 has agreed to add a clarification in TS28.558 that the measurement bullet g) is only applicable for management-based activation of 5GC UE level measurement collection, and the bullet g) in NG-RAN UE level measurements are set to “N/A” instead of “S-TMSI”, please see the agreed CRs attached.

Observation 2: Following RAN3 observations on the (5G-)S-TMSI used as the “Measured UE Identifier” for UE level measurements in NG-RAN, SA5 clarified that such parameter in TS 28.558 is only applicable to 5GC UE level measurements collection. Hence SA5 updated TS 28.558 to v18.1.0 by setting “Measured UE Identifier” to “N/A” instead of “S-TMSI” for all UE level measurements performed in NG-RAN.

In [1] SA5 further expressed some opinions about possible benefits from having a temporary identifier in the management-based MDT measurement report, as reported below:

# 1 Overall description **** Skip non-relevant text **** For the following additional RAN3 observations on the S-TMSI identifier, **** Skip non-relevant text **** SA5 would like to thank RAN3 for the observation. NR MDT Trace Record Content and Trace Record Header are specified in TS32.423. SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE. **** Skip non-relevant text ****

In our view the benefits of having a temporary (UE) identifier in the management-based MDT measurement report is in the scope of the Continuous MDT topic, being one of the leftovers from the Rel-18 WI on RAN AI/ML which has been selected for Rel-19 normative phase of AI/ML for NG-RAN [6]. We provided our views on Continuous MDT in our contribution in [7].

Observation 3: Even though SA5 updated TS 28.558 to v18.1.0 by setting “Measured UE Identifier” to “N/A” instead of “S-TMSI” for all UE level measurements performed in NG-RAN, SA5 sees some benefits in having a temporary (UE) identifier in the management-based MDT measurement report, being unique within the lifetime of the measurement session, so that it could be possible to indicate that the measurements reports are from the same or different UEs.

Observation 4: RAN3 will discuss whether and how a temporary (UE) identifier could be introduced in management-based MDT measurements reports as part of the Rel-19 normative phase for the Continuous MDT feature to address the issue of trace correlation in TCE.

## 2.3 On UE performance feedback in Rel-18 AI/ML for NG-RAN

In the context of the Rel-18 “AI/ML for NG-RAN” RAN3 introduced in TS 38.423 [8] a set of metrics representing the UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision, i.e., handover. Such UE performance could be used as feedback information to the source gNB for further optimization of the AI/ML model which recommended the previous UE’s mobility action from source gNB to target gNB.

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. **** Skip non-relevant text ****

The above metrics, i.e., Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL, are intended to be collected by the target gNB within a short time window starting immediately after the handover and on a per-UE basis. However, there is no clear understanding on how a gNB could perform such per-UE measurements because the corresponding measurement definitions in relevant specifications, e.g., TS 28.558 and TS 37.320, are specified for a different granularity, i.e., not per-UE.

Observation 5: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

In RAN3#124 meeting (May 2024) the contribution in [9] made an attempt to address the issue of how the gNB could derive the UE-level UE performance metrics introduced by RAN3 and, among the proposed options, it was captured in RAN3#124 meeting minutes the common understanding that, for Rel-18, UE level performance is derived as the average value of per-DRB level measurements of the UE performance metrics [10].

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

In last RAN3 meeting (RAN3#125, August 2024) the CR to TS 38.423 in [11] was submitted to add a NOTE in TS 38.423 clause 9.2.3.179 clarifying that the value of each IE of the UE Performance results from averaging the corresponding DRB level measurements over all configured DRBs in the UE, which is in line with the common understanding in RAN3 as captured in [10]. However, the CR was not pursued due to the concerns of some companies that such a NOTE may create issues when dealing with the Rel-19 normative phase discussions related to the finer granularity of UE performance metrics for the AI/ML-based Network Slicing use case.

During the pre-meeting offline discussion among RAN3 companies involved in this UE performance granularity discussion, a revised version of the CR to TS 38.423 was proposed where it is clarified that the NOTE above only applies to UE-level UE performance metrics. However, it was argued that TS 28.558, clause 6.3.1.4.1 (6.3.1.4.2), actually refers to the Average DL UE Throughput in gNB (Average UL UE Throughput in gNB) as a per-UE measurement:

##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for option 3) and for each supported S-NSSAI. If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s] If ThpTimeDl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise ThpTimeDl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE.. The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer).. d) Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.UEThpDl, or optionally DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI. **** Skip non-relevant text **** **** Skip non-relevant text ****

Therefore, based on the above, it seems that the NOTE reflecting the RAN3 common understanding on how to derive UE-level UE performance metrics applies to Average Packet Delay and Average Packet Loss DL only.

Observation 6: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be stated that these measurements are already specified as per-UE. Therefore, the RAN3 common understanding on how to derive UE-level UE performance metrics (as captured in RAN3#124 meeting minutes) applies to Average Packet Delay and Average Packet Loss DL only.

Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB measurements is different from the RAN3 common understanding as captured in RAN3#124 meeting. The following numerical example should provide indications according to the above statement.

Let’s consider the case of two DRBs being configured for a certain UE:

- for DRB#1 the data volume is DV1=5Mbit and the time interval to deliver such data volume it ∆T1=500ms; this implies that the throughout for this DRB#1 is TputDRB#1=DV1∆T1=5MBit500ms=10Mbps;

- for DRB#2 the data volume is DV2=10Mbit and the time interval to deliver such data volume it ∆T2=200ms; this implies that the throughout for this DRB#2 is TputDRB#2=DV2∆T2=10MBit200ms=50Mbps.

By applying the formula as in clause 6.3.1.4.1 of TS 28.558 to calculate the Average DL UE Throughput in gNB the following results:

AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps

Note that the above calculation assumes that there is neither a time overlap between the transmissions over the two DRBs configured to the UE nor a time gap between such transmissions. In our understanding, this means that the transmission over the second DRB is assumed to start just after the transmission over the first one ends.

But if we compute the per-UE Average UE Throughput DL as per the RAN3 common understanding, i.e., by averaging per-DRB average throughput, the Average UE Throughput DL will be:

AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps

The above simple numerical example shows that even though we are referring to the same measurement, i.e., Average UE Throughput DL, the way such measurement is derived could lead to different values if there is no clear understanding of which approach needs to be followed.

Observation 7: Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB parameters (UE data volume and delivering time interval) differs from the RAN3 common understanding of averaging per-DRB UE throughout DL.

From the excerpt of TS 28.558 clause 6.3.1.4.1 reported above, it can also be noticed that field e) – “Measurement Type” as described in Annex A of TS 28.558 – indicates DRB.UEThpDl (and optionally DRB.UEThpDl.QoS as well as DRB.UEThpDl.SNSSAI) as “the measurement name used to identify the measurement type in the result files”. In our understanding such name indicates a per-DRB measurement of the UE throughput measured in the downlink direction and, according to the analysis made so far, this seems to us a misalignment between the name of the measurement (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is provided by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput in DL).

Observation 8: There seems to be a misalignment between the name of the measurement in TS 28.558 clause 6.3.1.4.1 (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is derived by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput).

Besides the name misalignment as per Observation 8, we think that it would be better to stick with the formulas specified by SA5 in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL, respectively, which provide a clear indication from the standard for deriving metrics to be used for, e.g., UE performance feedback. Moreover, from a pure technical perspective, it should be recalled that, in order to monitor the per-UE UE performance as a consequence of an AI/ML-driven handover, it could be needed to collect per-UE UE performance metrics before the handover (hence by the source node) which will then be compared to the same metrics but collected after the handover (hence by the target node): if per-UE metrics are collected as per the RAN3 common understanding, there could be some impairments in the measurement process due to the possible DRB re-mapping at the target node side.

Observation 9: Formulas specified by SA5 in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL provide a clear indication from the standard for deriving such metrics, and avoid potential impairments in the measurement process during per-DRB UE performance collection at both source and target sides.

Therefore, we propose the following:

Proposal 1: RAN3 to agree to refer to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for deriving per-UE Average UE Throughput DL and Average UE Throughput UL, respectively.

If Proposal 1 is agreed, it should be discussed how to avoid any misunderstanding in the derivation of the remaining per-UE UE performance metrics, i.e., Average Packet Delay and Average Packet Loss DL. Currently such measurements are defined in TS 28.558 on a per-DRB granularity, meaning that if corresponding per-UE measurements are needed the only way to derive them is by following the RAN3 common understanding as captured in RAN3#124 meeting.

Observation 10: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB granularity only. The corresponding per-UE measurements lack of a clear indication from the standard as for the Average UE Throughput DL/UL.

The risk that we see in adopting the RAN3 common understanding for deriving per-UE metrics from corresponding per-DRB measurements – which should be applicable only for Average Packet Delay and Average Packet Loss DL – is that we would still refer in general to per-UE UE performance which however are computed by following two distinct “averaging” approaches, and only one of these is standardized (in TS 28.558) and applicable only for the average UE throughput. In other words:

- for Average UE Throughput DL/UL: these metrics should be calculated as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, and we will label these metrics as per-UE “average” metrics;

- for Average Packet Delay and Average Packet Loss DL: these metrics can only be calculated by averaging the corresponding per-DRB measurements (i.e., as per the RAN3 common understanding), and we will also label these metrics as per-UE “average” metrics even though the averaging method is different from the standardized one suggested for the UE throughput.

The above situation should be avoided so that the risk of misinterpreting the per-UE UE performance metrics results is minimized. In our view this can only be achieved if a clear indication from the standard is made available also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics, and we think that this should be in the scope of SA5 rather than RAN3.

Observation 11: The risk of misinterpreting the per-UE UE performance metrics results should be minimized. This can only be achieved if a clear indication from the standard is made available also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput DL/UL.

Based on the above Observations we propose to liaise with SA5 to ask for feedback on whether a clear indication from the standard for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics could be provided by SA5 as already done for the per-UE Average UE Throughput. Clarifications on the contents of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 for deriving per-UE Average UE Throughput DL and Average UE Throughput UL could also be asked to SA5 as well. Hence, we propose:

Proposal 2: RAN3 to send a LS to SA5 to ask for feedback on whether a clear indication from the standard for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics could be provided by SA5 as already done for the per-UE Average UE Throughput.

A draft LSout to SA5 can be found in [12].

# 3	Conclusion

This document proposes the following observation and proposals:

On misalignment between TS 28.558 and TS 28.552

Observation 1: Following RAN3 indications, SA5 updated TS 28.558 to v18.1.0 to correct the misalignment between TS 28.552 and TS 28.558 v18.0.0 concerning some UE level measurements performed in NG-RAN, and introduced the missing M4 measurement as well.

On the usage of (5G-)S-TMSI as “Measured UE Identifier”

Observation 2: Following RAN3 observations on the (5G-)S-TMSI used as the “Measured UE Identifier” for UE level measurements in NG-RAN, SA5 clarified that such parameter in TS 28.558 is only applicable to 5GC UE level measurements collection. Hence SA5 updated TS 28.558 to v18.1.0 by setting “Measured UE Identifier” to “N/A” instead of “S-TMSI” for all UE level measurements performed in NG-RAN.

Observation 3: Even though SA5 updated TS 28.558 to v18.1.0 by setting “Measured UE Identifier” to “N/A” instead of “S-TMSI” for all UE level measurements performed in NG-RAN, SA5 sees some benefits in having a temporary (UE) identifier in the management-based MDT measurement report, being unique within the lifetime of the measurement session, so that it could be possible to indicate that the measurements reports are from the same or different UEs.

Observation 4: RAN3 will discuss whether and how a temporary (UE) identifier could be introduced in management-based MDT measurements reports as part of the Rel-19 normative phase for the Continuous MDT feature to address the issue of trace correlation in TCE.

On UE performance feedback in Rel-18 AI/ML for NG-RAN

Observation 5: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

Observation 6: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be stated that these measurements are already specified as per-UE. Therefore, the RAN3 common understanding on how to derive UE-level UE performance metrics (as captured in RAN3#124 meeting minutes) applies to Average Packet Delay and Average Packet Loss DL only.

Observation 7: Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB parameters (UE data volume and delivering time interval) differs from the RAN3 common understanding of averaging per-DRB UE throughout DL.

Observation 8: There seems to be a misalignment between the name of the measurement in TS 28.558 clause 6.3.1.4.1 (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is derived by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput).

Observation 9: Formulas specified by SA5 in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL provide a clear indication from the standard for deriving such metrics, and avoid potential impairments in the measurement process during per-DRB UE performance collection at both source and target sides.

Observation 10: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB granularity only. The corresponding per-UE measurements lack of a clear indication from the standard as for the Average UE Throughput DL/UL.

Observation 11: The risk of misinterpreting the per-UE UE performance metrics results should be minimized. This can only be achieved if a clear indication from the standard is made available also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput DL/UL.

Proposal 1: RAN3 to agree to refer to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for deriving per-UE Average UE Throughput DL and Average UE Throughput UL, respectively.

Proposal 2: RAN3 to send a LS to SA5 to ask for feedback on whether a clear indication from the standard for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics could be provided by SA5 as already done for the per-UE Average UE Throughput.

A draft LSout to SA5 can be found in [12].

# 4	References

- R3-245021 (S5-244658), Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, Ericsson

- R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, Nokia

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- R3-240058 (S5-241086), Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, Intel, Verizon, CMCC

- TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18, v18.0.0

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- R3-245422, On Continuous MDT for AI/ML, Huawei

- TS 38.423, NG-RAN; Xn Application Protocol (XnAP), Rel-18

- R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-244503, Clarification of UE performance, CR to TS 38.423, CATT, Huawei, Nokia, Qualcomm, Deutsche Telekom, NTT Docomo, Lenovo, NEC, China Telecom, China Unicom, Ericsson, ZTE, Samsung, InterDigital

- R3-245415, [Draft] LS on per-UE UE performance metrics, Huawei