3GPP TSG-RAN WG3 Meeting #125bis	R3-245421

Hefei, China, 14th – 18th October, 2024

Title: 	(TPs for AI/ML BLCR to TS 38.473 and AI/ML BLCR to TS 37.483) Split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In RAN#105 the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (latest SID in [2]) whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the split architecture support for Rel-18 use cases, as excerpted from [1] and highlighted in yellow below:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

This contribution analyzes the technical enablers for the split architecture support for Rel-18 use cases and provides proposals for discussion.

# 2. Discussion

## 2.1 Background

In the context of the Rel-18 WI “AI/ML for NG-RAN” [4] RAN3 specified data collection enhancements and signalling support within existing NG-RAN interfaces and architecture for three selected use cases, namely, (network) Energy Saving (ES), Mobility Enhancements (ME) and Load Balancing (LB). To this end, RAN3 introduces two new procedures, i.e., Data Collection Reporting Initiation (class 1 procedure) and Data Collection Reporting (class 2), which allow to signal over Xn both predictions and measurements to properly enable the considered use cases. However, due to time constraints, RAN3 agreed in RAN3#121bis that enhancements to support AI/ML for split RAN architecture are not pursued in Rel-18 (refer to the highlighted part in green in the agreement below), meaning that support for AI/ML in NG-RAN is only specified for the non-split architecture in Rel-18 [5]:

WA: Defining new procedures to transfer the measured EC over F1 follows the design over Xn.

The following overwrites the previous WA: enhancements to support AI/ML for split RAN architecture are not pursued in R18.

As a consequence, the support of the split architecture for the Rel-18 use cases was eventually included in the list of leftovers for further discussion in Rel-19.

As clearly stated in the WID [1], the specification work in Rel-19 for AI/ML for NG-RAN – including split architecture support – is based on the conclusions captured in TR 38.743, which are excerpted from [3] and reported below:

# 6 Conclusion The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework. The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase: - AI/ML-based Network Slicing - AI/ML-based Coverage and Capacity Optimization Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2. For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work. The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

During the Rel-19 SI, for split architecture support, the use case description and potential standard impacts have been investigated and reported in TR 38.743 clause 5.2 as follows:

## 5.2 Split architecture support for Rel-18 use cases ### 5.2.1 Use case description The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization. In case of CU-DU architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP); - AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP). ### 5.2.2 Potential Standard impacts The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18: - The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase. - Measured Energy Cost from gNB-DU to gNB-CU.

In the next sections we provide our view on how the agreed AI/ML-related information listed in TR 38.743 clause 5.2 could be transferred over E1 and F1 interfaces, as well as an analysis on the specification impact required to enable such transfer.

## 2.2 On gNB-DU Energy Cost reporting over F1

In TR 38.743 it is captured that the measured Energy Cost (EC) can be transmitted over F1 from DU to CU(-CP):

## 5.2 Split architecture support for Rel-18 use cases ### ****Skip non-relevant text**** ### 5.2.2 Potential Standard impacts The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18: - The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase. - Measured Energy Cost from gNB-DU to gNB-CU.

Hence, we think RAN3 need to only decide how to enable such reporting, i.e., discuss whether to re-use existing F1AP procedures or introduce brand new procedures similar to the XnAP’s Data Collection Reporting (Initiation) procedures.

Observation 1: RAN3 is requested to only discuss and decide on how to enable the reporting over F1 of the DU’s measured EC to the CU(-CP).

Considering that it is expected to exchange multiple information over F1, i.e., not only gNB-DU’s measured EC, we think it would be better to introduce new independent procedures mirroring the XnAP’s Data Collection Reporting (Initiation) procedures as specified in Rel-18. Therefore, we propose the following:

Proposal 1: RAN3 to discuss and decide on how to enable the reporting of the measured DU’s EC to CU(-CP), e.g., to introduce new F1AP procedures similar to the XnAP’s Data Collection Reporting (Initiation).

## 2.3 On reporting measured UE performance

Again, in TR 38.743 it is captured that signalling details for transferring measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need to be discussed during the Rel-19 WI phase:

## 5.2 Split architecture support for Rel-18 use cases ### ****Skip non-relevant text**** ### 5.2.2 Potential Standard impacts The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18: - The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase. - Measured Energy Cost from gNB-DU to gNB-CU.

The above objective relates to UE performance metrics collection at CU-CP of a split target gNB from CU-UP and/or DU – such measurement collection should be triggered after an AI/ML-based mobility decision taken by the source gNB is successfully executed.

RAN3 introduced in Rel-18 the Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss as the per-UE metrics representing the UE performance to be reported from target gNB to source gNB over Xn. In case the target gNB is a split node, we think it is important that the target CU/CU-CP is able to collect UE performance information, so that the target CU/CU-CP can report the aggregated UE performance to the source gNB for, e.g., AI/ML model monitoring in the source node.

Observation 2: After an AI/ML-based mobility action, and if the target node is a split gNB, the target CU/CU-CP needs to collect UE performance information from CU-UP and/or DU so that the target CU/CU-CP can report the aggregated UE performance to the source gNB.

As per TS 28.558, the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss are measured in the gNB-DU.

While, for the Average Packet Delay, TS 28.558 specified the delay components for both DL and UL – refer to Figure 1 and Figure 2, respectively – some of which are measured by the gNB-DU and others by the gNB-CU-UP.

Figure 1 - RAN part of DL packet delay measurement.

Figure 2 – RAN part (including UE) of UL packet delay measurement.

Table 1 – UE performance metrics, where they are measured.

|  | gNB-DU | gNB-CU-UP |
| --- | --- | --- |
| Average UE Throughput DL | X |  |
| Average UE Throughput UL | X |  |
| Average Packet Loss | X |  |
| DL packet delay component | D1, D2 | D3, D4 |
| UL packet delay component | D2.1, D2.2 | D2.3, D2.4 |

The above Table 1 shows where each of the UE performance metrics is measured, and it is clear that Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss should be reported to the gNB-CU(-CP) via F1 signalling. Therefore, we propose:

- Proposal 2: RAN3 to agree on exchanging over F1 (DU to CU/CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

The exchange of Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss can be performed via the new F1AP procedures mirroring XnAP’s Data Collection Reporting (Initiation) procedures as proposed in Proposal 1.

For the Average Packet Delay, instead, since there are multiple components, some of them measured at DU level and others at CU-UP level, we think that the CU(-CP) could be made aware of the overall Average Packet Delay via one of the following options:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively; it’s then up to CU-CP to aggregate the received measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1; with respect to Option 1, in this way it could still be possible for the CU-CP to have visibility of the different delay components in DU and CU-UP (same as in Option 2).

Therefore, we propose:

Proposal 3: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively, and CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1.

To support the finer-grained measurement transfer, Option 2 and 3 can be adopted to inform the CU-CP about the specific component of the Average Packet Delay. Between them, Option 2 is more reasonable and efficient since the components are directly sent to CU-CP from the network entity where such components are measured.

If Option 2 is agreed, the DU-measured Average Packet Delay components can be sent directly from DU to CU-CP by means of the new procedures mirroring the XnAP’s Data Collection Reporting (Initiation) procedures as per Proposal 1 above. While for the CU-UP-measured Average Packet Delay components a new procedure also for E1 – also mirroring the XnAP’s Data Collection Reporting (Initiation) procedures – can be introduced.

TPs for the AI/ML BLCR to TS 38.473 and AI/ML BLCR to TS 37.483 reflecting the above proposals are provided in the Annexes of this paper. For the BLCR to TS 37.483 Option 2 above for the transfer of the Average Packet Delay components is considered.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals:

Observation 1: RAN3 is requested to only discuss and decide on how to enable the reporting over F1 of the DU’s measured EC to the CU(-CP).

Proposal 1: RAN3 to discuss and decide on how to enable the reporting of the measured DU’s EC to CU(-CP), e.g., to introduce new F1AP procedures similar to the XnAP’s Data Collection Reporting (Initiation).

Observation 2: After an AI/ML-based mobility action, and if the target node is a split gNB, the target CU/CU-CP needs to collect UE performance information from CU-UP and/or DU so that the target CU/CU-CP can report the aggregated UE performance to the source gNB.

- Proposal 2: RAN3 to agree on exchanging over F1 (DU to CU/CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

Proposal 3: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively, and CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1.

TPs for the AI/ML BLCR to TS 38.473 and AI/ML BLCR to TS 37.483 reflecting the above proposals are provided in the Annexes of this paper. For the BLCR to TS 37.483 Option 2 above for the transfer of the Average Packet Delay components is considered.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#121b Chairman’s notes (RAN3_121bis_agenda_20231013_EOM1.doc)

# 5. Annex 1 - TP for AI/ML BLCR to TS 38.473

-------------------------------------------------------------Start of Change-----------------------------------------------------------------

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Reporting procedures

### 8.X.1	Data Collection Reporting Initiation

#### 8.X.1.1	General

This procedure is used by the gNB-CU/CU-CP to request from the gNB-DU node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU(-CP) initiates the procedure by sending the DATA COLLECTION REQUEST message to the gNB-DU to start information reporting or to stop information reporting. Upon receipt, the gNB-DU:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the gNB-DU is capable of providing all of the requested information, it shall initiate the information reporting as requested by the gNB-CU(-CP) and respond with the DATA COLLECTION RESPONSE message.

If the gNB-DU is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-DU shall take it into account for the configuration of UE performance collection and reporting. The gNB-DU shall terminate the collection when at least one of the following conditions is fulfilled:

-	the collecting time is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type and/or the component of objects the gNB-DU performs measurements on. The gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Energy Cost IE, if the first bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average UE Throughput DL IE, if the second bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average UE Throughput UL IE, if the third bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Packet Loss DL IE, if the forth bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay DL air-interface IE, if the fifth bit, "Average Delay DL air-interface" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay DL in gNB-DU IE, if the sixth bit, "Average Delay DL in gNB-DU" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay UL on over-the-air interface IE, if the seventh bit, "Average Delay UL on over-the-air interface" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average RLC Packet Delay UL IE, if the eighth bit, "Average RLC Packet Delay UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, the gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU(-CP) does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU(-CP) may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-DU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU(-CP) Measurement ID IE corresponding to an existing on-going Data Collection reporting, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.X.2	Data Collection Reporting

#### 8.X.2.1	General

This procedure is for the gNB-DU to report measured information accepted to the gNB-CU(-CP) following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Data Collection Reporting, successful operation

The gNB-DU shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.X	Data Collection Reporting Messages

#### 9.2.X.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU(-CP) to initiate data collection measurements.

Direction: gNB-CU(-CP)  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU(-CP) UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN performance Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Forth Bit = Average Packet Loss DL, Fifth Bit = Average Delay DL air-interface, Sixth Bit = Average Delay DL in gNB-DU, Seventh Bit = Average Delay UL on Over-the-air interface, Eighth Bit = Average RLC Packet Delay UL. Other bits are ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.X.2	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to indicate that the requested data collection is successfully initiated.

Direction: gNB-DU  gNB-CU(-CP).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN performance Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-DU. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Loss DL, Fifth Bit = Average Delay DL air-interface, Sixth Bit = Average Delay DL in gNB-DU, Seventh Bit = Average Delay UL on Over-the-air interface, Eighth Bit = Average RLC Packet Delay UL. Other bits are ignored by the gNB-CU(-CP). | – |  |
| >>Cause | M |  |  | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.3	DATA COLLECTION FAILURE

This message is sent by gNB-DU to indicate that the requested data collection cannot be initiated.

Direction: gNB-DU  gNB-CU(-CP).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN performance Measurement ID | M |  | INTEGER (1..16, …) |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.4	DATA COLLECTION UPDATE

This message is sent by gNB-DU to report the results of the requested data collection measurement.

Direction: gNB-DU  gNB-CU(-CP).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN performance Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Performance | O |  | 9.3.1.X |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a gNB-DU node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9. 3.1.X	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate | Defined in TS 38.423 9.2.3.4 |
| Average UE Throughput UL | O |  | Bit Rate | Defined in TS 38.423 9.2.3.4 |
| Average Packet Loss DL | O |  | Packet Loss Rate | Defined in TS 38.423 9.2.3.11 |
| Average Delay DL air-interface | O |  |  | Defined in TS 28.558 / 6.3.1.1.1 |
| Average Delay DL in gNB-DU | O |  |  | Defined in TS 28.558 / 6.3.1.1.2 |
| Average Delay UL on over-the-air interface | O |  |  | Defined in TS 28.558 / 6.3.1.1.6 |
| Average RLC Packet Delay UL | O |  |  | Defined in TS 28.558 / 6.3.1.1.7 |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 6. Annex 2 - TP for AI/ML BLCR to TS 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Reporting procedures

### 8.X.1	Data Collection Reporting Initiation

#### 8.X.1.1	General

This procedure is used by the gNB-CU-CP to request from the gNB-CU-UP node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

The procedure is initiated with a DATA COLLECTION REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurement.

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to the gNB-CU-UP to start information reporting or to stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the gNB-CU-UP is capable of providing all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the gNB-CU-UP is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-CU-UP shall take it into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the collecting time is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type and/or the component of objects the gNB-CU-UP performs measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay DL on F1-U IE, if the first bit, "Average Packet Delay DL on F1-U" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average Packet Delay DL in CU-UP IE, if the second bit, "Average Packet Delay DL in CU-UP" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average Packet Delay UL on F1-U IE, if the third bit, "Average Packet Delay UL on F1-U" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average PDCP re-ordering delay in the UL IE, if the forth bit, "Average PDCP re-ordering delay in the UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, the gNB-CU-UP shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU-CP does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-CU-UP Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-CU-UP shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then the gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going Data Collection reporting, then the gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.X.2	Data Collection Reporting

#### 8.X.2.1	General

This procedure is for the gNB-CU-UP to report measured information accepted to the gNB-CU-CP following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Data Collection Reporting, successful operation

The gNB-CU-UP shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.X	Data Collection Reporting Messages

#### 9.2.X.1	DATA COLLECTION REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay DL on F1-U, Second Bit = Average Packet Delay DL in CU-UP, Third Bit = Average Packet Delay UL on F1-U, Forth Bit = Average PDCP re-ordering delay in the UL. Other bits are ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.X.2	DATA COLLECTION RESPONSE

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay DL on F1-U, Second Bit = Average Packet Delay DL in CU-UP, Third Bit = Average Packet Delay UL on F1-U, Forth Bit = Average PDCP re-ordering delay in the UL. Other bits are ignored by the gNB-CU-UP. | – |  |
| >>Cause | M |  |  | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.3	DATA COLLECTION FAILURE

This message is sent by the gNB-CU-UP to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | O |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.4	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Performance | O |  | 9.3.1.X |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a gNB-DU node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9. 3.1.X	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay DL on F1-U | O |  |  | Defined in TS 28.558 / 6.3.1.1.3 |
| Average Packet Delay DL in CU-UP | O |  |  | Defined in TS 28.558 / 6.3.1.1.4 |
| Average Packet Delay UL on F1-U | O |  |  | Defined in TS 28.558 / 6.3.1.1.3 |
| Average PDCP re-ordering delay in the UL | O |  |  | Defined in TS 28.558 / 6.3.1.1.8 |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>