3GPP TSG-RAN WG3 Meeting #125bis						R3-245435

Hefei, China, 14th - 18th October 2024

Agenda item:	 8.1

Source:	 Lenovo

Title:	 Discussion on incoming LSs on XR

Document for:	Discussion and Decision

- Introduction

This paper discusses RAN3 replies to incoming LSs on [1] and [2].

- Discussion

2.1 UL PSI based SDU discard

In RAN2#124, it was agreed that:

Agreements: 1 In the MAC CE for the activation/deactivation of the PSI-based discard, introduce a bitmap for DRB to efficiently control multiple DRBs separately and simultaneously.

For UL PDCP discard, as per RAN2’s agreement, it means that gNB can configure two PDCP discard timers to UE: Normal PDCP discard timer and PSI based discard timer to the UE.  And gNB can signals an activation/deactivation of the PSI based discard timer to the UE by MAC CE.

For NR-DC split bearer, the DRB is served by both MCG and SCG, and each has its own MAC entity. It is possible both MN and SN can send the PSI based Discard Activation/Deactivation MAC CE of the same DRB to the UE. In some cases, the UE may receive contradicting instruction of Activation/Deactivation of the PSI based discard timer from MN and SN if there is no coordination between MN and SN.

Observation 1: For a split bearer, UE may receive contradicting instruction of Activation/Deactivation of the PSI based discard timer from MN and SN if there is no coordination between MN and SN.

The reply LS from RAN2 [1] also states that:

The following is the current UE behavior: - UE activates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE activating the discard, regardless of whether the MAC CE is received from MN or SN; - UE deactivates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE deactivating the discard, regardless of whether the MAC CE is received from MN or SN. It is up to RAN3 to discuss whether any network side solution to improve PSI-based SDU discard is needed.

In the LS, RAN2 clarifies UE behaviour and expect network based solution to solve the issue.

Observation 2: NW based solution is needed for PSI-based SDU discard coordination for split bearer.

To solve the issue for split bearer, there would be several solutions:

-	Solution 1: Only PDCP terminated node can send the MAC CE. This solution has strong restriction on NW implementation.

-	Solution 2: Once one node sends the MAC CE to UE, the node informs the other node about the information.

-	Solution 3. NW determines one node which can send the MAC CE to UE. When the peer node wants to deactivate/activate the PSI based discard, the peer node sends the request to the node.

In RAN3#123bis meeting, solution 3 is excluded. Since solution 1 has strong restriction on NW implementation, solution 2 is preferred.

For split bearers, once one node sends the PSI based Discard Activation/Deactivation MAC CE to the UE, the node informs the other node about PSI based Discard Activation/Deactivation status of DRBs.

To be more specific, a new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in Xn-AP:

-	the PDU Session Resource Modification Info – MN terminated IE contained in the S-NODE MODIFICATION REQUEST message;

-	the PDU Session Resource Modification Response Info – SN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message;

-	the PDU Session Resource Modification Required Info – SN terminated IE contained in the S-NODE MODIFICATION REQUIRED message; and

-	the PDU Session Resource Modification Required Info – MN terminated IE contained in the S-NODE MODIFICATION REQUIRED message.

A new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in Xn-AP:

- -	the PDU Session Resource Modification Info – MN terminated IE contained in the S-NODE MODIFICATION REQUEST message;

- -	the PDU Session Resource Modification Response Info – SN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message;

- -	the PDU Session Resource Modification Required Info – SN terminated IE contained in the S-NODE MODIFICATION REQUIRED message; and

- -	the PDU Session Resource Modification Required Info – MN terminated IE contained in the S-NODE MODIFICATION REQUIRED message.

In case of CU-DU split, it is up to DU makes the decision to activate or deactivate the UL PSI based SDU discard. When DU of one node decides to activate or deactivate the UL PSI based SDU discard, the DU of the node needs to inform it to the CU of the node. The CU of the node further informs it to the CU of peer node. and then the CU of peer node further forwards it to the DU of peer of node. The expected impact on F1-AP is a new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in F1-AP:

-	the DRB Required to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUIRED message.

-	the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message.

A new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in F1-AP:

- -	the DRB Required to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUIRED message.

- -	the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message.

2.2 Multi-Modality

For Multi-Modal XR applications transmitted via a mobile communication system like NR the interactions between different input signals can be translated to some inter-dependencies between transmissions of different bearers/LCHs. Due to the separate handling of the multiple media components, synchronization between different media components is critical in order to avoid having a negative impact on the user experience. As the asynchrony between different modalities increases, users’ sense of presence and realism will decrease.

The table below from TS22.847 depicts typical synchronization thresholds between two or more modalities.

Table 5.1.6-2: Typical synchronization thresholds for immersive multi-modal VR applications

|  | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

For instance, for VR applications that require visual-tactile synchronization, the visual media component should not be delayed for more than 15ms when comparing to the tactile media component.

SA2 also discussed the support of multi-modal services. They introduced the concept of a Multi-modal Service ID which is an explicit indication that data flows being associated with such Multi-modal service ID are related to a multi-modal service. PCF may use this information to derive the corresponding PCC rules and apply appropriate QoS policies for the inter-related data flows being part of a multi-modal application. In the following the relevant text from TS23.501 is copied:

### 5.37.2 Policy control enhancements to support multi-modal services A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources (e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session. The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements: - The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application. - The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow. NOTE: In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from AF within a single request or, in case of multiple requests, within a short period of time.

This new synchronization requirements between the modalities and the related inter-dependencies among data of different modalities pose new challenges for an efficient support in RAN as discussed in the following.

We think that in order allow an efficient support of multi-modal applications at the RAN level, RAN needs to be aware of the relations among different QoS flows belonging to one multi-modal application. Essentially similar to the multi-modal service ID, also RAN needs to be made aware of a group of inter-related QoS flows belonging to a multi-modal application. We think that the multi-modal awareness/information should include information of the relations among different QoS flow belonging to a multi-modal application (similar to MMSID information) as well as the corresponding synchronization requirements, e.g. relative delay requirements, between the different QoS flows belonging to one multi-modal application.

In our view, the awareness of synchronization requirements among multiple modalities is not only required to ensure the synchronized delivery of packets of multiple inter-correlated flows and hence to allow for an optimized user experience but may represent an additional opportunity for RAN to save radio resource. Specifically, some enhanced discarding operation considering the synchronization requirements may contribute to a capacity improvement, e.g. when a packet of one of the flows is discarded, or when a packet cannot be transmitted before the synchronization deadline, RAN/UE may discard some of the pending packets of the group of inter-related flows as they may no longer be useful from Application perspective.

Not only MMSID but also synchronization thresholds for multi-modal QoS flows should be aware by RAN node.

we also assume that similar to the PSDB, PSIHI information also some multi-modal information could be provided as PDU Set QoS Parameters by the SMF to the gNB as part of the QoS profile of a QoS flow. Depending on level of granularity required for the inter-dependency information PDU set information signalled within the GTP-U header may be also a candidate for conveying multi-modal information, e.g. PDU set A of QoS flow X is related to PDU set B of QoS flow Y. A motivation to provide the inter-dependency information on a PDU set level is, that resource allocation/scheduling is usually done on a PDU/PDU set level. For an efficient resource allocation, e.g. RAN scheduling DL transmissions, it would be best to provide the dependency information on a PDU set level. This would allow for a very accurate resource allocation/scheduling. This would however come at the expense of an increased signalling overhead, e.g. PDU set information in GTP-U header is increased.

The level of granularity of the inter-dependency information, e.g. PDU set level or QoS flow level, needs to be further discussed.

In general, such multi-modal information could be also provided by the UE as well. By extending the UAI of uplink traffic information defined in Rel-18 XR, the UE could additionally indicate whether a QoS flow has a synchronization requirement with at least one other QoS flow(s). However, we are not sure, whether it is necessary to actually extend the UE assistance information reporting. Since it will anyways depend on the reply from SA2, we think that this should be discussed later.

Whether an extension of the UAI, to provide the assistance information relating to multi-modal synchronization among multiple uplink QoS flows to RAN, is needed/beneficial should be discussed after having the received further inputs from SA2.

3	Conclusion

This paper discusses the incoming LSs on XR. And we propose:

- For split bearers, once one node sends the PSI based Discard Activation/Deactivation MAC CE to the UE, the node informs the other node about PSI based Discard Activation/Deactivation status of DRBs.

- A new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in Xn-AP:

- -	the PDU Session Resource Modification Info – MN terminated IE contained in the S-NODE MODIFICATION REQUEST message;

- -	the PDU Session Resource Modification Response Info – SN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message;

- -	the PDU Session Resource Modification Required Info – SN terminated IE contained in the S-NODE MODIFICATION REQUIRED message; and

- -	the PDU Session Resource Modification Required Info – MN terminated IE contained in the S-NODE MODIFICATION REQUIRED message.

A new UL PSI based Discard Status IE with two codepoints (Activation, Deactivation) per DRB should be added in the following IEs in F1-AP:

- -	the DRB Required to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUIRED message.

- -	the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message.

- Not only MMSID but also synchronization thresholds for multi-modal QoS flows should be aware by RAN node.

- The level of granularity of the inter-dependency information, e.g. PDU set level or QoS flow level, needs to be further discussed.

- Whether an extension of the UAI, to provide the assistance information relating to multi-modal synchronization among multiple uplink QoS flows to RAN, is needed/beneficial should be discussed after having the received further inputs from SA2.

Reference

[1] R3-245011	Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC	RAN2(Qualcomm)

[2] R3-245017	LS reply on multi-modality awareness at RAN	SA2(Huawei)