3GPP TSG-RAN WG3 Meeting #125bis	R3-245437

Hefei, China, 10 - 14 October 2024

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 37.340) Discussion on MRO for subsequent CPAC and CHO with candidate SCG(s)

Document for:		Discussion and Decision

1	Introduction

In RAN3#123bis, we have achieved following agreements:

Work on the scenarios of failure in S-CPAC. The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

MRO for CHO with candidate SCG failure and near failure cases.

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

In RAN3#124, we further agreed that:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

In RAN3#125, we further agreed that:

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution.

Editor’s note: FFS whether the differentiation between initial and following CPC is needed.

To be continued...

In this paper, we would further discuss the details of MRO for subsequent CPAC and CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for subsequent CPAC

RAN2#126 meeting agreed

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

Based on agreements achieved in RAN2 and RAN3 till now, we further discuss MRO for failure and near failure cases in a subsequent CPAC procedure.

    - 2.1.1 MRO for connection failure in a subsequent CPAC procedure

In R18 MRO for CPAC, it aims to address failure types such as Too Late CPC Execution, Too Early CPC/CPA Execution, and CPC/CPA Execution to wrong PSCell, failure type definition and detection mechanism are specified in TS37.340 [1]  as following.

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA procedure:

-	Too Late CPC Execution: if CPC is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

In RAN3#124, we agreed that CPAC failure scenarios and detection mechanism captured in stage2 used as baseline. Compared with a legacy CPAC procedure wherein only one CPAC execution is performed, a subsequent CPAC procedure may be composed of initial CPAC execution and at least one following subsequent CPC execution, to explicitly show the differences, some updates of failure type definition and detection mechanism for CPAC are needed to adapt to subsequent CPAC, examples are given in the Annex part.

Proposal 1: Reuse existing failure type definition and detection mechanism for CPAC with updates to adapt to Too Late subsequent CPAC Execution, Too Early subsequent CPAC Execution, and subsequent CPAC Execution to wrong PSCell.

For MN initiated SCPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at SCPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated SCPAC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation. Source SN may have released UE context when receiving the SCG Failure Information Report message, to enable source SN identify UE context, subsequent CPAC configuration (e.g. candidate PSCell list for SCPAC and SCPAC execution condition(s)) may be included in the SCG Failure Information Report message.

Proposal 2: Include subsequent CPAC configuration (e.g. candidate PSCell list for SCPAC and SCPAC execution condition(s)) in the SCG Failure Information Report message.

    - 2.1.2 MRO for near failure in a subsequent CPAC procedure

In R18, SPR is introduced to detect sub-optimal successful PSCell addition or PSCell change or CPAC, RAN2 specified that the SPR is not sent immediately after a successful PSCell addition or PSCell change or CPAC, and only the latest successful PSCell addition or PSCell change or CPAC is stored or reported by the UE.

A near-failure successful CPAC execution may occur during initial CPAC execution phase or at least one following subsequent CPC execution phase in a subsequent CPAC procedure. Some companies concerned that the near failure report may have heavy impacts on specifications during offline discussion. Since RAN2#126 meeting agreed to look into near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed, it is up to RAN2 to discuss whether SPR can be reused e.g. as legacy only the latest successful CPA or CPC execution is stored or reported by the UE, or whether SPR should be enhanced to include multiple CPAC executions, or whether to introduce a new successful report for subsequent CPAC procedure. We can wait for RAN2’ progress.

  - 2.2 MRO for CHO with candidate SCG(s)

    - 2.2.1 MRO for connection failure case

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

It was agreed in last RAN3#125 meeting that MRO for case 4/5/6 is in the scope and it was FFS whether there is any specification impact for case 4/5/6. In case of 4/5/6, configuration of CHO only is configured to the UE together with configuration of CHO with candidate SCG(s) configuration, the UE performs CHO only execution due to CHO with candidate SCG(s) can’t be triggered (e.g. CHO execution condition of CHO with candidate SCG(s) is fulfilled but CPAC execution condition of CHO with candidate SCG(s) is not fulfilled, CPAC execution condition of CHO with candidate SCG(s) is fulfilled but CHO execution condition of CHO with candidate SCG(s) is not fulfilled, neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled). To improve mobility performance, it is beneficial for network to know the reason about why CHO with candidate SCG(s) together with CHO only is configured but CHO only is performed instead of CHO with candidate SCG(s), e.g. only CHO execution condition or only CPAC execution condition of CHO with candidate SCG(s) is fulfilled, or neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled. Last RAN3#125 meeting agreed the RLF report includes the type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. This agreement applies to the case 4/5, for case 6, SCG failure information message needs to include the type of the first fulfilled execution condition of CHO with candidate SCG(s) if any e.g. CPAC or CHO.

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Last RAN3#125 meeting also agreed that MRO for case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed. For case 3a, 3b, 4, 5, 7a, 7b, 8a, 8b and 9, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions before the failure is encountered in the RLF report. For case 2a, 2b, 7a, 7b, 8a, 8b and 9, it is needed to include identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the RLF report. For case 6, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message.

Proposal 4: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in RLF report or SCG failure information message.

For case 7 and case 8, connection failure happens after CHO with candidate SCG(s) is triggered. To analyse whether the failure is caused due to inappropriate CHO execution condition and/or CPAC execution condition, it is beneficial to store and report time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition in RLF report or SCG failure information message.

Proposal 5: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in RLF report or SCG failure information message.

For the left case 1c, 2c, 3c, 7c and 8c, since both MCG failure and SCG failure happen, we should prioritize the single failure cases, if time allows, the concurrent failure cases can be further discussed later.

Proposal 6: Deprioritize Case 1c, 2c, 3c, 7c and 8c.

    - 2.2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen. For sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, legacy SHR trigger condition(s) can be reused, and SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, legacy SPR trigger condition(s) can be reused, and SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 8: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

In case the SPR is retrieved in a node different from source MN that sent the SPR configuration to the UE or target MN that to which the UE successfully access, since SPR trigger condition(s) may be generated by target MN or target SN, for SPR forwarding, there are two potential options:

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

In Option 1, to enable the receiving node to forward the SPR to source MN, the UE should store and report cell ID of source PCell in the SPR. Furthermore, the UE should store and report cell ID of target PCell in the SPR for forwarding the SPR from source MN to target MN. Compared with Option 1, in Option 2, the UE should only store and report cell ID of target PCell in the SPR, which has fewer spec impacts.

Proposal 9: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

3	Conclusion

In this paper, the details of MRO for subsequent CPAC and CHO with candidate SCG(s) are discussed. We have the following proposals:

MRO for subsequent CPAC

Proposal 1: Reuse existing failure type definition and detection mechanism for CPAC with updates to adapt to Too Late subsequent CPAC Execution, Too Early subsequent CPAC Execution, and subsequent CPAC Execution to wrong PSCell.

Proposal 2: Include subsequent CPAC configuration (e.g. candidate PSCell list for SCPAC and SCPAC execution condition(s)) in the SCG Failure Information Report message.

MRO for CHO with candidate SCG(s)

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Proposal 4: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in RLF report or SCG failure information message.

Proposal 5: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in RLF report or SCG failure information message.

Proposal 6: Deprioritize Case 1c, 2c, 3c, 7c and 8c.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 8: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

Proposal 9: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

Reference

[1] TS37.340

Annex for TS37.340

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration or S-CPAC configuration, while a SCG failure occurs before CPC execution condition or initial/following subsequent CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC or initial CPC execution/following subsequent CPC execution in S-CPAC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA or initial CPA execution in S-CPAC, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA or by the node initiating the S-CPAC, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA or at the node initiating the S-CPAC.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

In case of S-CPAC, the Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell in the definition above means Too Late initial/following subsequent CPC Execution, Too Early initial/following subsequent CPC/CPA Execution, and initial/following subsequent CPC/CPA Execution to wrong PSCell.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC or S-CPAC in case of conditional mobility, if CPC or subsequent CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA procedure or the S-CPAC procedure:

-	Too Late CPC Execution: if CPC or S-CPAC is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.