3GPP TSG RAN WG3 #125	R3-245455

Hefei, China, 14th – 18th Oct 2024

Agenda item:	17.2

Source: 	Lenovo

Title: 	Discussion on On-demand SSB for SCell

Document for:	Discussion and Decision

Introduction

In RAN#102, a new work item “Enhancements of network energy savings for NR” was agreed [1]. The one objective of the work item is the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we analyse RAN3 impact and high-level solution based on NW-triggered on-demand SSB operation.

Discussion

2.1 RAN1 progress

In RAN1#116, RAN1 has discussed 3 triggering methods of on-demand SSB:

- Method 1: UE uplink wake-up-signal using an existing signal/channel

- Method 2: Cell on/off indication via backhaul

- Method 3: SCell activation/deactivation signaling

The 2nd and 3rd methods can be categorized into NW-triggered on-demand SSB operation. Many companies have not been convinced yet whether to support UE-triggered on-demand SSB operation. In high level, NW-triggered on-demand SSB operation is adopted, how to design NW triggering message and when to transmit message is FFS. And it was agreed:

- RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. - For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission - Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for

In RAN1#116bis, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

- identified scenarios and cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1, - Scenario #2A and Case #2 - Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - when and how to transmit that on-demand SSB burst(s) after trigger on-demand SSB SCell operation by NW. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

In RAN1#117, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

- For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

In RAN1#118，RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Observation 1: NW-triggered on-demand SSB operation has been agreed by RAN1 for network energy saving:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A, wherein MAC CE can indicate one of multiple candidate values configured by RRC.

Observation 2: Two Scenarios of NW triggering message for on-demand SSB have been identified by RAN1:

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

  - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command.

Observation 3: Two cases of SSB transmission on a cell supporting on-demand SSB Scell include: 1) no always-on SSB transmission, 2) SSB transmission with long periodicity.

2.2 on-demand SSB operation

If NG-RAN node determines to trigger on-demand SSB transmission, NG-RAN transmits NW trigged message to UE. In case of CU-DU split case, RAN3 discusses which node decides to trigger on-demand SSB. If CU determines to trigger on-demand SSB by MAC CE message, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command. If CU determines to trigger on-demand SSB by RRC message, CU transmits a command and RRC message to DU, and DU transmits SSB based on the command and RRC message to UE.

Proposal 1: If CU determines to trigger on-demand SSB by MAC CE message, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

Proposal 2: If CU determines to trigger on-demand SSB by RRC message, CU transmits a command and a RRC message to trigger on-demand SSB to DU, and DU transmits SSB based on the command and transmits the RRC message to UE.

NG-RAN node may determine to trigger on-demand SSB transmission based on request/indication by the neighbor NG-RAN node. For example, if NG-RAN node enters energy saving mode by switch off SSB transmission or switch to long SSB transmission period for a NES cell and it transmits an indication to indicate that the concerned cell is in this energy saving mode to the neighbour NG-RAN node. The neighbour NG-RAN requests to trigger On-demand SSB transmission of the cell if it needs to be used for layer 3 measurement.

A straightforward for the high-level solution from RAN3 point of view is introduce an on-demand SSB trigger IE, which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

Proposal 3: RAN3 to introduce an on-demand SSB trigger IE which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

Conclusion

In summary, we have following observations and proposals for on-demand SSB for SCell in carrier aggregation:

Observation 1: NW-triggered on-demand SSB operation has been agreed by RAN1 for network energy saving:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A, wherein MAC CE can indicate one of multiple candidate values configured by RRC.

Observation 2: Two Scenarios of NW triggering message for on-demand SSB have been identified by RAN1:

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

  - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command.

Observation 3: Two cases of SSB transmission on a cell supporting on-demand SSB Scell include: 1) no always-on SSB transmission, 2) SSB transmission with long periodicity.

Proposal 1: If CU determines to trigger on-demand SSB by MAC CE message, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

Proposal 2: If CU determines to trigger on-demand SSB by RRC message, CU transmits a command and a RRC message to trigger on-demand SSB to DU, and DU transmits SSB based on the command and transmits the RRC message to UE.

Proposal 3: RAN3 to introduce an on-demand SSB trigger IE which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-244330, Discussion on On-demand SSB for SCell, Lenovo