3GPP TSG RAN WG3 #125b	R3-245456

Hefei, China, 14th – 18th Oct 2024

Agenda item:	17.3

Source: 	Lenovo

Title: 	Discussion on On-demand SIB1 for Idle/Inactive UE

Document for:	Discussion and Decision

Introduction

In RAN#105, a revised work item “Enhancements of network energy savings for NR” was approved [1]. The one objective of the work item is the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN3 has discuss the on demand SIB1 for Case 2 and reached the agreement：

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

In this contribution, we further analyse potential RAN3 impact based on RAN1 and RAN2 progress.

Discussion

2.1 RAN1 and RAN2 progress

In RAN1#116, RAN1 has on-demand SIB1 operation, and it was agreed:

- For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - For study of UL WUS design, consider at least PRACH as a starting point - FFS: Whether there is dedicated PRACH resource for SIB1 request - For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS. - For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

In RAN1#116bis discussion on the on demand SIB1 with the following agreements:

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

In RAN1#117 discussion on the on demand SIB1 with the following agreements:

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact. From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Based on RAN1 agreement, we have following observations.

Observation 1: R19 focus on case 2 on on-demand SIB1 for idle/inactive mode UE:

- Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

In RAN2#125bis initial discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements on on-demand SIB1:

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

In RAN2#126 discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES The same Cell A may assist more than one NES Cells.Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In RAN2#127 discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on OD-SIB1

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Based on RAN2 agreement, we have following observations on case 2(Option 1+B+X in RAN1 agreement).

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

2.2 on-demand SIB1 procedure

Based on RAN3 agreement, DU of NES cells generates UL WUS config and then transmits it to CU. Before DU transmission UL WUS config to CU, one possibility is that CU determines which cells can use on-demand SIB1 and transmits the request to the DU for the WUS config of the NES cell e.g., via F1 SETUP REQUEST message. Then DU transmits UL WUS config to the CU. Another possibility is DU determines which cells use on-demand SIB1 and initiatively transmits the WUS config of the NES cell to CU. Since CU has overview of the load and coverage of cells, we slightly prefer the former.

Proposal 1: CU of NES cell determines which cells are allowed to use on-demand SIB1 and transmits request to DU for the WUS config for an NES cell.

Before NES cell selects the CELL A for providing the UL WUS config for NES cell, NES cell needs to know whether the neighbour cell supporting to transmit UL WUS config for NES cell. The NES cell may acquire the capability of neighbour cell by OAM or informed by the neighbour cell. After the CU of NES cell transmits UL WUS config of a list of NES cells to the CU of Cell A, if receiving accept response for a list of NES cells from Cell A, then CU of NES cell determines to enter NES mode for the list of NES cells, and transmits on-demand SIB1 activation to DU of the OD-SIB1 for the NES cells, the NES cells starts the OD-SIB1 operation, e.g., indicating UE the OD-SIB1 of the cell is enabled.

Proposal 2:  If CU of NES cells receive accept response for providing the WUS config for a NES cell from Cell A, CU of the NES cell transmits On-demand SIB1 operation activation command to DU of the NES cell, the NES cell starts the OD-SIB1 operation.

If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of NES cells, Cell A should be considered it starts providing WUS config for the NES cells, i.e., no need further instruction from NES cell. In addition, Cell A should be able to reject for providing WUS config for an NES cell even after having provided WUS config for the NES Cells for a period. If Cell A decides to stop providing WUS config for the NES cells later, it transmits the decision to CU of NES cells, so that NES cells can decide to stop the OD-SIB1 operation by taking into account this decision.

Proposal 3:  If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of the NES cells, Cell A should be considered it starts providing WUS config for the NES cells.

Proposal 4. Cell A should be able to reject for providing WUS config for an NES cell even after having provided WUS config for the NES Cell for a period, but the Cell A needs to notify the decision to the NES cell.

The OD-SIB1 mechanism is similar as on-demand other SI. RAN2 agreed to use the existing Msg 1 based on-demand procedure for on-demand SIB1 acquisition. If receiving request via MSG1, DU can response via MSG2 and broadcasts SIB1. This procedure has no impact on RAN3. Since CU control a cell to use on-demand SIB1, DU may further inform CU the SIB1 is broadcasting if DU broadcasts SIB1 triggered by UE. Whether this notification has no RAN3 impact is FFS, it depends on RAN1 design how to indicate SIB1 is broadcasting. Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB1 operation to the Cell A. If the OD-SIB1 for an NES cell is not needed, the NES cell can request Cell A to release the WUS configuration for the NES cell by WUS configuration exchange message.

Proposal 5: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Proposal 6: Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB operation to the Cell A. If the OD-SIB1 for an NES cell is not needed, NES cell can request Cell A to release the WUS configuration for the NES cell.

Conclusion

In summary, we have following observations and proposals for on-demand SIB1 for Idle/Inactive UE:

Observation 1: R19 focus on case 2 on on-demand SIB1 for idle/inactive mode UE:

Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Proposal 1: CU of NES cell determines which cells are allowed to use on-demand SIB1 and transmits request to DU for the WUS config for an NES cell.

Proposal 2:  If CU of NES cell receiving accept response for providing the WUS config for a NES cell from CU of Cell A, CU of NES cell transmits On-demand SIB1 operation activation command to DU of NES cell, the NES cell starts the OD-SIB1 operation.

Proposal 3:  If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of NES cells, Cell A should be considered it starts providing WUS config for the NES cells.

Proposal 4. Cell A should be able to reject for providing WUS config for an NES cell even after having accepted WUS config for NES Cell, but it needs to notify the decision to the NES cell.

Proposal 5: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Proposal 6: Considering the OD-SIB1 operation for an NES Cell is not so dynamical, it is not necessary to notify the status of OD-SIB operation to the Cell A. If the OD-SIB1 for an NES cell is not needed, NES cell can request Cell A to release the WUS configuration for the NES cell.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-244331, Discussion on On-demand SIB1 for Idle/Inactive UE, Lenovo