3GPP TSG-RAN WG3 Meeting #125bis	R3-245461

Hefei, China, October 14 – 18, 2024

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	AI/ML based positioning accuracy enhancements

Document for:		Discussion and Decision

# 1.	Introduction

The Work Item on artificial intelligence/machine learning for NR air interface has the following objectives on AI/ML positioning [1]:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

The five positioning cases mentioned in the objective above are illustrated in Figure 1 below.

Figure 1: Cases of AI/ML positioning (D-AIML: Direct AI/ML positioning, A-AIML: AI/ML assisted positioning).

Case 1 (and 2nd priority Cases 2a/2b) pertain to UE-based/UE-assisted positioning based on DL-PRS measurements. The general signalling for these cases is between UE and LMF, and therefore, should have no additional RAN3 impacts. Potential RAN3 impacts for these cases may be related to the DL-PRS transmission (e.g., on-demand PRS), which however, is an existing procedure. Any necessary enhancements may depend on RAN1 conclusions (e.g., RAN3 impacts may be limited to the implementation of the RAN1 parameter list for these cases). For Case 1 (and Case 2a) potential RAN3 impacts may also be related to the procedure for UE-sided data collection for model training and validation, which is under study in RAN2/SA2 [2].

Cases 3a/3b are NG-RAN node assisted/network-based positioning methods, and therefore, may have RAN3 specification impacts which are discussed in this contribution.

# 2.	Life Cycle Management (LCM) for Network-based/NG-RAN node assisted positioning (Case 3a/b)

## 2.1	Status in RAN1/RAN2 [3],[4],[5]

For network-based positioning (Case 3a/3b), RAN2 made the following general agreements:

RAN2#125bis For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN2#126 RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

For 'Model Training', RAN1 made the following agreements:

RAN1#116bis For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. RAN1#118: For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement: - For channel measurement generated by TRP/gNB, existing IE "Time Stamp" in TS 38.455 can be reused from RAN1 perspective Note: Purpose, such as above "training data collection", will not necessarily be specified in RAN 1 specifications.

For Case 3b (direct AI/ML positioning with LMF-side model, incl. 2nd priority Case 2b), SA2 concluded on 'model training' [6]:

Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning; the trigger for data collection and for model training in LMF is up to implementation.

The above agreement is already specified in TS 23.273 [7] (incl. 2nd priority Case 2b):

Additional functions which may be performed by an LMF to support location services include the following. […] - Support requesting ML model provision from NWDAF containing MTLF for LMF-based AI/ML Positioning. - Determine whether to use an ML Model for UE positioning. Trigger data collection for training of the ML Model and for monitoring of the performance of the ML Model. The LMF retrieves user consent for data collection for the purpose of ML model training from UDM as defined in TS 23.502. […] The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provides the location to the consumer. […] The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation.

with the following Editor's Notes [7]:

Editor’s note: The procedure of LMF collects input data from NG-RAN for ML model training is FFS. Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

For 'Model Performance Monitoring', RAN2 and RAN1 made the following agreements:

RAN2#125bis For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs.

RAN1#116bis For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A: NG-RAN node performs monitoring metric calculation for its own model. - Option B: LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation. RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A: NG-RAN node performs monitoring metric calculation for its own model. - Option B: LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

For Case 3b (direct AI/ML positioning with LMF-side model, incl. 2nd priority Case 2b), SA2 concluded on 'model performance monitoring' [6]:

Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. (MTLF: Model Training Logical Function [9])

The above agreement is already specified in TS 23.273 [7] (incl. 2nd priority Case 2b):

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

with the following Editor's Note [7]:

Editor's note: The Performance monitoring procedures are FFS.

For 'measurement configuration', RAN1 made the following conclusion:

RAN1#118 From RAN1 perspective, for Case 3a measurements: - The existing procedures can be reused in terms of SRS configuration. Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications.

The above agreements/status are summarized in the Table below.

Table 1: Summary of RAN1/2 and SA2 status/agreements.

|  | Case 3a | Case 3b |
| --- | --- | --- |
| Signalling protocol between gNB and LMF | NRPPa |  |
| Source of training data generation (measurements) | gNB/TRP |  |
| SRS configuration for training/monitoring/inference | Existing procedures |  |
| Model training |  | LMF |
| Label generation for training data | LMF | PRU/UE/LMFNOTE1 |
| Model performance monitoring | gNB or LMF | LMF(MTLF) |
| Label generation for label-based monitoring | gNB or LMF |  |
| NOTE1: RAN1 Working Assumption. |  |  |

## 2.2	Discussion of RAN1/RAN2 assumptions/agreements

### 2.2.1	General

Many aspects summarized in Table 1 above are (at least partly) in the realm of RAN3. The first three items/rows in Table 1 appear rather obvious and are proposed to be confirmed by RAN3. Note, for the SRS configuration (3rd row in Table 1) RAN1 concluded so far on Case 3a only. However, the same should apply to Case 3b, since for both cases, the UE impact is only to transmit (positioning) SRS and existing RAN3 procedures should apply to both cases, i.e., Positioning Information Exchange (with enhancements, if needed).

Proposal 1:	For Case 3a and Case 3b, RAN3 confirms the following RAN1/RAN2 assumptions:

-	NRPPa is used for the signalling between gNB and LMF.

-	For training data generation, the measurements and related attributes (e.g., timestamp) are generated by the TRP/NG-RAN node.

-	The existing procedures for SRS configuration are used (i.e., Positioning Information Exchange).

### 2.2.2	Data collection and Model Training

For Case 3b (LMF-side models), the LMF is assumed to perform 'Model Training'. This also implies that an LMF is collecting the necessary data from the TRPs/NG-RAN to perform the 'Model Training'.

For Case 3a (gNB-side models) 'Model Training', no agreements have been made yet in RAN1/RAN2. The Study Item noted the following [10]:

- Model Training: o For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

Therefore, it seems that there are three options for 'Model Training' in Case 3a: gNB, OAM, LMF.

Proposal 2:	For Case 3a 'Model Training', RAN3 consider the following options:

Option 1:	NG-RAN performs the 'Model Training'.

Option 2:	OAM performs the 'Model Training'.

Option 3:	LMF performs the 'Model Training'.

Proposal 3: 	For Case 3b, RAN3 confirms the following RAN1/RAN2/SA2 assumptions:

-	LMF performs the 'Model Training'.

Required for 'Model Training' is the label generation for the training data. The 'ground truth labels' comprise the 'true' SRS based measurements in Case 3a (AI/ML assisted), and the 'true' UE location in Case 3b (Direct AI/ML). As summarized in section 2.1 above (Table 1), RAN1 (so far) assumes the following:

-	Case 3a: LMF generates the ground truth label.

-	Case 3b: LMF, UE, or PRU generates the ground truth label.

For both, Case 3a and Case 3b, this boils down to having knowledge of the actual UE/PRU position estimate available at the training entity when collecting the training data. In addition, for Case 3a, TRP/ARP location information may be needed, e.g., to determine 'true' UL rTOA or gNB Rx-Tx Time differences, etc.

Location Services (LCS) allows an LCS Client to obtain the location of one or more target UEs [11] using a variety of positioning methods [12]. Location information for one or multiple target UEs may be requested by and reported to an LCS client or an AF within or external to a PLMN, or a control plane NF within a PLMN.

For PRUs, the PRU location would be "automatically" available at an LMF e.g., during PRU registration [11].

These UE/PRU LCS procedures also ensure that user privacy is preserved via various means [11]. Therefore, for ground-truth label generation, existing LCS and UE positioning procedures/methods can be used to obtain the UE/PRU location. This allows determining a desired UE (or PRU) location with any UE and NW supported method(s) (incl. RAT-dependent and RAT-independent methods, as well as network-based methods) and using PRUs with known location (e.g., surveyed location).

Proposal 4:		For Case 3a/3b ground-truth label generation for model training, the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [12] procedures are used to obtain the UE/PRU location estimate by an LMF.

For Case 3b, Proposal 4 together with Proposal 3 results in the following general procedure for data collection shown in Figure 2.

Figure 2: General data collection procedure for LMF-side model training in Case 3b.

1.	In the case the target is a UE, the LMF subscribes to the AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request operation. The AMF then sends a Namf_EventExposure_Subscriber_Response including the list of SUPIs in the area of interest, as defined in [8]. The LMF also checks the user consent for data collection for model training, as defined in [8], [11] (at this Step or at Step 5 below).

NOTE 1:	This step depends on further SA2 progress.

2.	The LMF performs the NRPPa Positioning Information Exchange procedure [14] to request the serving NG-RAN node to configure a required SRS in the target (UE or PRU) [12].

NOTE 2:	Additional parameter values for SRS configuration (if any) depend on RAN1.

3.	The LMF sends a NRPPa Measurement Request [14] to the serving and possibly neighbour NG-RAN nodes (TRPs) to request the SRS-based measurements for model training. This may be a request for any of the existing measurement types and/or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.).

4.	The TRPs provide the measurement results for model training to the LMF in a NRPPa Measurement Response message.

5.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [11] and 38.305 [12] to obtain the UE/PRU location.

NOTE 3:	The positioning method used for obtaining the UE/PRU location should not be based on the same SRS as used for steps 2-4, since the UE/PRU location should be independent on the actual data collected for model training. E.g., the UE/PRU location may be determined via GNSS.

NOTE 4:	In case the target is a PRU, the LMF may already have the PRU location available (e.g., from the PRU registration [11]) and step 5 may not be needed.

NOTE 5:	Step 5 supports the RAN1 assumption that the label can be generated by PRU (i.e., PRU location information reported by PRU/available at LMF), UE location information reported by UE (e.g., using any UE-based positioning method), and LMF determining the UE location using any UE-assisted and/or network-based positioning method.

After step 5, the LMF would have all the data available for 'Model Training', i.e., the measurement data (SRS-based measurements) together with the ground truth label (UE location).

Proposal 5:		For training data generation of AI/ML based positioning Case 3b, RAN3 confirms the following RAN1 Working Assumption:

-	The label and its related data (e.g., time stamp) can be generated by PRU, UE, LMF.

As mentioned in section 2.1 above, there is currently an Editor's Note in TS 23.273 [7]:

Editor’s note: The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

Note, a general corresponding procedure for Case 2b was already agreed by SA2 in [8].

Therefore, it is proposed:

Proposal 6:		Agree the general procedure in Figure 2 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The model output are "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., rTOA, gNB Rx-Tx Time Difference, etc.) or new measurements. For training data generation, the measurements and related attributes (e.g., timestamp) are generated by the TRP/NG-RAN node (per Proposal 1). Therefore, the training data are directly available at the NG-RAN node (gNB/TRP).

However, similar to Case 1 (and Case 2a) for UE-side models, the model training at the gNB may be impractical considering the computational requirements and complexity. For UE side data collection for UE-side model training RAN2 considers a "server for data collection for UE-side model training" [2], which could also be introduced for gNB-side model training. I.e., the gNB obtains the training data (SRS measurements available at gNB, probably together with gNB vendor specific/proprietary information) and provides them to a "server for data collection for gNB-side model training".  This "server for data collection for gNB-side model training" could be physically located anywhere in the network, including NG-RAN or OAM, and does not need to be specified. Using the terminology from [2],[10] the "AI/ML-specific Data Transfer Path" could be 'gNBserver for data collection for gNB-side model training', or 'gNBOAMserver for data collection for gNB-side model training'. Both should have no additional specification impacts. I.e., the model training could be assumed to be "logically" performed by the NG-RAN/gNB.

Option 3 in Proposal 2 (LMF performs the 'Model Training') may also be impractical, since the AI/ML model is typically implementation/vendor specific and may contain proprietary information. The data collected from/by a gNB vendor may not be shared with other gNB or LMF vendors.

Proposal 7: 	For gNB-side data collection for gNB-side model training (Case 3a), RAN3 considers a "Server for data collection for gNB-side model training".

Proposal 8: 	For Case 3a the 'Model Training' is logically performed at the NG-RAN.

The label for the training data can be provided by an LMF upon request, as currently assumed by RAN1 (see Table 1 in section 2.1 above). According to Proposal 4, an LMF can use any of the existing LCS [11] and UE positioning [12] procedures to obtain the ground-truth label. The NG-RAN node could send a request for label delivery to an LMF, as illustrated in Figure 3.

Figure 3: General data collection procedure for gNB-side model training in Case 3a.

1.	The UE/PRU SRS transmission may be initiated by the serving gNB.

2.	The serving gNB measures the SRS and derives any data required for model training.

NOTE 1:	The gNB-side model training may be physically performed by the "Server for data collection for gNB-side model training" according to Proposal 7 and does not need to be specified.

3.	The serving gNB sends a request to an LMF via the serving AMF for the ground truth label.

This could be a new NGAP message [13] followed by a Namf service operation between AMF and LMF, or a new NRPPa message [14]. 
For example, the serving gNB could send an 'Uplink UE Associated NRPPa Transport message' [13] to the serving AMF. The Routing ID in the 'Uplink UE Associated NRPPa Transport message' could be a special/reserved value to indicate to the AMF that this is a gNB triggered NRPPa message. A range of Routing ID values could be reserved for gNB triggered NRPPa messages (i.e., a range of Routing IDs which cannot be assigned by an AMF would enable an AMF to distinguish a Routing ID assigned by a gNB from Routing IDs assigned by an AMF). The AMF could then assign a LCS Session ID (Correlation ID) for this UE NGAP ID. The AMF would select a proper LMF and provide the NRPPa message to the LMF using the Namf_Communication_N2InfoNotify operation.

4.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [11] and 38.305 [12] to obtain the UE location.

NOTE 2:	The LMF may check the user consent for data collection for model training in the same way as in Figure 2 (Case 3b) [8].

5.	The LMF provides the UE (or PRU) location ('ground-truth label') to the serving gNB (via the serving AMF).
If an NRPPa message is used for Step 3, this could also be an NRPPa message similar to Step 3.

Proposal 9: 	RAN3 confirms the following RAN1 assumption:

-	For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) are generated by an LMF.

Proposal 10: 	Define a procedure (e.g., NGAP or NRPPa) which allows a gNB to request ground-truth labels from an LMF.

For LMF-side model training (Case 3b), there may be an additional aspect to consider: The NRPPa Positioning Information Request with the Requested SRS Transmission Characteristics (Step 2a in Figure 2) currently allows an LMF to request only some general SRS characteristics and the final SRS configuration is decided by the serving gNB [14], [12]. However, an LMF may need to train models valid for a range of different SRS configurations. For the inference phase, we assume that existing positioning procedures can be used (see also section 2.3.3 below). In these procedures, the gNB may provide a particular SRS configuration to the LMF (decided by the gNB) and the LMF would need to search for an applicable model in its model repository suitable to this particular SRS configuration. Therefore, when using the Requested SRS Transmission Characteristics for model training, a LMF may need to specify in more detail a required (and not desired) SRS configuration, such that the LMF can train models for a variety of SRS configurations. The NRPPa Validity Area Specific SRS Information [14] already allows an LMF to specify a more detailed requested SRS characteristics, but currently only when area-specific SRS is requested. This may need to be extended to the 'Model Training' use case, and the 'desired SRS configuration' may need to be defined as a 'required SRS configuration'.

Observation 1:	For LMF-side model training, the Requested SRS Transmission Characteristics may need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

### 2.2.3	Model Performance Monitoring

For LMF-side models (Case 3b), RAN2 and SA2 assumes that the LMF is performing the 'Model Performance Monitoring'. This seems sensible since the same procedure as for data collection could be used for performance monitoring. The procedure in Figure 2 would then have an additional 6th step to determine the desired model performance metric using the calculated UE position with the measurements from Step 4 in Figure 2 and comparing the result with the 'ground-truth label' from Step 5 in Figure 2. As discussed above related to Observation 1, a LMF may have several trained models corresponding to several different SRS configurations available. Therefore, also for Model Performance Monitoring, a LMF may need to be enabled to specify a more detailed required SRS Configuration in Step 2a of Figure 2 (see Observation 1).

Proposal 11:	For Case 3b, RAN3 confirms the following RAN2/SA2 assumption:

-	LMF/MTLF is responsible for the performance monitoring for Case 3b.

For gNB-side models (Case 3a), RAN1/RAN2 assumes that the gNB performs model performance monitoring metric calculation for its own model (Option A), or LMF performs the performance monitoring metric calculation for the model located at the gNB (Option B). The agreement from RAN1#118 has the following Note (see section 2.1):

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

For the gNB-side model, the procedure in Figure 3 could also be used for 'Model Performance Monitoring'. The gNB could measure the SRS and derive the model input data and perform the model inference to obtain the "SRS-based measurements" (e.g., AI/ML enhanced legacy measurements (e.g., rTOA, UE Rx-Tx Time Difference Measurements) or any new measurements, dependent on RAN1 progress. To obtain the 'ground-truth label', the gNB would need the 'true' UE location and the 'true' gNB/ARP location (for example, to determine a 'true' rTOA, which depends on the distance between UE and gNB/ARP). Assuming the gNB has its own gNB/ARP location coordinates available, the procedure for gNB-side 'Model Performance Monitoring' could be the same as in Figure 3 with an additional Step 6 to derive the model performance metric based on the model output and UE/gNB/ARP location. There seems no need/use case for the LMF performing the 'Model Performance Monitoring' for gNB-side models (Case 3a).

Proposal 12:	For performance monitoring metric calculation for Case 3a in label-based monitoring, the NG-RAN node performs monitoring metric calculation for its own model.

Observation 2:	The model performance monitoring at gNB-side can share the same procedures as for model training.

Note, similar to Proposal 7, the monitoring metric calculation in Case 3a may be physically performed by the "Server for data collection for gNB-side model training", which does not need to be specified.

## 2.3	Additional LCM Aspects

### 2.3.1	Network-side (additional) Conditions

The performance of an AI/ML model for positioning may be degraded when network conditions changed between model training and inference. A discrepancy between training and inference could occur for example due to:

-	changes in network synchronization (e.g., SFN Initialization Time);

-	changes to gNB timing error (e.g., TEGs);

-	changes to TRP/ARP locations and antenna orientation;

-	changes to TRP Rx beams;

-	changes to TRP Rx ARP-IDs for SRS measurements.

The above and other information may be investigated by RAN1. However, the network conditions should be available in the NG-RAN (Case 3a) or may be provided to an LMF via Positioning Information Exchange [14] (Case 3b). Additional parameter for the Positioning Information Exchange could be added, based on RAN1 agreements.

Observation 3:	For Case3a/3b, the network-side (additional) conditions that may affect the training and inference consistency can already be available at gNB and LMF (via existing signalling). Additional parameter (if needed/defined by RAN1) can be added to the TRP Information Exchange procedure (Case 3b).

### 2.3.2	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [10] defines in section 7.2.4 the following:

- Management: o For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. o For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB. o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For gNB-side models (Case 3a), the functionality control can be inside the gNB. The gNB may decide on a particular model based on e.g., requested measurement type, currently used SRS configuration, etc. When requesting measurements from a gNB, the LMF specifies the measurements needed and provides all relevant information to the gNB in the NRPPa Measurement Request [14], incl. e.g., the SRS configuration. Therefore, a gNB should have all information available to perform the functionality management operation of activating/deactivating a particular model or fallback to non-AI/ML based measurements. Enhancements may be needed to the particular messages (e.g., additional parameter to request AI/ML enhanced measurements, etc.) which however, depend on further RAN1 progress. In any case, existing procedures can be used for the positioning operation (see also section 2.3.3 below). Therefore, it is proposed to confirm the study item conclusion:

Proposal 13:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

For LMF-side model (Case 3b), the LMF can request measurements from the gNBs as usual [12]. The requested measurements may be any of the existing measurement types (rTOA, AOA, gNB Rx-Tx Time Difference, etc.) or any new measurements defined by RAN1. The LMF performs the inference operation based on available models. All information required to perform model selection, activation, switching, fallback, etc. should be available at the LMF. Per the study item conclusion mentioned above, the model/functionality control may be performed by the LMF when the monitoring resides within the LMF, which is the case per Proposal 11. Therefore, it is proposed to confirm the study item conclusion:

Proposal 14:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

Observation 4:	For Case 3a/3b model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.), existing positioning procedures can be used.

### 2.3.3	Model inference operation

For Case 3a model inference, model input data are internally available at the gNB and the inference operation is running internally on the gNB. The model output for Case 3a are AI/ML enhanced SRS measurements (e.g., existing measurements and/or any additional measurements defined by RAN1). NG-RAN node measurements can be reported via the NRPPa Measurement Information Transfer procedures [14].

For Case 3b, the NG-RAN nodes provide SRS measurements, and the model inference is running at the LMF. Existing positioning procedures can be used in both, Case 3a and Case 3b.

For example, Figure 5 below shows the typical UL-TDOA positioning procedure from [12]. The LMF may request a required SRS at Step 2 and requests measurements at Step 6. A gNB may provide AI/ML enhanced measurements (Case 3a) or 'normal' measurements (Case 3b). There may be specification impacts to specifically request AI/ML enhanced measurements, request a specific SRS configuration, etc. dependent on RAN1 progress. However, the overall positioning procedures do not require changes.

Figure 5: Exemplary UL-TDOA positioning procedure

Observation 5:	For Case 3a/3b model inference, existing positioning procedures can be used.

# 3. Summary

In this contribution, we discussed the AI/ML enhanced network-based positioning cases (Case 3a/b) based on the progress in RAN1/RAN2 and SA2. From the discussion we observe that AI/ML enhanced network-based positioning can largely be supported with existing positioning/NRPPa procedures. Enhancements (e.g., new measurement parameter, etc.) may be needed based on further RAN1 progress as usual (i.e., implementation of RAN1 parameter list).

The main RAN3 impact we identified in this contribution is the need for a new NG-RAN node triggered (e.g., NRPPa) procedure towards an LMF to support the various functions related to model training and performance monitoring for gNB-side models (Case 3a).

The following Observations and Proposals were made.

Proposal 1:	For Case 3a and Case 3b, RAN3 confirms the following RAN1/RAN2 assumptions:

-	NRPPa is used for the signalling between gNB and LMF.

-	For training data generation, the measurements and related attributes (e.g., timestamp) are generated by the TRP/NG-RAN node.

-	The existing procedures for SRS configuration are used (i.e., Positioning Information Exchange).

Proposal 2:	For Case 3a 'Model Training', RAN3 consider the following options:

Option 1:	NG-RAN performs the 'Model Training'.

Option 2:	OAM performs the 'Model Training'.

Option 3:	LMF performs the 'Model Training'.

Proposal 3: 	For Case 3b, RAN3 confirms the following RAN1/RAN2/SA2 assumptions:

-	LMF performs the 'Model Training'.

Proposal 4:		For Case 3a/3b ground-truth label generation for model training, the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [12] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 5:		For training data generation of AI/ML based positioning Case 3b, RAN3 confirms the following RAN1 Working Assumption:

-	The label and its related data (e.g., time stamp) can be generated by PRU, UE, LMF.

Proposal 6:		Agree the general procedure in Figure 2 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

Proposal 7: 	For gNB-side data collection for gNB-side model training (Case 3a), RAN3 considers a "Server for data collection for gNB-side model training".

Proposal 8: 	For Case 3a the 'Model Training' is logically performed at the NG-RAN.

Proposal 9: 	RAN3 confirms the following RAN1 assumption:

-	For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) are generated by an LMF.

Proposal 10: 	Define a procedure (e.g., NGAP or NRPPa) which allows a gNB to request ground-truth labels from an LMF.

Observation 1:	For LMF-side model training, the Requested SRS Transmission Characteristics may need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

Proposal 11:	For Case 3b, RAN3 confirms the following RAN2/SA2 assumption:

-	LMF/MTLF is responsible for the performance monitoring for Case 3b.

Proposal 12:	For performance monitoring metric calculation for Case 3a in label-based monitoring, the NG-RAN node performs monitoring metric calculation for its own model.

Observation 2:	The model performance monitoring at gNB-side can share the same procedures as for model training.

Observation 3:	For Case3a/3b, the network-side (additional) conditions that may affect the training and inference consistency can already be available at gNB and LMF (via existing signalling). Additional parameter (if needed/defined by RAN1) can be added to the TRP Information Exchange procedure (Case 3b).

Proposal 13:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

Proposal 14:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

Observation 4:	For Case 3a/3b model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.), existing positioning procedures can be used.

Observation 5:	For Case 3a/3b model inference, existing positioning procedures can be used.

# References

[1]	RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	RP-242389, "LS on AIML data collection".

[3]	RP-240499, "Status Report to TSG RAN#103".

[4]	RP-241153, "Status Report to TSG RAN#104".

[5]	RP-242398, "Status Report to TSG RAN#105".

[6]	S2-2407188, "Interim conclusions for KI#1", SA2#163.

[7]	S2-2409414, CR to 23.273: "Introduction of LMF enhancement for Direct AI/ML based Positioning".

[8]	S2-2409415, CR to 23.273: "KI#1 - LMF enhancements for UE positioning using a ML model".

[9]	3GPP TS 23.288, "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[10]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[11]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[12]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[13]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[14]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".