3GPP TSG-RAN WG3 Meeting #125bis	R3-245465

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1334 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE specific SRS reservation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT,Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, SRS Information Reservation Notification procedure was introduced in the NRPPa to reserve the area-specific SRS configuration(s). However, the target gNBs in the validity area(s) could not identify the reserved SRS configuration(s) for the UE, it still need to retrieve the area-specific SRS configuration(s) from the last serving gNB. To make use of reserved SRS configuration(s) more efficiently, it’s better to link the reserved SRS configuration(s) with the I-RNTI. This CR provides the Xn changes to support activation of pre-configured SRS for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: A new indicator Preconfigured SRS activated IE, is introduced to advice the olf NG-RAN node that the pre-configured SRS are activated. The old NG-RAN node keeps the context, then replies with RETRIEVE UE CONTEXT FAILURE, with regards of Old NG-RAN node To New NG-RAN node Resume Container IE to release the UE and then it should contact then the LMF... Impact Analysis: The CR has a protocol and function impact. The impact is limited to the positioning SRS preconfigured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE specific SRS reservation and activation are not well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.3, 9.1.1.8, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.473 CR1457 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR 38.455 CR0159 TS/TR 38.305 CR (RAN2 allocation) |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: re-sumission at RAN3#125bis |  |  |  |  |  |  |  |  |

### //****** Beginning of Change ******//

### 8.2.4	Retrieve UE Context

### //****** No of Change ******//

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU or in case of RACH based SDT, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

If the Preconfigured SRS activated IE set to "true" is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN shall, if supported, respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message with the Old NG-RAN node to New NG-RAN node Resume Container IE and make corresponding response to LMF when positioning a UE.

Interaction with Partial UE Context Transfer procedure

In case of RACH based SDT, if the old NG-RAN node decides to not transfer/relocate the UE Context to the new NG-RAN node, it may trigger the Partial UE Context Transfer procedure as specified in TS 38.300 [9]. After the old NG-RAN node has decided to end the SDT session, it shall terminate the Retrieve UE Context procedure by sending the RETRIEVE UE CONTEXT FAILURE message.

#### 8.2.4.4	Abnormal Conditions

### //****** Next of Change ******//

#### 9.1.1.8	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new NG-RAN node to request the old NG-RAN node to transfer the UE Context to the new NG-RAN.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | reject |
| UE Context ID | M |  | 9.2.3.40 |  | YES | reject |
| Integrity protection | M |  | BIT STRING (SIZE (16)) | RRC Resume: Corresponds to information provided either in the resumeMAC-I either contained in the RRCResumeRequest or the RRCResumeRequest1 message as defined in TS 38.331 [10]) or in the shortResumeMAC-I in the RRCConnectionResumeRequest message as defined in TS 36.331 [14]) RRC Reestablishment: Corresponds to information provided either in the shortMAC-I contained in the RRCReestablishmentRequest message as defined in TS 38.331 [10]) or in the shortMAC-I in the RRCConnectionReestablishmentRequest message as defined in TS 36.331 [14]). RRC Resume for UP CIoT Optimization: Corresponds to information provided in the shortResumeMAC-I in the RRCConnectionResumeRequest message or the RRCConnectionResumeRequest-NB message as defined in TS 36.331 [14]. | YES | reject |
| New Cell Identifier | M |  | NG-RAN Cell Identity 9.2.2.9 | RRC Resume: Corresponds to information provided either in the targetCellIdentity within the VarResumeMAC-Input as specified in TS 38.331 [10] or in the cellIdentity within the VarShortINACTIVE-MAC-Input as specified in TS 36.331 [14]. RRC Reestablishment: Corresponds to information provided in the targetCellIdentity within the VarShortMAC-Input as specified in TS 38.331 [10] or in the cellIdentity within the VarShortMAC-Input as specified in TS 36.331 [14]. RRC Resume for UP CIoT Optimization: Corresponds to information provided either in the cellIdentity within the VarShortResumeMAC-Input or the VarShortResumeMAC-Input-NB as specified in TS 36.331 [14]. | YES | reject |
| RRC Resume Cause | O |  | 9.2.3.61 | In case of RNA Update, contains information provided in the resumeCause by the UE in the RRCResumeRequest or the RRCResumeRequest1 message, as defined in TS 38.331 [10], or information provided in the resumeCause-r15 in the RRCConnectionResumeRequest message, as defined in TS 36.331 [14]. | YES | ignore |
| SDT Support Request | O |  | 9.2.3.163 |  | YES | ignore |
| SRS Positioning Configuration Or Activation Request | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Preconfigured SRS activated | O |  | (true, …) |  | YES | ignore |

### //****** Next of Change ******//

### //****** Beginning of Change ******//

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

### //****** Next of Change ******//

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

### //****** Next of Change ******//

ReportCharacteristicsForDataCollection,

ReportingPeriodicityForDataCollection,

NodeAssociatedInfoResult,

SLPositioning-Ranging-Services-Info,

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

PreconfiguredSRSActivated

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

### //****** Next of Change ******//

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-PreconfiguredSRSActivated,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

### //****** Next of Change ******//

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UEContextID								CRITICALITY reject		TYPE UEContextID									PRESENCE mandatory}|

{ ID id-MAC-I									CRITICALITY reject		TYPE MAC-I											PRESENCE mandatory}|

{ ID id-new-NG-RAN-Cell-Identity				CRITICALITY reject		TYPE NG-RAN-Cell-Identity						PRESENCE mandatory}|

{ ID id-RRCResumeCause							CRITICALITY ignore		TYPE RRCResumeCause									PRESENCE optional }|

{ ID id-SDTSupportRequest						CRITICALITY ignore		TYPE SDTSupportRequest							PRESENCE optional }|

{ ID id-SRSPositioningConfigOrActivationRequest	CRITICALITY ignore	TYPE SRSPositioningConfigOrActivationRequest	PRESENCE optional }|

{ ID id-PreconfiguredSRSActivated				CRITICALITY ignore	TYPE PreconfiguredSRSActivated						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

### //****** Next of Change ******//

}

### 9.3.5	Information Element definitions

### //****** Next of Change ******//

PosPartialUEContextInfo ::= SEQUENCE {

requestedSRSTransmissionCharacteristics		RequestedSRSTransmissionCharacteristics	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosPartialUEContextInfo-ExtIEs} }	OPTIONAL,

...

}

PosPartialUEContextInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PreconfiguredSRSActivated::= ENUMERATED {true, ...}

PredictedUETrajectory-Item ::= SEQUENCE{

predictedtrajectoryCellInfo		PredictedTrajectoryCellInfo,

iE-Extensions					ProtocolExtensionContainer { { PredictedUETrajectory-Item-ExtIEs} }	OPTIONAL,

...

}

PredictedUETrajectory-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

### //****** Next of Change ******//

### 9.3.7	Constant definitions

-- ASN1START

### //****** Next of Change ******//

ProtocolIE-ID ::= 456

id-Source-M-NG-RANnodeID																			ProtocolIE-ID ::= 457

id-S-CPAC-CompleteConfig-Indicator																	ProtocolIE-ID ::= 458

id-SourceSN-to-TargetSN-QMCInfo																		ProtocolIE-ID ::= 459

id-RegistrationRequestForDataCollection																ProtocolIE-ID ::= 460

id-ReportCharacteristicsForDataCollection															ProtocolIE-ID ::= 461

id-ReportingPeriodicityForDataCollection															ProtocolIE-ID ::= 462

id-NodeAssociatedInfoResult																			ProtocolIE-ID ::= 463

id-SLPositioning-Ranging-Services-Info																ProtocolIE-ID ::= 464

id-XR-Bcast-Information					 															ProtocolIE-ID ::= 465

id-PDUSessionsListToBeReleased-UPError																ProtocolIE-ID ::= 466

id-MaximumDataBurstVolume																			ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-PreconfiguredSRSActivated															ProtocolIE-ID ::= x

### //****** End of Change ******//