3GPP TSG-RAN3 Meeting # 125bis 	R3-245466

# Hefei, P. R. China,  14th – 18th October, 2024

Agenda Item:	8.1

Source:	Ericsson

Title :	Discussion on SA2 LS regarding Multimodality application and protocol impacts

Document for:	Discussions & Approval

# Introduction

In this contribution, we discuss the incoming LS from SA2 [1], where SA2 asks RAN3 for feedback on the protocol impacts of signalling the MMSID from CN to RAN via CP, and the applicability of this information from the usage discussed by RAN2 in Rel-19 XR work. The questions are repeated below highlighting the RAN3 queries.

1. Overall Description: SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. 2. Actions: To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively.

In this contribution we present our response to these questions.

# Background and Discussion

During the Rel-19 XR study phase, RAN2 agreed the following regarding multi-modality:

Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. FFS whether this can be applied to DL RAN2 considers that based on multi-modal information: The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation).

RAN2 has made agreement that MMSID can be used by NG-RAN to perform some new functionality called “joint admission control”. However, admission control mechanisms are based on network vendor implementation-specific decisions. In addition, content originating from applications, services and the admission control mechanism does not fall within the domain of RAN2. Consequently, RAN3 and SA2 must evaluate the agreement captured by RAN2.

Observation 1: The agreement made by RAN2 “The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.” Is outside of RAN2’s ToR. This needs assessment from RAN3 group.

Nevertheless, RAN2 have sent a LS to SA2 in [2] asking whether/what multi-modal information can be provided from SA2 to allow e.g., “coordinated handling” of the flows at RAN. SA2 has replied in [1] (extract in section 1), mentioning the existence of Multi-modal Service ID (MMSID) in AF and PCF specified in TS 23.501 clause 5.37.2. We copy below the definition of MMSID from SA2 spec:

### 5.37.2 Policy control enhancements to support multi-modal services A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session. The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements: - The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application. - The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow. NOTE: In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from AF within a single request or, in case of multiple requests, within a short period of time.

SA2 further asks RAN3 to comment on the feasibility of signalling such information and the protocol impacts. Based on the above extract, we can understand the following:

- An MMSID indicates that data flows are closely related;

- PCF considers the MMSID when generating the QoS requirements.

- How the MMSID affects RAN is not specified, nor seems needed,

In that case, the SMF can get the MMSID along with QoS information. If there are several QoS flows that are linked with the same MMSID, the SMF can know and potentially add this indication as either a new IE in the QoS Flow Level QoS Parameters IE, or added per QFI in the QoS Flow Setup Request Item IE.

Observation 2: According to the definition of MMSID, it can indicate if several QoS flows are related and require strong application coordination. SA2 did not further specify in which way RAN should process/take into account the MMSID.

Observation 3: The SMF can be aware of the MMSID from the PCF together with the QoS information; and SMF can indicate it per QFI in the QoS flow Level QoS Parameters IE in the message to RAN.

With respect to the usage of MMSID in RAN, admission control mechanisms as mentioned are implementation specific. The same for QoS low to DRB mapping. It can be for example of video calls, as the voice data flow can be established by the system even when there are no resources to establish the video data flow, but the service can still operate. In general, not all multimodal applications will require all media flows to operate, but some may be optional/complementary, but “good” for the RAN to know. Therefore, RAN3 shall not specify how the MMSID is used at RAN; it suffices for RAN to just be aware of its existence in the QoS flow information that the QoS flows are related. For instance, when receiving the MMSID for  particular QoS flows over NG during the PDU Session management procedures, the NG-RAN shall, if supported, consider that the indicated QoS flows are related to a multi-modal service, as described in TS 23.501.

Observation 4: Admission control and QoS-flow to DRB mapping are vendor implementation specific mechanisms. MMSID is for awareness purpose only at RAN level that some flows are related and can have similar treatment. How RAN uses such information and what “treatment” is performed are up to implementation.

Proposal 1: RAN3 shall not specify how the MMSID is used by NG-RAN. If MMSID for a particular QoS flow is signalled over NG during the PDU Session management procedures, the NG-RAN shall, if supported, simply consider that the indicated QoS flows are related to a multi-modal service, as described in TS 23.501.

From the above, it can hence be possible to add an indication of the MMSID in QoS Flow Level QoS Parameters IE signalled in NGAP, which seems to have the minimum impacts on the protocol. Regarding the other impacted interfaces:

In XnAP:

- MMSID can be signalled from NG-RAN to another NG-RAN, within the QoS Flow Level QoS Parameters IE in the PDU session transfer containers, during e.g.,  handover, context retrieval or dual connectivity procedures (between MN and SN).

In F1AP:

- We expect that typically the gNB-CU will use the MMSID to decide how to perform QoS flow to DRB mapping and that providing the MMSID via F1 is not needed. However, it can still be sent for information as part of the QoS profile. (FFS)

In E1AP:

- No use case for signalling MMSID to gNB-CU-UP

Hence, the MMSID is processed in the gNB-CU only. (FFS)

Proposal 2: The MMSID can be signalled over Xn, within the QoS Flow Level QoS Parameters IE in the PDU session transfer containers, during e.g.,  handover, context retrieval or dual connectivity procedures (between MN and SN.

Proposal 3 : the MMSID can be signalled over F1 for information and no need to signal MMSID over E1 interface.

We attach a response LS to RAN2 and SA2 in [3]. CRs to NGAP, F1AP and XnAP adding the MMSID in the QoS Flow QoS level parameters IEs are described in [4], [5] and [6], respectively.

# Conclusion

In this contribution we discuss the questions to RAN3 from the SA2 LS [1].

Observations and proposals are listed below:

Observation 1: The agreement made by RAN2 “The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.” is outside of RAN2’s ToR. This needs assessment from RAN3 group.

Observation 2: According to the definition of MMSID, it can indicate if several QoS flows are related and require strong application coordination. SA2 did not further specify in which way RAN should process/take into account the MMSID.

Observation 3: The SMF can be aware of the MMSID from the PCF together with the QoS information; and SMF can indicate it per QFI in the QoS flow Level QoS Parameters IE in the message to RAN.

Observation 4: Admission control and QoS-flow to DRB mapping are vendor implementation specific mechanisms. MMSID is for awareness purpose only at RAN level that some flows are related and can have similar treatment. How RAN uses such information and what “treatment” is performed are up to implementation.

Proposal 1: RAN3 shall not specify how the MMSID is used by NG-RAN. If MMSID for a particular QoS flow is signalled over NG during the PDU Session management procedures, the NG-RAN shall, if supported, simply consider that the indicated QoS flows are related to a multi-modal service, as described in TS 23.501.

Proposal 2: The MMSID can be signalled over Xn, within the QoS Flow Level QoS Parameters IE in the PDU session transfer containers, during e.g.,  handover, context retrieval or dual connectivity procedures (between MN and SN.

Proposal 3 : the MMSID can be signalled over F1 for information and no need to signal MMSID over E1 interface.

# References

[1] R3-245017, LS reply on multi-modality awareness at RAN, SA2

[2] R2-2405782, LS on multi-modality awareness at RAN, RAN2

[3] R3-245470, Reply LS on multi-modality awareness at RAN, (draft), Ericsson, RAN3

[4] R3-245467, Addition of MMSID, NGAP CR, Ericsson

[5] R3-245468, Addition of MMSID, F1AP CR, Ericsson

[6] R3-245469, Addition of MMSID, XnAP CR, Ericsson