3GPP TSG-RAN3 Meeting #125bis	R3-245489

Heifei, China, Oct 14-18, 2024

Agenda Item:	31.1

Source: 		Qualcomm Incorporated, Vodofone Group Plc, Vodofone Telekomünikasyon A.S.

Title: 	Discussion on Time Reference Distribution Subscription for UEs in LTE

Document for:	Discussion/Decision

# Background

SA2 agreed a CR S2-2409198 in S2#164 to enable subscription control for reference time distribution for LTE. This was a small work item (e.g. similar in size, or smaller, to those agreed in the recent RAN#105) with the work item code TEI19_TIME_SUB_EPS supported by various operators. This change needs to be replicated over S1AP from MME to eNB. In this paper we provide our proposals for S1AP change based on SA2 CR approved at SA #105.

S2-2409198 4.3.X Support for Time Reference Information Distribution Support for time reference information distribution in RRC is specified in TS 36.331 [37]. This feature is optional. When it is supported, it enables the operator to control, based on subscription, whether to deliver time reference information to a UE. The MME receives the indication from the HSS that the UE is subscribed to receive time reference information in access stratum. If the MME has received the indication that the UE is subscribed to receive time reference information, then the MME provides a Time Reference Information Distribution Indication to the eNodeB whenever the UE context is established in E-UTRAN, e.g., during the Attach procedure, Service Request procedure, TAU procedure and handover procedures. If an eNodeB receives the Time Reference Information Distribution Indication and if the eNodeB supports time reference information distribution, then the eNodeB shall provide the time reference information to the UE in a unicast RRC message as specified in TS 36.331 [37]. The UE may further distribute the time reference information to devices connected to the UE using implementation-specific means.

# Discussion

In SA2 CR S2-2409198, it was agreed to provision operator to provide subscription-based time reference information to the UE. When MME received the UE subscription information from HSS, MME need to provide the indication on the subscription to eNB in Attach, Service Request, TAU and handover procedures. eNB if it supports this feature and also get the time reference distribution subscription information from MME, forwards this information to UE via unicast RRC message as per the subscription.

In SA2 approved CR S2-2309198, it is agreed to provide flexibility for operator to control the Time reference information distribution to UE based on UE’s subscription.

As per the SA2 CR, when MME receives the UE subscription information on time reference information, MME provides this information to eNB in Attach, Service Request, TAU and handover procedures and eNB forwards the same to the UE via unicast RRC message.

In order to align with the SA2 CR, UE’s subscription to the time reference distribution information needs to be sent from MME to eNB in the following S1AP procedures as an indication.

- Initial Context Setup Request

- UE Context Modification Request

- Handover Request

- Path Switch Request Acknowledge

RAN3 to agree to include Time reference distribution information indication from MME to eNB as an enumerated IE in the following messages. Stage 3 CR in R3-245642

- Initial Context Setup Request

- UE Context Modification Request

- Handover Request

- Path Switch Request Acknowledge

# Summary

Based on the discussion above, we have following observations:

Observation 1.	In SA2 approved CR S2-2309198, it is agreed to provide flexibility for operator to control the Time reference information distribution to UE based on UE’s subscription.

Observation 2.	As per the SA2 CR, when MME receives the UE subscription information on time reference information, MME provides this information to eNB in Attach, Service Request, TAU and handover procedures and eNB forwards the same to the UE via unicast RRC message.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to agree to include Time reference distribution information indication from MME to eNB as an enumerated IE in the following messages. Stage 3 CR in R3-245642

	Initial Context Setup Request

	UE Context Modification Request

	Handover Request

	Path Switch Request Acknowledge