3GPP TSG-RAN WG3 Meeting #125-bis	R3-245500

Hefei, P.R.C, 14th Oct – 18th Oct 2024

Title: 	(TP for BLCR for AIML for 38.423)  Support for AI/ML based Network Slicing

Source: 	Ericsson

Agenda item:	11.2 AI/ML enabled Slicing

Document Type:	Others

# Introduction

Int this contribution we provide a Text Proposal for 3GPP TS 38.423 V18.3.0 (2024-09) on AI/ML based Network Slicing. The TP reflects the proposals we presented in R3-245499.

# Text Proposal for TS 38.423 V18.3.0 (2024-09)

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates slice-specific information reporting, the Slice To Report for Data Collection List IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average UE Throughput DL IE, if the tenth bit, "Slice Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average UE Throughput UL IE, if the eleventh bit, "Slice Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Delay IE, if the twelfth bit, "Slice Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Loss DL IE, if the thirteenth bit, "Slice Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the fourteenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

#### 8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.4.13.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the NG-RAN node1 may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the NG-RAN node2 Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the NG-RAN node2 shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going Data Collection reporting, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.4.14	Data Collection Reporting

#### 8.4.14.1	General

This procedure is initiated by an NG-RAN node to report information accepted by the NG-RAN node following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.14.2	Successful Operation

Figure 8.4.14.2-1: Data Collection Reporting, successful operation

NG-RAN node2 shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.4.14.3	Unsuccessful Operation

Not applicable.

#### 8.4.14.4	Abnormal Conditions

Void.

/*Next Change*/

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Slice Average UE Throughput DL, Eleventh Bit = Slice Average UE Throughput UL, Twelfth Bit = Slice Average Packet Delay, Thirteenth Bit = Slice Average Packet Loss DL Fourteenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report for Data Collection List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report for Data Collection Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.1.3.28	DATA COLLECTION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Slice UE Performance | O |  | 9.2.3.x |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

/*Next Change*/

#### 9.2.3.x	Slice UE Performance

This IE indicates the per Slice, per PDU Session, UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>PDU Session Performance List |  | 1 |  | It indicates the UE performance for each PDU Session associated to an S-NSSAI |
| >>>>PDU Session Performance Item |  | 1..<maxnoofPDUSessions> |  |  |
| >>>>PDU Session ID | M |  | 9.3.1.50 |  |
| >>>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

/*Next Change*/

CellToReportForDataCollection-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

sliceToReportForDataCollection-List						SliceToReportForDataCollection-List			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellToReportForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellToReportForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

/*Next Change*/

CellInfoResultForDataCollection-Item ::= SEQUENCE {

cellID											GlobalNG-RANCell-ID,

predictedRadioResourceStatus					RadioResourceStatus					OPTIONAL,

predictedNumberofActiveUEs						NumberofActiveUEs					OPTIONAL,

predictedRRCConnections							RRCConnections						OPTIONAL,

predictedSliceAvailableCapacity					SliceAvailableCapacity			OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {CellInfoResultForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellInfoResultForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

/*Next Change*/

SliceToReport-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions						ProtocolExtensionContainer { { SliceToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReportForDataCollection-Item

SliceToReportForDataCollection-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions						ProtocolExtensionContainer { { SliceToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReportForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }	OPTIONAL

}

SNSSAI-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

/*Next Change*/

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

sliceUEPerformance							SliceUEPerformance			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

/*Next Change*/

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceUEPerformance ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceUEPerformance-Item

SliceUEPerformance-Item	::= SEQUENCE {

pLMNIdentity						PLMN-Identity,

sNSSAIUEPerformance-List		SNSSAIUEPerformance-List,

iE-Extensions						ProtocolExtensionContainer { { SliceAvailableCapacity-Item-ExtIEs} }	OPTIONAL,

...

}

SliceUEPerformance-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIUEPerformance-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIUEPerformance-Item

SNSSAIUEPerformance-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

pduSessionPerformance-List		PDUSessionPerformance-List,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

SNSSAIUEPerformance-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionPerformance-List ::= SEQUENCE (SIZE(1.. maxnoofPDUSessions)) OF PDUSessionPerformance-Item

PDUSessionPerformance-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sliceBasedUEPerformance		UEPerformance,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

PDUSessionPerformance-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformanceCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUEPerformance				INTEGER(1..5000, ...),

iE-Extensions								ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,

...

}

UEPerformanceCollectionConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}