3GPP TSG-RAN WG3 Meeting #125-bis	R3-245502

Hefei, P.R.C, 14th Oct – 18th Oct, 2024

Title: 	Signalling aspects of AI/ML support for CCO

Source: 	Ericsson, Deutsche Telekom, InterDigital

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

At RAN3#125 the SI on AI/ML in RAN in Rel-19 was completed, and one of the conclusions captured in the TR 38.743 is to proceed with normative work for AI/ML based Coverage and Capacity Optimization (CCO) use case. In this contribution, we discuss aspects related to the enhancements needed for signalling over the NG-RAN interfaces (for split and non-split architecture) to support this use case. The proposals are derived from the discussion paper in R3-245501.

# 2. Discussion

## F1 signalling design (gNB-CU to gNB-DU)

As explained in R3-245501, the F1 signalling from gNB-CU to gNB-DU enables the gNB-CU to indicate a certain type of predicted CCO issue, which cells or beams will be affected, and a timing information (when the CCO issue will occur). Although the logic is similar to the legacy CCO, we think the information exchanged to support legacy CCO should be enhanced in a way that it can be possible to distinguish what is legacy information and what is information exchanged to support AI/ML based CCO. This separation is possible by reusing existing F1 and Xn messages. This would ensure that the existing procedures for CCO remain the ones also used for AI/ML supported CCO. It would indeed be cumbersome and over complex to define a new set of procedures to support AI/ML supported CCO.

In F1AP, the existing CCO Assistance Information IE (gNB-CU to gNB-DU) provides assistance information for legacy CCO. The names and the descriptions of the sub-IEs contained in the CCO Assistance Information IE clearly points at a detected CCO issue (the detected CCO issue is signalled via the CCO issue detection IE, the Affected Cells and Beams refers to cells and beams affected by detected CCO issue), these IEs alone cannot convey predicted/future information. We therefore propose RAN3 to discuss the following options:

Option 1: extend the CCO Assistance Information IE in the GNB-CU CONFIGURATION UPDATE message:

  - Reuse the CCO issue detection IE to indicate a predicted CCO issue. When the CCO Issue Prediction Time IE is present, the CCO issue detection IE refers to a predicted CCO issue.

  - Reuse the Affected Cells and Beams IE. When the CCO Issue Prediction Time IE is present, the Affected Cells and Beams IE refers to a predicted Affected Cells and Beams.;

  - Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

  - Add an optional IE to indicate timing information of the future coverage state at source gNB (e.g. Coverage Modification Time). The latter IE is needed to let the target gNB-DU to determine at what time the source gNB-DU will apply its future coverage state, and therefore to coordinate CCO coverage state changes at target gNB.

Option 2: add a new IE to the GNB-CU CONFIGURATION UPDATE message (e.g., Predicted CCO Assistance Information) including the information listed in Option 1.

We would prefer Option 1, as it reduces complexity and impact to the standard, while at the same time it ensures a clear and unequivocal understanding of whether the existing IE refer to an action that already occurred or to a predicted/future action.

Proposal 1: To support AI/ML based CCO in F1AP signalling from gNB-CU to gNB-DU, extend the CCO Assistance Information as follows:

  - Reuse the CCO issue detection IE to indicate a predicted CCO issue. When the CCO Issue Prediction Time IE is present, the CCO issue detection IE refers to a predicted CCO issue.

  - Reuse the Affected Cells and Beams IE. When the CCO Issue Prediction Time IE is present, the Affected Cells and Beams IE refers to a predicted Affected Cells and Beams.;

  - Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

  - Add an optional IE to indicate timing information of the future coverage state at source gNB (e.g. Coverage Modification Time)

A TP for F1AP mirroring the proposals above is presented in R3-245504.

## F1 signalling design (gNB-DU to gNB-CU)

The F1 signalling from gNB-DU to gNB-CU enables the gNB-DU to inform the gNB-CU about the future CCO state. Any CCO state (present or future) is expressed in terms of Cell Coverage State IE and the SSB Coverage State IE, already included in the Coverage Modification Notification IE. So, this information can be reused, as long as it is clear in the signalling that a certain Cell Coverage State or SSB Coverage State is valid now or in the future. The latter will be indicated by the timing information for the future CCO state.

We also note that for legacy CCO the gNB-DU sends to the gNB-CU a Coverage Modification Cause IE, to indicate which (detected) CCO issue led to the coverage modification. While in legacy CCO, the gNB-DU can monitor the radio environment and derive an issue (that can be communicated via the gNB-DU to gNB-CU Coverage Modification Cause IE, the same is not possible when the gNB-DU derives a future coverage state. This is because at the time the future coverage state is derived by the gNB-DU the issue predicted by the gNB-CU has not occurred yet. For this reason, the Coverage Modification Cause IE from the gNB-DU to the gNB-CU shall not be signalled if the gNB-DU produces a future coverage state.

Therefore, we propose to reuse and extend the Coverage Modification Notification IE in the GNB-DU CONFIGURATION UPDATE as follows:

- For the future CCO state, to indicate which coverage state(s) will be used for a cell, reuse the NR CGI and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

- For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future SSB coverage state;

- For timing information of the future coverage state, add a new optional IE (e.g., Coverage Modification Time) in the Coverage Modification Notification IE. If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells and SSB Coverage State(s) of the SSBs listed in the Coverage Modification Notification IE will be applied. If the Coverage Modification Time is absent, the coverage modification information corresponds to legacy information, namely the coverage modification is applied “now”.

- Clarify in the specifications that if the Coverage Modification Time IE is present, the Coverage Modification Cause IE shall not be present.

Proposal 2: To support AI/ML based CCO in F1AP signalling from gNB-DU to gNB-CU reuse and extend the CCO Assistance Information IE as follows:

  - For the future CCO state, to indicate which coverage state(s) will be used for a cell, reuse the NR CGI and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

  - For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future beam coverage state;

  - For timing information of the future coverage state, add a new optional IE (e.g., Coverage Modification Time) in the Coverage Modification Notification IE. If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells and SSB Coverage State(s) of the SSBs listed in the Coverage Modification Notification IE will be applied. If the Coverage Modification Time is absent, the coverage modification information corresponds to legacy information, namely the coverage modification is applied “now”.

  - Clarify in the specifications that if the Coverage Modification Time IE is present, the Coverage Modification Cause IE shall not be present.

A TP for F1AP mirroring the proposals above Option 1 is presented in the TP to F1AP in our companion contribution in R3-245504.

## Xn signalling

In XnAP, additions are needed in the signalling to exchange the future CCO state, the associated (predicted) coverage modification cause (corresponding to the predicted CCO issue), the timing information for future CCO state and the timing information for the predicted CCO issue. To support that, we propose to reuse and extend the Coverage Modification Item IE in the NG-RAN NODE CONFIGURATION UPDATE as follows:

- For the future CCO state, to indicate which coverage state(s) will be used for a cell at the source gNB, reuse the Global NG-RAN Cell Identity and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

- For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future SSB coverage state;

- For timing information of the future CCO state, add an optional IE (e.g., a Coverage Modification Time). If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells listed in the Coverage Modification Item and the SSBs Coverage State(s) of the SSB listed in the SSB Coverage Modification Item will be applied.  If the Coverage Modification Time is absent, the legacy behaviour applies, namely the coverage modification has already been applied.

- Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

- To identify which predicted CCO issue type led to the future CCO state, reuse the codepoints of the Coverage Modification Cause IE within the Coverage Modification Item IE. When the CCO Issue Prediction Time IE is present, the Coverage Modification Cause IE refers to a predicted CCO issue.

Proposal 3: To support AI/ML based CCO in XnAP signalling reuse and extend the Coverage Modification Item IE in the NG-RAN NODE CONFIGURATION UPDATE as follows:

  - For the future CCO state, to indicate which coverage state(s) will be used for a cell at the source gNB, reuse the Global NG-RAN Cell Identity and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

  - For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future SSB coverage state;

  - For timing information of the future CCO state, add an optional IE (e.g., a Coverage Modification Time). If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells listed in the Coverage Modification Item and the SSBs Coverage State(s) of the SSB listed in the SSB Coverage Modification Item will be applied.  If the Coverage Modification Time is absent, the legacy behaviour applies, namely the coverage modification has already been applied.

  - Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

  - To identify which predicted CCO issue type led to the future CCO state, reuse the codepoints of the Coverage Modification Cause IE within the Coverage Modification Item IE. When the CCO Issue Prediction Time IE is present, the Coverage Modification Cause IE refers to a predicted CCO issue.

A TP for XnAP mirroring the proposals above is presented in R3-245503.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Proposal 1: To support AI/ML based CCO in F1AP signalling from gNB-CU to gNB-DU, extend the CCO Assistance Information as follows:

  - Reuse the CCO issue detection IE to indicate a predicted CCO issue. When the CCO Issue Prediction Time IE is present, the CCO issue detection IE refers to a predicted CCO issue.

  - Reuse the Affected Cells and Beams IE. When the CCO Issue Prediction Time IE is present, the Affected Cells and Beams IE refers to a predicted Affected Cells and Beams.;

  - Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

  - Add an optional IE to indicate timing information of the future coverage state at source gNB (e.g. Coverage Modification Time)

Proposal 2: To support AI/ML based CCO in F1AP signalling from gNB-DU to gNB-CU reuse and extend the CCO Assistance Information IE as follows:

  - For the future CCO state, to indicate which coverage state(s) will be used for a cell, reuse the NR CGI and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

  - For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future beam coverage state;

  - For timing information of the future coverage state, add a new optional IE (e.g., Coverage Modification Time) in the Coverage Modification Notification IE. If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells and SSB Coverage State(s) of the SSBs listed in the Coverage Modification Notification IE will be applied. If the Coverage Modification Time is absent, the coverage modification information corresponds to legacy information, namely the coverage modification is applied “now”.

  - Clarify in the specifications that if the Coverage Modification Time IE is present, the Coverage Modification Cause IE shall not be present.

Proposal 3: To support AI/ML based CCO in XnAP signalling reuse and extend the Coverage Modification Item IE in the NG-RAN NODE CONFIGURATION UPDATE as follows:

  - For the future CCO state, to indicate which coverage state(s) will be used for a cell at the source gNB, reuse the Global NG-RAN Cell Identity and Cell Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future cell coverage state;

  - For the future CCO state, to indicate which coverage state(s) will be used for a beam, reuse the SSB Index and SSB Coverage State IEs. When the Coverage Modification Time IE is present, the information refers to a future SSB coverage state;

  - For timing information of the future CCO state, add an optional IE (e.g., a Coverage Modification Time). If the Coverage Modification Time is present, it indicates the time in the future when the Cell Coverage State(s) of the cells listed in the Coverage Modification Item and the SSBs Coverage State(s) of the SSB listed in the SSB Coverage Modification Item will be applied.  If the Coverage Modification Time is absent, the legacy behaviour applies, namely the coverage modification has already been applied.

  - Add an optional IE to indicate timing information of predicted CCO issue (e.g. CCO issue prediction time).

  - To identify which predicted CCO issue type led to the future CCO state, reuse the codepoints of the Coverage Modification Cause IE within the Coverage Modification Item IE. When the CCO Issue Prediction Time IE is present, the Coverage Modification Cause IE refers to a predicted CCO issue.

A TP for F1AP mirroring the proposals above is presented in R3-245504.

A TP for XnAP mirroring the proposals above is presented in R3-245503.