3GPP TSG-RAN WG3 Meeting #125-bis	R3-245508

Hefei, P.R.C, 14th Oct – 18th Oct, 2024

Title: 	AI/ML support for NR-DC

Source: 	Ericsson, Jio

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

At RAN3#125 the SI on AI/ML in RAN in Rel-19 was completed, and one of the conclusions captured in the TR 38.743 is to proceed with normative work for AI/ML assisted mobility optimization in NR-DC. In this contribution, we discuss aspects related to the enhancements needed in NG-RAN interfaces to support this use case.

# 2. Discussion

## Scope of normative work

According to the conclusions captured in the TR 38.473, the baseline for normative work for mobility optimization for NR-DC consists of enhancing dual connectivity procedures (e.g., SN Addition, MN-initiated SN Change) to trigger collection of measured UE performance. Also, the model inference is assumed at the MN only, and Conditional Dual Connectivity procedures are out of scope.

Proposal 1: For AI/ML assisted mobility optimization in NR-DC, confirm that: 1) model inference is only supported at the MN; 2) conditional procedures are out of scope.

## Enhancements to support mobility in NR-DC

Given the conclusions of the SI, we would like to focus the discussion on two scenarios:

- a UE is in single connectivity and the serving gNB infers the decision to initiate SN Addition

- a UE is in NR-DC and the MN infers the decision to initiate SN Change

We think the following types of enhancements are needed to support the scenarios above:

- Enhancements to assist inference at the MN (future MN or current MN, depending on the scenario)

- Enhancements to provide feedback to the MN

- Additional data collection for NR-DC

To assist inference at the MN (both if it is a “future MN” or a “actual MN”), we think it’s possible to reuse to a large extent the Rel-18 signaling design established to support AI/ML assisted use cases.

For example, reuse of the Data Collection Reporting Initiation and Data Collection Reporting procedures allows an MN (future or current) to request and obtain from a neighbor gNB the reporting of UE performance.

Proposal 2: For NR-DC reuse Data Collection Reporting Initiation procedure for requesting UE performances at the SN.

As described in the semantics description for Data Collection ID, when a Data Collection Reporting Initiation is executed, a Data Collection Reporting context is established. The definition of the Data Collection ID IE is shown below:

| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Together with NG-RAN node2 Measurement ID, identifies a Data Collection Reporting context. |
| --- | --- | --- | --- | --- |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Together with NG-RAN node1 Measurement ID, identifies a Data Collection Reporting context. |

If the Data Collection ID is added to the HANDOVER REQUEST, the target gNB understands that it should start collecting UE performances as configured via the previous Data Collection Reporting Initiation procedure (and corresponding to the Data Collection ID). For NR-DC, a similar approach can be used, both for SN Addition and MN-initiated SN Change, as we described below.

For SN Addition:

- The future MN requests data (e.g., UE performance) to a neighbour gNB (potential SN). The Data Collection Reporting context is setup, and both the future MN and the potential SN are aware of the Data Collection ID;

- The future MN infers the decision to add the neighbour gNB as SN;

- The future MN adds the Data Collection ID in the S-NODE ADDITION REQUEST, and the SN understands that it should start collecting the data previously requested by the MN (e.g., UE performance).

For MN-initiated SN Change:

- The MN requests data (e.g., UE performance) to a neighbour gNB (potential target SN). The Data Collection Reporting context is setup, and both the MN and the potential new SN are aware of the Data Collection ID;

- The MN infers the decision to change the SN;

- The MN adds the Data Collection ID in the S-NODE ADDITION REQUEST, and the new target SN understands that it should start collecting the data previously requested by the MN (e.g., UE performance).

Proposal 3: For AI/ML assisted mobility in NR-DC, add Data Collection ID in S-NODE ADDITION REQUEST to let the SN know that it should start collecting performance of the UE at the SN if requested by the MN.

Another part of the Rel-18 solution concerns the conditions that the target gNB needs to monitor to stop collection of UE performance requested by the source gNB. For NR-DC, similar conditions can be defined, so that the SN takes them into account to terminate the collection of UE performance requested by the MN. In our view, at least the following conditions should be considered:

-	a time interval, after successful SN Addition;

-	the SN is removed from NR-DC (MN initiated or SN initiated);

Proposal 4: For AI/ML assisted mobility in NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) a time interval, after successful SN Addition; 2) SN removal (MN initiated or SN initiated).

Furthermore, in Rel-18 solution, a condition is specified according to which the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST and the target gNB knows exactly for which cell it should report the UE performances. Also, since the collection should stop when UE is handed over to another cell, there is only one cell the UE performance can refer to. With some enhancements, the existing signaling design can be reused for NR-DC as we discuss below.

First, the reported UE performances should be extended to include the PSCell used by the UE, so that it is clear to the MN as to which cell the UE performances refer to. The MN can use this information, e.g., to understand whether its assumptions to select the SN were optimal or not.

Proposal 5: For AI/ML assisted mobility in NR-DC, extend the UE performance IE to include the PSCell serving the UE in the SN.

Second, to extend to NR-DC the logic used for mobility in single connectivity, we propose to specify that SN stops collection of UE performances in the cell used as PSCell when the PSCell is changed.

Proposal 6: For AI/ML assisted mobility in NR-DC, the SN stops collection of data requested by the MN at PSCell change.

Once the UE performances are collected, the SN should report them to the MN. To do that, it is straightforward to reuse the Data Collection Reporting procedure already available.

Proposal 7: For AI/ML assisted mobility in NR-DC, the Data Collection Reporting procedure is reused for reporting UE performance from SN to MN.

It is reasonable to think that the MN has inferred the action of SN Addition (or SN Change) assuming that adding (or changing) the SN would bring some benefits to the UE. We think that it is good if MN informs the SN about some of assumptions made, so that the SN can try to provide best possible service to the UE.

For example, when an MN infers the action to add or change SN, it can assume that a specific PSCell will be selected for the UE. This is obvious because the UE performance feedback received in the past for each UE is specific to a PSCell (described in Proposal 5). Hence, if UEs were served well by an SN, on one or more PSCells, the MN is more likely to predict that the SN and one or more specific PSCells, are the best options for a DC configuration at the UE.

Another example is that when the MN carries out an AI/ML generated SN addition, it may request to the SN the establishment of SN Terminated bearers. However, the inferred decision can be based on the assumption that specific QoS flows will be served by the SN (as SCG). This is not in line with what could happen today, where an SN may decide that all the QoS flows for which the MN requests SN Terminated bearers, shall be served by MCGs. Therefore, when the MN derives an AI/ML based request to the SN for some resources for SN terminated bearers, it should tell the SN that this request was made assuming that SN could serve some or all of the QoS flows as SCGs. After receiving this notification, the SN is left free to decide whether to serve the QoS flows as MCG or SCG, but it can use the information and try to accommodate the MN assumptions if possible.

Therefore, we propose that, at SN Addition, the MN could tell the SN that MN assumes that a specific cell, e.g. Cell A, is selected as PSCell. The SN is free to decide independently, but it can try to select Cell A as PSCell if possible. It is also proposed that the MN indicates to the SN which are the assumptions in terms of the QoS Flows the SN will have to serve as SCG.

Proposal 8: Enhance dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to indicate MN assumptions at the basis of the AI/ML based inference on the SN to add (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

## Additional data collection for NR-DC

To better support inference at the (future or current) MN, the Data Collection Reporting Initiation and Data Collection Reporting procedures could also be extended, e.g., to collect and report additional non-UE Associated metrics that can be used as inputs to infer the next SN addition or SN change.

An example of non-UE associated metric that can be advantageous to collect is the propagation delay over the F1 interface that affects an SN/PSCell. Such latency is often the largest delay component in the traffic propagation delay over an NR-DC leg, and it is mainly depending on the transport network performance. It shodl also be pointed out that this performance is variable, namely F1 delays vary dynamically depending on transport network conditions.

It is known that NR-DC configurations do not deliver a good performance if the traffic propagation delay between the two legs of the NR-DC configuration is unbalanced (i.e. very different delays between the MCG and SCG branches and/or very high delays on both branches). It is therefore very useful for an MN to understand what is the F1 propagation delay of the SN/PSCell to select in the future those target SN/PSCell that are affected by more favourable propagation delays.

Proposal 9: For AI/ML based Mobility Optimization in NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

# 3. Conclusions

Based on the discussion in this paper, we derived the following proposals:

Proposal 1: For AI/ML assisted mobility optimization in NR-DC, confirm that: 1) model inference is only supported at the MN; 2) conditional procedures are out of scope.

Proposal 2: For NR-DC reuse Data Collection Reporting Initiation procedure for requesting UE performances at the SN.

Proposal 3: For AI/ML assisted mobility in NR-DC, add Data Collection ID in S-NODE ADDITION REQUEST to let the SN know that it should start collecting performance of the UE at the SN if requested by the MN.

Proposal 4: For AI/ML assisted mobility in NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) a time interval, starting at SN Addition; 2) SN removal (MN initiated or SN initiated).

Proposal 5: For AI/ML assisted mobility in NR-DC, extend the UE performance IE to include the PSCell serving the UE in the SN.

Proposal 6: For AI/ML assisted mobility in NR-DC, the SN stops collection of data requested by the MN at PSCell change.

Proposal 7: For AI/ML assisted mobility in NR-DC, the Data Collection Reporting procedure is reused for reporting UE performance from SN to MN.

Proposal 8: Enhance dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to indicate MN assumptions at the basis of the AI/ML based inference on the SN to add (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

Proposal 9: For AI/ML based Mobility Optimization in NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

A TP mirroring the proposals above is presented in R3-245509