3GPP TSG-RAN WG2 #125-bis	R3-245512

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.1

Source:	Ericsson

Title:	Regarding the LS on number of UEs in SDT

Document for:	Discussion

# 1	Introduction

An incoming LS from SA5 was received in [1] which tries to get feedback for “the number of SDT UEs in RRC_INACTIVE state”. This contribution shares our thoughts on this matter. While the LS is directed to RAN2, RAN3 has had lots of discussions on statistics for SON concerning numbers of UEs in a cell in various conditions, therefore feedback from RAN3 on this matter would also be valuable for SA5

# 2	Discussion

The LS from SA5 formulates the following question:

# 1 Overall description The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission. # 2 Actions To RAN2 ACTION: SA5 kindly asks RAN2 to provide feedbacks.

It seems from this request for feedback that there is no clear question to be answered, rather SA5 asks for feedback around the stated information in the “Overall description” chapter. Let’s break down the provided information piece by piece and provide our view on each of these:

Statement 1 in LS: “SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell.”

It seems reasonable that the number of UEs in a specific cell in state RRC_INACTIVE would be useful information to the operator. However, a UE in RRC_INACTIVE mode is not known to the network by cell level. The network might have transferred a UE to RRC_INACTIVE and the UE from that moment could have moved into another cell without informing the network.

The network does not know which UEs in RRC_INACTIVE are in a specific cell.

Besides, It is not possible to know whether a UE in RRC_Inactive will or will not perform SDT, hence it is not possible to know the load due to SDT that UEs will generate.

The network does not know which UEs in RRC_INACTIVE will trigger an SDT procedure and by that it is not possible to know what load SDT may generate overall.

Statement 2 in LS: “SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.”

For a UE configured with SDT, it might be for Mobile Originated and/or Mobile Terminated, and the configuration might include RA-SDT and/or CG-SDT. With any of these configurations the UE might be actively transmitting and/or receiving data. Statement 2 seems to imply that it is possible for the network to know in advance whether a UE in RRC_Inactive will perform SDT transmission. However, this is not the case, as SDT transmissions depend conditions such as the amount of data an application needs to exchange, which are difficult to predict. The SDT transmission runs for the duration of timer T319a, which runs  at the UE-side if the SDT procedure is considered to be ongoing.

SDT is triggered by factors such as small volumes data transmissions and it runs for a a short timer, kept at the UE. It is unclear if SA5 wants to monitor such short duration SDT loads during time windows comparable to T319a

The request for evaluation in statement 2 does not consider cell level, but it can be assumed from statement 1 in the LS that the question regards cell level knowledge of a UE configured for SDT as well as transmitting data.

If this interpretation is correct, we would like to make the following remarks:

- Timer T319a runs for a short time in this perspective, maximum around 4 seconds, and it is only known at the UE side.

- These specific UEs would be known on cell level while having an ongoing transmission in either UL or DL.

The usefulness of the counters proposed could be questioned, given that for the short duration of SDT, real time measurements of the UEs using SDT may not reflect the overall number of UEs that may use SDT in the cell.

Combining statement 1 and 2 from the LS, it can be deduced that the number of UEs transmitting or receiving data in SDT mode represent the total load status for a cell. This is incorrect as such UEs only represent a potentially small fraction of UEs generating load in the cell.

There are many other metrics, such as total PRB utilisation, or PRB utilisation per S-NSSAI, which provide a much better understanding of the load in a cell, either total, or per network slice, or per service.

RAN3 to send a reply-LS to SA5; stating; “RAN3 think it would be possible for the network to specify the current number of users in SDT with active data transmission for a specific cell but see no relation to the overall cell or network load with this. Furthermore, such a number would only be stable for a short time, since the SDT procedure is meant to be very short (max 4 seconds).”

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The network does not know which UEs in RRC_INACTIVE are in a specific cell.

Observation 2	The network does not know which UEs in RRC_INACTIVE will trigger an SDT procedure and by that it is not possible to know what load SDT may generate overall.

Observation 3	SDT is triggered by factors such as small volumes data transmissions and it runs for a a short timer, kept at the UE. It is unclear if SA5 wants to monitor such short duration SDT loads during time windows comparable to T319a

Observation 4	The usefulness of the counters proposed could be questioned, given that for the short duration of SDT, real time measurements of the UEs using SDT may not reflect the overall number of UEs that may use SDT in the cell.

Observation 5	Combining statement 1 and 2 from the LS, it can be deduced that the number of UEs transmitting or receiving data in SDT mode represent the total load status for a cell. This is incorrect as such UEs only represent a potentially small fraction of UEs generating load in the cell.

Observation 6	There are many other metrics, such as total PRB utilisation, or PRB utilisation per S-NSSAI, which provide a much better understanding of the load in a cell, either total, or per network slice, or per service.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to send a reply-LS to SA5; stating; “RAN3 think it would be possible for the network to specify the current number of users in SDT with active data transmission for a specific cell but see no relation to the overall cell or network load with this. Furthermore, such a number would only be stable for a short time, since the SDT procedure is meant to be very short (max 4 seconds).”

# 4	References

- S5-245138, LS on Number of UEs in RRC_INACTIVE state with data transmission