3GPP TSG-RAN WG3 Meeting #125bis	             R3-245532

Hefei, CN, 14th – 18th Oct 2024

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on signalling procedure for topology2

Document for:	Discussion

1	Introduction

In current TR, the candidate procedure for A-IoT Inventory for Topology 2 is shown as follows:

#### 6.5.3.1 Candidate procedures for A-IoT Inventory for Topology 2 Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc. Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 (if RRC-based solution is used) Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used) Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

In this contribution, we will continue to discuss the signalling and procedure for Topology 2.

2	Discussion

2.1	Inventory and Command Procedure for Topology 2

Currently, three candidate solutions are discussed for topology 2. Since the signalling transmission for both NAS-based solution and UP-based solution are terminated between A-IoT CN and UE, these two solutions can be represented in a single message flow. Existing message flow for topology 2 in TR only includes the inventory procedure, the message flow for command procedure should be depicted in this meeting. In order to present the entire procedure more clearly, the inventory and command procedure for RRC-based solution and NAS/UP-based solution are placed as follows:

Figure 1: Message flow for AIoT Inventory and Command in Topology 2 (if RRC-based solution is used)

For RRC-based solution, the steps are described as follows:

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled gNB.

2. The AIoT-enabled gNB sends an Inventory response message to the AIoT CN.

NOTE：In step 2, the AIoT-enabled gNB may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s).

4/4b. After receiving inventory result reported from the AIoT enabled UEs, the AIoT-enabled gNB may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE: The Step 4/4b may happen in parallel with Step 3.

5. The AIoT CN performs security check to verify device ID.

6. The AIoT CN sends a Command request message to the AIoT-enabled gNB.

7. The AIoT-enabled gNB node requests the AIoT-enabled UE(s) to trigger command procedure towards the AIoT device(s).

8. After receiving command result from the AIoT enabled UEs, the AIoT-enabled gNB may send Command response towards the AIoT CN including the received command result.

From the above message flow, if the RRC-based solution is adopted, the AIoT CN can initiate a class-1 command procedure to the AIoT-enabled gNB.

Proposal 1: For the RRC-based solution, the AIoT CN can initiate a class-1 command procedure to the AIoT-enabled gNB.

In addition, when the A-IoT CN can initiates a class-1 Inventory request procedure to the AIoT-enabled gNB, AIoT-enabled gNB can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 2: For the RRC-based solution, the AIoT-enabled gNB can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Figure 2: Message flow for AIoT Inventory and Command in Topology 2 (if NAS/UP-based solution is used)

For NAS/UP-based solution, the steps are described as follows:

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled UE(s).

2. The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE：In step 2, the AIoT-enabled UE(s) may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s).

4/4b. After receiving inventory result from the AIoT device(s), the AIoT enabled UE(s) may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 3: The Step 4/4b may happen in parallel with Step 3.

5. The AIoT CN performs security check to verify device ID.

6. The AIoT CN sends a Command request message to the AIoT-enabled UE(s).

7. The AIoT-enabled UE(s) triggers command procedure towards the AIoT device(s).

8. After receiving command result from the AIoT device(s), the AIoT-enabled UE(s) may send Command response towards the AIoT CN including the received command result.

From the above message flow, if the NAS/UP-based solution is adopted, the AIoT CN can initiate a Command procedure to the AIoT-enabled UE by NAS message or PDU data transmission.

Proposal 3: For the NAS/UP-based solution, the AIoT CN can initiate a Command procedure to the AIoT-enabled UE by NAS message or PDU data transmission.

In addition, when the A-IoT CN can initiates a class-1 Inventory request procedure to the AIoT-enabled UE, AIoT-enabled UE can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 4: For the RRC-based solution, the AIoT-enabled UE can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

2.2	UE selection in Topology 2

Regard to AIoT enabled UE reader selection, both the AIoT CN or A-IoT enabled gNB can perform AIoT-enabled UE selection. If the UE selection is dominated by the AIoT CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be reached. While, if the UE selection is dominated by A-IoT enabled gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be reached directly. Note that gNB-dominated selection and CN-dominated selection are not contradictory, that is, they may exist simultaneously. The AIoT CN also maintains the mapping between AIoT-enabled gNB and AIoT-enabled UE, as well as mapping between AIoT-enabled UE and device. In addition, the authorization information is stored in the AIoT CN, so it totally knows which UE is authorized as the UE reader. However, AIoT CN does not know the UE radio condition and whether there are enough AIoT radio resources to be allocated for the selected UE, so the gNB-dominated selection is also required. Based on above analysis, AIoT CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Proposal 5: For topology 2, both the AIoT CN or A-IoT enabled gNB can perform AIoT-enabled UE selection.

2.3	Assistance Information

To avoid repetition, the assistance information sent from CN written in R3-244531 used for topology1 can also be applied in the topology2.

Proposal 6: The assistance information sent from CN used for topology1 can also be applied in the topology2.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some on the signalling and procedure for Topology 2:

Proposal 1: For the RRC-based solution, the AIoT CN can initiate a class-1 command procedure to the AIoT-enabled gNB.

Proposal 2: For the RRC-based solution, the AIoT-enabled gNB can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 3: For the NAS/UP-based solution, the AIoT CN can initiate a Command procedure to the AIoT-enabled UE by NAS message or PDU data transmission.

Proposal 4: For the RRC-based solution, the AIoT-enabled UE can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 5: For topology 2, both the AIoT CN or A-IoT enabled gNB can perform AIoT-enabled UE selection.

Proposal 6: The assistance information sent from CN used for topology1 can also be applied in the topology2.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN;