3GPP TSG RAN WG3 125-bis	R3-245539

Hefei, China, 14 - 18 October, 2024

Title: 	Discussion on Timing Aspects for AIML support in CCO

Source: 	Jio, NOKIA

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

At the most recent RAN3#125 meeting, which focused on the Study Item (SI) regarding AI/ML applications in RAN Release 19, participants discussed and summarized several key agreements concerning the proactive approach to Coverage and Capacity Optimization (CCO) as outlined in TR 38.743 [1]. In contrast to the legacy reactive methodology, which typically responds to issues only after they have emerged and begun to impact network and user equipment (UE) performance, the new approach advocates for a more forward-looking proactive strategy. This proactive framework leverages AI/ML to predict and prevent potential CCO issues before they escalate into full-blown network degradation events. By addressing problems at an early stage, this method seeks to mitigate or completely avoid disruptions to network performance that would otherwise negatively affect the user experience. Rather than waiting for a CCO issue to manifest, the proactive model emphasizes preemptive action, allowing the network to anticipate coverage or capacity challenges and implement solutions in advance.

Following the 3GPP TS 38.401[2] in sub-clause 7.9.3: “In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.” the CCO reactively detection function is located in gNB CU.

The following points have been captured as to be continued in this  meeting:

-	Whether to introduce time infor related to future CCO state?

-    Whether to introduce time infor related to predicted CCO issue?

In this Discussion contribution, we provide further discussion related to AI/ML based Coverage and Capacity Optimization (CCO) timing aspects.

# 2. Discussion

In the recent RAN3#125 meeting it has been agreed for AI/ML based CCO that the task of generating proactively predictions for potential CCO issues will be handled by gNB for non-split architecture as well by the gNB-CU in split architecture, as discussed in the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698 [3]) and the corresponding 3GPP TR 38.743 V2.0.0 (2024-08), and indicated explicitly in the step 0 as follows:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU,.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Following [3] once the gNB-CU identifies a predicted CCO issue, it relays this information to the gNB-DU (step1) with a conditional future CCO assistance state as to be further discussed as well in this meeting. Additionally, during the latest RAN3 meeting, it was confirmed that after receiving the predicted CCO issue, the gNB-DU will determine the future coverage state corresponding to that issue (step2). This determination can be made based on whether the implementation follows AI/ML-based methods or the traditional, legacy approaches. The gNB-DU’s calculation of the future coverage state will be send back to gNB CU (step3) to be shared with neighbouring NG-RAN nodes (step 4). This last step 4 exchange of information serves two key purposes:

- it assists with Mobility Robustness Optimization (MRO),

- it enables neighbouring nodes to match their coverage configurations with the new future coverage state,

as described in TS 38.300 sub-clause 15.5.5.3, “the coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells' coverage configurations.” This mechanism ensures that the coverage transitions across multiple NG-RAN nodes are smooth and coordinated, preventing degradation of performance and maintaining network robustness. In this way, neighbouring nodes can adjust their coverage to match and coordinate with the changes made in the originating node.

For both MRO and the implementation of matching coverage configurations, it is then critical for the NG-RAN gNB DU to know the exact absolute time or within a range of when to apply the concluded future CCO state and for the neighbour gNB CUs and gNB DUs to know precisely when to adjust their own coverage, to be well aligned with the received (via Xn interface) new future CCO coverage state.

## Local gNB CU predicted CCO issue timing info

Currently, in RAN3, some timing-related information could be conveyed, as discussed in the Summary of offline Discussions (SoD) for AIRAN2_CCO (R3-244698 [3]) and the corresponding 3GPP TR 38.743 V2.0.0 (2024-08), specifically in step 1 as follows:

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue along with timing info to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the predicted and future CCO issue (including predicted affected cells/beams) to neighbour gNBs together with timing information an associated coverage modification cause.

We believe that the timing information in gNB CU is critical indeed for ensuring the accuracy, relevance, and effectiveness of the gNB-CU’s CCO predictions, especially before passing them to the gNB-DU for execution, managing to capture any variations in the traffic load, user mobility, and interference conditions.

Observation 1: Considering a timing execution framework in gNB CU, before sending the predicted CCO via F1 interface to gNB DU is important to account for highly predicted traffic load and mobility conditions.

According to 3GPP TS 38.300 in sub-clause 16.20.2

“For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

gNB-CU is able to analyse the expected rate of change in the channel conditions, traffic variations, and user mobility patterns, thus enforcing the timing info to ensure that the gNB-DU processes relevant predictions based on real-time or near-future conditions.

Observation 2: The predicted CCO issue may become irrelevant or out of date if there is a significant delay between prediction and execution

Following figure 1 description, gNB CU after predicting a specific CCO coverage issue should spend some time to collect information, activate the AI/ML inference and estimate a timing info before sending the predicted CCO issue via F1 interface signalling F1AP gNB CU CONFIGURATION UPDATE to gNB DU, as described in the 3GPP TS 38.473[4] sub-clause 8.2.5.2 “The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data”. The gNB CU CONFIGURATION UPDATE message includes the legacy CCO Assistance Information IE, which already includes the CCO issue as ENUMERATED value, and a list of affected cells and beams. However there is no reference or IE for the timing info which has to be added in order to fully support this new functionality.

Observation 3: F1AP gNB CU CONFIGURATION UPDATE signalling from the gNB-CU to the gNB-DU for predicted CCO issue needs to include prediction time information along with the predicted CCO issue

Figure 1: The timing info in the gNB CU predicted CCO coverage issue framework

The information to be collected as input to the AI/ML inference for the timing info estimation would potentially include Traffic Load Projections (i.e. Predicted data traffic demand in the cell or sector), User Mobility Profiles (i.e. Information about the speed, direction and trajectory, and density of mobile users), Resource Block Scheduling Window (i.e. Time granularity in which resource allocations are made in the gNB-DU), Interference Projections (i.e. Predictions of future interference, especially from neighbouring cells or external sources) and perhaps Neighbouring Cell Synchronization (i.e. Status and potential handoff synchronization with adjacent cells that may be impacted). By considering these factors, the gNB-CU can accurately predict not only the future CCO issue status but also when it will be most effective for the gNB-DU to activate it. This leads to more precise timing, ensuring minimal disruption and maximizing network efficiency.

Proposal 1: To activate the gNB CU AI/ML inference in order to estimate the proper timing info for the resolution of the predicted CCO coverage issue by the gNB DU further actions.

In a next step (figure 2) gNB DU will consider the predicted CCO coverage issue, generate the future CCO status and update the gNB CU via F1 interface signalling F1AP gNB DU CONFIGURATION UPDATE, as described in the 3GPP TS 38.473 sub-clause 8.2.4.2 “The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data”.

All the necessary info is included within the Coverage Modification Notification IE. gNB DU then executes the future CCO status taking into consideration the timing info reference framework.

Proposal 2: For AI/ML based CCO, agree to introduce time information to predicted CCO issue between the local gNB CU and gNB DU via F1 interface, namely reply positively to the following open question:

- Whether to introduce time infor related to predicted CCO issue?

When signaling a CCO issue from gNB-CU to gNB-DU, including timing information, it is essential to clarify whether the CCO issue reported by the gNB-CU is a current (detected) issue or a future (predicted) one. To ensure this is conveyed in a straightforward manner, we propose reusing the existing F1AP message used for legacy CCO. However, to support AI/ML-based CCO, we anticipate that new Information Elements (IEs) or extensions of the current IEs will be required in the signaling mechanisms, as follows:

- In F1AP signalling messages (3GPP 38.473 gNB CU CONFIGURATION UPDATE and gNB DU CONFIGURATION UPDATE) :

  - new IE is needed to indicate a reference prediction time corresponding to a predicted CCO issue,

  - There is a need to differentiate predicted CCO issue with current (detected) CCO issue.

Proposal 3: F1AP gNB CU CONFIGURATION UPDATE to include new IEs for the predicted and detected CCO issue timing info.

Figure 2: The timing info consideration during the gNB DU future CCO state execution

## Neighbour gNB CU future CCO issue timing info

Referring to figure 3 gNB1 CU has predicted a CCO issue, estimated a timing info and informed via F1 interface the gNB DU, waiting in a next step for the DU response with the future CCO status. After getting the necessary gNB1 DU response on the future CCO state, we strongly believe that if gNB1 CU does not include the timing info along with the future CCO in the forwarded via the Xn interface NG-RAN NODE CONFIGURATION UPDATE message towards gNB2 CU, then several discrepancies could potentially occur.

Consider one typical example of gNB1 CU, which has predicted a timing info TI1, sending to the gNB2 CU the future CCO state without this timing info TI1. Then gNB2 CU upon receiving the future CCO state will activate on its own the AI/ML inference and will predict a new timing info TI2 most probably and likely different from TI1, eventually forwarding via F1 interface the future CCO state along with this TI2 to gNB2 DU. Then:

- if TI2<TI1 gNB2 DU will execute the future CCO state earlier than gNB1 DU and will most likely result in a mismatch in the network with potential light or severe degraded performance on MRO or CCO coverage recovery.

- if TI2>>TI1 gNB2 DU might execute the future CCO much later than TI1with the potential danger of different the radio conditions where the executed future CCO state might be out of date in the worst scenario or partially  valid in the best one.

Consider another example of gNB1 CU, which has predicted a timing info TI1, sending to the gNB2 CU the future CCO state without again the timing info TI1. Then gNB2 CU upon receiving the future CCO state might not activate on its own any new timing info (configuration or implementation based), thus immediately forwarding via F1 interface the received future CCO state to gNB2 DU without any further timing info reference. Then gNB2 DU will immediately execute the future CCO state.

- If the execution instance is again earlier than gNB1 DU future state execution will result in a mismatch in the network.

- On the other hand if gNB2 DU executes immediately the future CCO state much later then the radio conditions or the CCO issue might not be valid anymore.

Observation 4: For both MRO and the implementation of matching coverage configurations, it is critical for the neighbour gNB CUs to know precisely when the received future CCO status is about to be executed in the source (sending) gNB DU, in order to adjust on their side the coverage to align with the new future CCO coverage state.

To facilitate this, the NG-RAN source gNB1 CU, sharing the future CCO coverage state over the Xn interface to the neighbour gNB2 CU, should also provide the same timing info (as per the local F1 interface predicted CCO issue timing info to gNB1 DU) to the neighbours, informing the exact time when this future CCO state is scheduled to be applied, referred to as the activation time.

Figure 3: The timing info consideration in Xn interface between neighbor NG-RAN gNBs

Proposal 4: serving gNB CU to send to neighbour gNB CUs the future CCO state along with the predicted CCO issue timing info via Xn interface.

Coming back to the open questions since the last RAN3#125 meeting:

Proposal 5: For AI/ML based CCO between neighbour gNBs, agree to allow source gNB1 CU to send timing info via Xn interface along with the future CCO state to the target gNB2 CU, namely reply positively to the following open question:

- Whether to introduce time infor related to predicted CCO issue?

Consider now another example of gNB1 CU, which has predicted a timing info TI1, sending to the gNB2 CU via Xn interface the future CCO state along with the timing info TI1. If the Xn interface introduces delays then gNB2 CU will receive the future CCO state and timing info with a relative delay τ. Upon receiving the future CCO state and the timing info TI1, as expected, will forward both via F1 interface to gNB2 DU.

- If there is no other delay gNB2 DU will execute the future CCO state with a reference timing TI1 + τ.

  - If  TI1 > τ the marching of the CCO issue will be resolved as expected

  - If TT1< τ the execution of the future CCO in the gNB2 DU will be delayed with potential mismatch on the CCO coverage resolution

- If there is also another extra delay T in gNB2 DU execution instance due to other reasons (i.e. processing load, other functions in queue etc) then:

  - If TI1 > T + τ the marching of the CCO issue will be resolved as expected

  - If τ < TI1 < Τ then the execution of the future CCO in the gNB2 DU will be delayed with potential mismatch on the CCO coverage resolution

  - If TI1 < T + τ then the execution of the future CCO in the gNB2 DU will be further more delayed with most likely mismatch on the CCO coverage resolution and most definite out of date radio conditions.

Observation 5: The predicted timing info TI1 in source gNB1 CU should always be at least higher than the extra expected transmission network latency τ over Xn and the potential execution delays T in the neighbour gNB2 CU & DU in order to have a synchronized and aligned execution of the future CCO state in both source and neighbour cells.

To facilitate this, the NG-RAN source gNB1 CU, before sharing the future CCO coverage state and the timing info over the Xn interface to the neighbour gNB2 CU, should:

- execute measurements on the average Xn transmission network latency

- request a report from the neighbour gNB2 CU  on the average processing load delay.

Observation 6: The source gNB1 CU, after predicting a future CCO and before predicting the associated timing info, should be at least aware of the average transmission network latency (via Xn measurements) and the average neighbour gNB CU & DU delays (via Xn signalling reporting), in order to account for the overall latency and provide a more accurate timing info.

Proposal 6: To add as input to the source gNB1 CU AI/ML inference based timing info prediction, among other parameters, the average Xn transmission network latency τ.

Proposal 7: Serving gNB1 CU to request from neighbour gNB2 CU a report on the average processing delays before estimating the future CCO state timing info.

Proposal 8: To add to the source gNB1 CU AI/ML inference based timing info prediction, among other parameters, the average neighbour gNB1 CU processing load and/or delay T.

## Future CCO state timing info signalling aspects

A way forward to implement the source gNB CU request to the neighbour gNB CU on the average processing load/delay would be the use of the DATA COLLECTION REQUEST / RESPONSE / UPDATE XnAP class 2 procedure messages.

Proposal 9: To use the DATA COLLECTION REQUEST / RESPONSE / UPDATE XnAP class 2 procedure messages to collect the average neighbour gNB CU processing load/delay.

Proposal 10: To further discuss during the meeting on the signalling aspects over Xn Interface and the corresponding XnAP IEs of the source gNB CU’s request to the neighbour gNB CU average processing delay and the corresponding response.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: Considering a timing execution framework in gNB CU, before sending the predicted CCO via F1 interface to gNB DU is important to account for highly predicted traffic load and mobility conditions.

Observation 2: The predicted CCO issue may become irrelevant or out of date if there is a significant delay between prediction and execution

Observation 3: F1AP gNB CU CONFIGURATION UPDATE signalling from the gNB-CU to the gNB-DU for predicted CCO issue needs to include prediction time information along with the predicted CCO issue.

Proposal 1: To activate the gNB CU AI/ML inference in order to estimate the proper timing info for the resolution of the predicted CCO coverage issue by the gNB DU further actions.

Proposal 2: For AI/ML based CCO, agree to introduce time information to predicted CCO issue between the local gNB CU and gNB DU via F1 interface, namely reply positively to the following open question:

- Whether to introduce time infor related to predicted CCO issue?

Proposal 3: F1AP gNB CU CONFIGURATION UPDATE to include new IEs for the predicted and detected CCO issue timing info.

Observation 4: For both MRO and the implementation of matching coverage configurations, it is critical for the neighbour gNB CUs to know precisely when the received future CCO status is about to be executed in the source (sending) gNB DU, in order to adjust on their side the coverage to align with the new future CCO coverage state.

Proposal 4: serving gNB CU to send to neighbour gNB CUs the future CCO state along with the predicted CCO issue timing info via Xn interface.

Proposal 5: For AI/ML based CCO between neighbour gNBs, agree to allow source gNB1 CU to send timing info via Xn interface along with the future CCO state to the target gNB2 CU, namely reply positively to the following open question:

- Whether to introduce time infor related to predicted CCO issue?

Observation 5: The predicted timing info TI1 in source gNB1 CU should always be at least higher than the extra expected transmission network latency τ over Xn and the potential execution delays T in the neighbour gNB2 CU & DU in order to have a synchronized and aligned execution of the future CCO state in both source and neighbour cells.

Observation 6: The source gNB1 CU, after predicting a future CCO and before predicting the associated timing info, should be at least aware of the average transmission network latency (via Xn measurements) and the average neighbour gNB CU & DU delays (via Xn signalling reporting), in order to account for the overall latency and provide a more accurate timing info.

Proposal 6: To add as input to the source gNB1 CU AI/ML inference based timing info prediction, among other parameters, the average Xn transmission network latency τ.

Proposal 7: Serving gNB1 CU to request from neighbour gNB2 CU a report on the average processing delays before estimating the future CCO state timing info.

Proposal 8: To add to the source gNB1 CU AI/ML inference based timing info prediction, among other parameters, the average neighbour gNB1 CU processing load and/or delay T.

Proposal 9: To use the DATA COLLECTION REQUEST / RESPONSE / UPDATE XnAP class 2 procedure messages to collect the average neighbour gNB CU processing load/delay.

Proposal 10: To further discuss during the meeting on the signalling aspects over Xn Interface and the corresponding XnAP IEs of the source gNB CU’s request to the neighbour gNB CU average processing delay and the corresponding response.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)

[2] 3GPP TS 38.401

[3] R3-244698 Summary of offline Discussion (SoD) for AIRAN2_CCO

[4] 3GPP TS 38.473