3GPP TSG-RAN WG3 Meeting #125-bis	R3-245558

Hefei, China, 14 – 17 October, 2024

Agenda Item:	11.3

Source:	Nokia, Jio, Deutsche Telekom

Title:	Initial discussion on stage 3 impact for AI/ML-based Coverage and Capacity Optimization (CCO)

Document for:	Discussion

# 1	Introduction

We here discuss stage 3 impacts (XnAP, F1AP) based on proposals provided in [1] submitted to this meeting.

# 2	Discussion

## 2.1	Xn Impact

In current (Rel-17) XnAP signalling, coverage modification information for CCO is sent in the Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message.

The following is indicated as Xn impact in the TR:

Xn interface:

Enhance existing procedure to collect information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

This Coverage Modification List IE is constructed as a list of NG-RAN cells, and a simple approach would be to include a new Future Coverage Information IE within this list of cells. Based on Xn impact from the TR and assumptions relative to timing information as discussed in section 2.2 of [1], we propose the following content of such new IE:

| >>Future Coverage Information | O |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Activation Time | M |  | FFS |  |  |  |
| >>>Future Cell Coverage State | M |  | INTEGER (1..63, …) | Indicates the future coverage configuration of the concerned cell. | – |  |
| >>>Future Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the future coverage modification in NG-RAN node1. |  |  |
| >>>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>>Future SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>>Future SSB Coverage State | M |  | INTEGER (0..15, …) | Indicates the future coverage configuration of the concerned SSB beam. | – |  |

Proposal 1: RAN3 to consider a simple approach for XnAP enhancement for CCO based on the inclusion of a a new Future Coverage Information IE within the list of cells contained in the legacy Coverage Modification List IE.

## 2.2	F1 Impact

In current (Rel-17) F1AP signalling, the CCO issue information is sent from the gNB-CU to the gNB-DU in the CCO Assistance Information IE in the GNB-CU CONFIGURATION UPDATE message.

The coverage modification information for CCO is sent from the gNB-DU to the gNB-CU in the Coverage Modification Notification IE within the GNB-DU CONFIGURATION UPDATE message.

The following is indicated as F1 impact in the TR:

F1 interface:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU

-	Timing Information

Also two corresponding notes are captured, discussed in section 2.4 of [1].

CCO assistance information from gNB-DU to gNB-CU:

In legacy CCO, the CCO issue detection is a process running in the gNB-CU, monitoring real-time activity as part of SON operations per cell served by the gNB-CU. The outcome is provided in the following IE:

#### 9.3.1.211	CCO Assistance Information

The CCO Assistance Information IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |

The predicted CCO issue information as discussed in [1] could be defined along the following lines:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | M |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue detected. |
| Cells and Beams Affected By Predicted CCO issue | M |  | 9.3.1.212 |  |
| Predicted Start Time | M |  | FFS | [see discussion in section 2.3 of [1]] |
| Predicted Duration | O |  | FFS | [see discussion in section 2.3 of [1]] |
| Recommended Future Coverage Information | O |  |  |  |
| >Recommended Activation Time | O |  | FFS | [see discussion in section 2.3 of [1]] |
| >Recommended Future Cell Coverage State | M |  | INTEGER (1..63, …) | Indicates the future coverage configuration of the concerned cell. |
| >Recommended Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. |
| >>Recommended Future SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. |
| >>>Recommended Future SSB Coverage State | M |  | INTEGER (0..15, …) | Indicates the future coverage configuration of the concerned SSB beam. |

We have in the tabular above included an example of the recommended (suggested) future CCO state information, as optionally present, in line with our discussion in section 2.4 of [1]. This information could be added to the existing CCO Assistance Information IE, or be defined as a new F1AP IE.

RAN3 introduced in Rel-18 unified signalling procedures for signalling of prediction information over Xn, consisting of the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure, representing a subscription based mechanism. Introduction of similar procedures over F1 could serve the purpose of CCO predictions. Such procedures may be introduced in a bi-directional way serving the purpose of transferring data both in the direction from the gNB-CU to the gNB-DU, as well as in the opposite direction from the gNB-DU to the gNB-CU, if needed e.g. for other use cases handled in Rel-19. Another option would be to extend the CCO Assistance Information IE, and hence let the CCO prediction information be carried in the GNB-CU CONFIGURATION UPDATE message in the same way as the legacy CCO issue information. We believe that RAN3 should analyze these two main options for conveying prediction information from the gNB-CU to the gNB-DU.

Proposal 2: RAN3 to analyze the two main options for conveying prediction information from the gNB-CU to the gNB-DU, i.e. 1) use of legacy gNB-CU Configuration Update procedure or 2) subscription-based mechanism, i.e. Data Collection Reporting procedures.

As background for this analysis we would like to point out that AI/ML-based CCO requires, in order to be operational, an AI/ML model generating predicted CCO issue. This AI/ML model needs to be trained for each served cell, and, once trained, the AI/ML model will in inference phase on top of real-time measurements also require some prediction information from other AI/ML models. Such prediction information could be Predicted Radio Resource Status provided from local node and neighbour nodes, which is the only prediction information captured in the TR, but it seems reasonable to assume that the CCO inference phase would in many implementations additionally require e.g. predicted UE trajectory information.

The legacy signalling based on the GNB-CU Configuration Update procedure is typically used when a node 1 has applied configuration changes and needs to inform neighbour node (node 2) about these changes, e.g. to enable node 2 to make its own matching changes. The subscription-based mechanism based on Data Collection Reporting Initiation and Data Collection Reporting signalling procedures is typically suitable when the node 1 (in this case the gNB-DU) is a consumer of some information that it would like the node 2 (the gNB-CU) to generate.  This is the case for CCO issue prediction. We provide an example of such subscription-based mechanism in Figure 1.

Figure 1: Example of subscription-based mechanism for CCO issue prediction.

Functionally, a subscription-based solution will have some advantages compared to using the gNB-CU Configuration Update procedure, e.g. to enable the gNB-DU to inform the gNB-CU about which cells support alternative CCO states, hence avoiding potentially costly and energy consuming processing in the gNB-CU for cells for which corrective CCO action would anyway not be available in the gNB-DU and would therefore be useless. Therefore, a subscription-based option seems to be a natural choice for signalling of the predicted CCO issue and also helps to align F1 signalling with Xn signalling for most of the predicted information standardized in Rel-18.

Proposal 3: RAN3 to agree on the use of a subscription-based signalling mechanism, i.e. Data Collection Reporting procedures, for the predicted CCO issue.

For signalling of the future CCO state from the gNB-DU to the gNB-CU, we believe that the legacy signalling (Coverage Modification Notification IE) can serve as baseline, but that an activation time needs to be added as discussed in section 2.2 of [1].

The future CCO state corresponds to a configuration change in the gNB-DU, and use of the legacy gNB-DU Configuration Update procedure therefore seems a natural choice for signalling of this information over F1.

Proposal 4: RAN3 to discuss and agree on the use of the legacy gNB-DU Configuration Update procedure for signalling of the future CCO state.

Based on these proposals, the exchange of Rel-19 CCO information over F1 and Xn could look as illustrated in Figure 2.

Figure 2: Exchange of CCO information over F1 and Xn.

# 3	Conclusion

We have made the following proposals:

Proposal 1: RAN3 to consider a simple approach for XnAP enhancement for CCO based on the inclusion of a a new Future Coverage Information IE within the list of cells contained in the legacy Coverage Modification List IE.

Proposal 2: RAN3 to analyze the two main options for conveying prediction information from the gNB-CU to the gNB-DU, i.e. 1) use of legacy gNB-CU Configuration Update procedure or 2) subscription-based mechanism, i.e. Data Collection Reporting procedures.

Proposal 3: RAN3 to agree on the use of a subscription-based signalling mechanism, i.e. Data Collection Reporting procedures, for the predicted CCO issue.

Proposal 4: RAN3 to discuss and agree on the use of the legacy gNB-DU Configuration Update procedure for signalling of the future CCO state.

# References

[1]		R3-245557, Scenarios and open points for AI/ML-based Coverage and Capacity Optimization (CCO), Nokia, Jio, Telecom Italia