3GPP TSG-RAN WG3 Meeting #125bis	R3-245572

Hefei, China, October 14th – 18th, 2024

Agenda item:	14.3

Source: 	LG Electronics Inc.

Title: 	(TP for NR_NTN_Ph3 TS 38.300)

Document for:	Other

# Introduction

This contribution proposes text proposal for TS 38.300 [1] (BL CR rapporteur: Ericsson), as discussed in [2].

# Reference

[1] TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

[2] R3-245571, “Discussions on NG management and Inactive support”, LG Electronics Inc.

# TP for NR_NTN_Ph3 TS 38.300

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

In addition, the following NTN related parameters shall be provided by O&M to the gNB hosted by the regenerative NTN payload:

-	The service areas of the AMFs for NG connection management.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////