3GPP TSG-RAN WG3 Meeting #125bis	R3-245580

Hefei, China, 14th – 18th October 2024

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for TR 38.769] on AIoT Architecture aspects

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-245579.

# 2	Text Proposal for TR 38.769 v1.0.0

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.4	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for A-IoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g. in TS 22.369 [10] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

Editor’s Note 1: Figure 6.3.1-2 serves as a starting point for further discussions.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

Editor’s Note 2: the signalling transport for XXAP is FFS.

Editor’s Note 3: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the A-IoT RAN node functions are FFS.

### 6.4.2	Support of Topology 2

#### 6.4.2.0	General

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function, which is able to communicate with the A-IoT enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

#### 6.4.2.2	Protocol stack for topology 2 – RRC based solution

Figure 6.4.2.2-1 Protocol Stack for topology 2 – RRC based solution

Figure 6.4.2-2 depicts the protocol stack for the RRC based solution for topology 2, with the XXAP and RRC transporting A-IoT upper layer information by means of explicit A-IoT specific XXAP and RRC protocol means, which provides visibility of the AIoT transaction to the AIoT enabled gNB allowing to control and coordinate AIoT radio resource allocation and usage.

#### 6.4.2.3	Protocol stacks for topology 2 – NAS based solution

Figure 6.4.2.3-1. Protocol Stack for topology 2 – NAS based solution – transport of AIoT upper layer information

Figure 6.4.2.3-1 depicts the protocol stack for the NAS based solution for topology 2 concerning the transport of AIoT upper layer information:

-	AIoT UE NAS information is transported – transparently to AIoT RAN –from the AIOT CN to the UE Reader by means of N1 NAS communication. Provision of AIoT upper layer information is performed without interaction in the AIoT RAN as the AIoT enabled gNB has no visibility of the AIoT transaction performed.

Editor’s note 1: Protocol means used for AIoT control between the AIoT enabled UE and the AIoT CN (XX*) are assumed to be equivalent or a subset to XXAP protocol means assumed for the RRC based solution.

Figure 6.4.2.3-2 Protocol Stack for topology 2 – NAS based solution – XXAP part

Figure 6.4.2.3-2 depicts the protocol stack for the NAS based solution for topology 2 concerning the network entities interacting with the AIoT RAN node function residing in the AIoT RAN node:

-	In order to trigger respective AIoT radio resource control, in advance to NAS based provision of AIoT upper layer information, XXAP based signalling provides related AIoT transaction information. Respective XXAP protocol elements carry sufficient information to allow the AIoT-enabled gNB to allocate AIoT radio resources accordingly.

-	The AIoT RAN node configures the UE reader to perform the AIoT transaction on allocated AIoT radio resources.

-	NAS based provision of AIoT upper layer information and XXAP trigger control of AIoT radio resources need to be coordinated and associated at the UE reader, e.g. by means of an AIoT specific transaction ID, provided by the AIOT CN via NAS/XXAP* and XXAP.

#### 6.4.2.4	Protocol stacks for topology 2 – UP based solution

Figure 6.4.2.4-1. Protocol Stack for topology 2 – UP based solution – transport of AIoT upper layer information

Figure 6.4.2.4-1 depicts the protocol stack for the UP based solution for topology 2 concerning transport of AIoT upper layeri information:

-	AIoT control information is transported – transparently to AIoT RAN – from the AIOT CN to the UE Reader by means of a dedicated PDU Session Resource, a GTP-U tunnel via NG-U and a DRB via Uu to the UE Reader.

-	Provision of AIoT control information is performed without interaction in the AIoT RAN as the AIoT enabled gNB has no visibility of the AIoT transaction performed.

Editor’s note 1: Protocol means used for AIoT control between the AIoT enabled UE and the AIoT CN (XX*) are assumed to be equivalent or a subset to XXAP protocol means assumed for the RRC based solution.

Figure 6.4.2.4-2. Protocol Stack for topology 2 – UP based solution

Analogous description applies for Figure 6.4.2.4-2 as for Figure 6.4.2.4-2:

-	In order to trigger respective AIoT radio resource control, in advance to UP based provision of AIoT upper layer information, XXAP based signalling provides related AIoT transaction information. Respective XXAP protocol elements carry sufficient information to allow the AIoT-enabled gNB to allocate AIoT radio resources accordingly.

-	The AIoT RAN node configures the UE reader to perform the AIoT transaction on allocated AIoT radio resources.

-	UP based provision of AIoT upper layer information and XXAP trigger control of AIoT radio resources need to be coordinated and associated at the UE reader, e.g. by means of an AIoT specific transaction ID, provided by the AIOT CN via UP/XXAP* and XXAP.

### 6.4.3	Deployment scenario of AIoT RAN supporting both, topology 1 and topology 2

Figure 6.4.3-1 depicts a deployment scenario where

-	an AIoT enabled gNB serves a multitude of Common reader functions, each deployed in an AIoTenabled UE,

-	and a multitude of Common reader functions deployed as integral part of the AIoT enabled gNB.

-	The AIoT RAN node function coordinates the usage of AIoT radio resources of all Common reader instances (topology 1 and topology 2).

-	The AIoT-enabled gNB is connected to the AMF(s) serving the AIoT-enabled UEs.

-	The AIoT-enabled gNB is logically connected to the AIoT CN by means of the XX interface, supporting a common set of protocol functions.

Editor’s note 1: AIoT CN details and the relation to the AMF(s) serving AIoT enabled UEs are pending SA2.

-	supporting AIoT transactions regardless the deployed topology is performed by means of single, common XX interface instance, corresponding to common set of XXAP protocol functions.

Editor’s note 2: Aspects and feasibility of supporting topology 1 and topology 2 on a single, common XX interface instance may need further study.

Figure 6.4.3-1 Deployment scenario of AIoT RAN supporting both, topology 1 and topology 2.

### 6.4.4	Further architectural topics

#### 6.4.4.1	Reader selection for topology 1 and topology 2

AIoT internal topology of common reader functions is not exposed to the 5GC.

AIoT RAN decides which readers to use to perform an AIoT transaction, based a “Scope of the AIoT transaction” information received provided by the AIoT CN, which indicates in which geographical area AIoT RAN shall select readers in order to perform the AIoT transaction, optionally if applicable, enhanced by information with which reader(s) a device has communicated previously.

Editor’s Note 1:	The term “Scope of the AIoT transaction” needs proper definition.

NOTE:	“Scope of the AIoT transaction” could be provided as geographical coordinates, an NR cell ID (in case topology is deployed), or a configured “AIoT Transaction Area ID”. A reader may be associated with one or several “AIoT Transaction Area IDs”.

Editor’s Note 2:	Details on different options for signalling “AIoT transaction area” are FFS.

#### 6.4.4.2	Paging of UE readers in RRC_IDLE/INACTIVE

Support of topology 2 would require the ability of the system to bring these reading devices into RRC_CONNECTED from either RRC_IDLE or RRC_INACTIVE, as it cannot be assumed that such reading devices are kept in RRC_CONNECTED at any time.

Editor’s Note 1:	Different options for paging UE readers need to be studied, e.g. feasibility of individual paging and group paging.

#### 6.4.4.3	Support of DO-A communication

The following DO-A traffic characteristics are identified.

-	DO-A traffic is not triggered by the AIOT CN

-	DO-A traffic is typically required to be delivered immediately from RAN to the CN without any optimizing accumulation of DO-A traffic from other devices at AIoT RAN, resulting in a single-device transaction on XX.

-	Assuming no interaction with CN initiated AIoT procedures, providing DO-A caused AIoT application-level data from the device would require an UL/D2R communication step followed by an acknowledgment of the network in DL/R2D direction.

-	Such device triggered DO-A procedure on XX will not need to be identified by a CN allocated transaction ID, but rather with a device ID.

-	After the successful execution of such simple 2-step communication, the DO-A procedure would terminate successfully.

Editor’s Note 1:	Further details are FFS.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>