3GPP TSG RAN WG3 Meeting #125-bis	R3-245588

Hefei, China, 14 - 18 October, 2024

Agenda Item:	14.3

Source:	ETRI

Title:	Discussion on NG interface management through ISL

Document for:	Discussion

- Introduction

The Rel-19 NR NTN WID[1] includes the following objectives to support regenerative payload with full gNB on board:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In the previous meeting, RAN3 got the following progresses on NR NTN regenerative payload:

RAN3#125: NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. Continue the discussion on above open issues…

In this contribution we propose to discuss an open issue on the NG interface management through ISL.

- Discussion

  - NG interface management

3GPP Rel-19 NTN normative work includes the support of regenerative payload. The satellite with regenerative payload provides the full gNB functionality and also provides Inter-Satellite Links(ISL) between the satellites. Therefore, the UE served by a gNB on board could access the 5G Core on the ground via feeder link or ISL[2].

Observation 1: The UE served by a gNB on board could access the 5G Core on the ground via feeder link or ISL.

In the last meeting, regarding the NG interface management, RAN3 has reached an agreement such that Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Observation 2: NG Removal/Setup can be used for Regenerative Payload NG interface management.

  - NG Setup via feeder link or ISL

Due to the movement of NGSO satellites with regenerative payload, the gNB on board can visit the serving area of the AMFs. As shown in Figure 1, if the gNB on board of SAT-1 visits the serving area of AMF-1 and it has a feeder link with NTN GW-1, the gNB needs to setup TNL association with a new TNL address anchored in the NTN GW-1. Thus, after changing its TNL address, the gNB sets up TNL association and NG with AMF-1 over the feeder link.

Figure 1: NG Setup via feeder link

However, as shown in Figure 2, when the gNB on board of SAT-1 visits the serving area of AMF-1, it may have no feeder link but an ISL with SAT-2 which has a feeder link with NTN GW-1. In this case, the gNB on board of SAT-1 needs to get a new TNL address anchored in NTN GW-1 through ISL. After changing its TNL address, the gNB on board of SAT-1 sets up TNL association and NG with AMF-1 through ISL with SAT-2. SAT-2 should relay all packets for TNL association and NG setup procedure between gNB on board of SAT-1 and AMF-1 over the feeder link with NTN GW-1.

Figure 2: NG Setup through ISL

Observation 3: If the gNB on board visits the serving area of a new AMF and it has only ISL with neighbouring satellites, the gNB may need to get a (new) TNL address anchored in a NTN GW through ISL. With the (new) TNL address, the gNB sets up TNL association and NG with a new AMF through ISL, and the neighbouring satellite(s) should relay all packets for TNL association and NG setup procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 1: RAN3 to discuss how to setup TNL association and NG with a new AMF through ISL.

  - NG Removal via feeder link or ISL

Due to the movement of NGSO satellites with regenerative payload, the gNB on board can leave the serving area of the AMFs. As shown in Figure 3, if the gNB on board of SAT-1 leaves the serving area of AMF-1 and it has a feeder link with NTN GW-1, the gNB needs to remove TNL association and NG with AMF-1 over the feeder link.

Figure 3: NG Removal via feeder link

However, as shown in Figure 4, when the gNB on board of SAT-1 leaves the serving area of AMF-1, it may have no feeder link but an ISL with SAT-2 which has a feeder link with NTN GW-1. In this case, the gNB on board of SAT-1 needs to remove TNL association and NG with AMF-1 through ISL with SAT-2. SAT-2 should relay all packets for TNL association and NG removal procedure between gNB on board of SAT-1 and AMF-1 over the feeder link with NTN GW-1.

Figure 4: NG Removal through ISL

Observation 4: If the gNB on board leaves the serving area of a AMF and it has only ISL with neighbouring satellites, the gNB needs to remove TNL association and NG with a AMF through ISL, and the neighbouring satellite(s) should relay all packets for TNL association and NG removal procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 2: RAN3 to discuss how to remove TNL association and NG with a AMF through ISL.

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: The UE served by a gNB on board could access the 5G Core on the ground via feeder link or ISL.

Observation 2: NG Removal/Setup can be used for Regenerative Payload NG interface management.

Observation 3: If the gNB on board visits the serving area of a new AMF and it has only ISL with neighbouring satellites, the gNB may need to get a (new) TNL address anchored in a NTN GW through ISL. With the (new) TNL address, the gNB sets up TNL association and NG with a new AMF through ISL, and the neighbouring satellite(s) should relay all packets for TNL association and NG setup procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 1: RAN3 to discuss how to setup TNL association and NG with a new AMF through ISL.

Observation 4: If the gNB on board leaves the serving area of a AMF and it has only ISL with neighbouring satellites, the gNB needs to remove TNL association and NG with a AMF through ISL, and the neighbouring satellite(s) should relay all packets for TNL association and NG removal procedure between the gNB and the AMF through ISL and over the feeder link with the NTN GW.

Proposal 2: RAN3 to discuss how to remove TNL association and NG with a AMF through ISL.

- Reference

RP-240775	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

TR 38.821 Solutions for NR to support NTN