3GPP TSG-RAN WG3 Meeting #125bis	R3-245605

Hefei, China, 14 – 18 October, 2024

Agenda item:	11.4

Source:	Nokia, Deutsche Telekom, Jio, Telecom Italia

Title:	Discussion on Energy Cost Reporting in Split Architecture

Document for:	Discussion and Decision

# 1	Introduction

During the Rel-19 NG-RAN AI/ML study item phase, we identified different information that needs to be exchanged in E1 and F1 interface for the support of the Rel-18 use cases in split architecture. Specifically, for the AI/ML Energy Saving use case in RAN3 #124 we agreed the following:

Measured Energy Cost can be transferred from DU to CU.

In this contribution we provide our views regarding the exchange of Energy Cost reporting over F1 interface. We also provide some observations related to the Energy Cost index calculation in the gNB-DU based on the existing measurements in SA5.

# 2	Discussion

The Energy Cost measurement is an Energy Consumption index, that takes values in the range between 0 and 10000. A gNB can calculate the Energy Cost index by using an operator-defined mapping rule that maps an Energy Consumption value to an Energy Cost index. During the Rel-19 study item phase, we discussed which entities of the gNB in split architecture shall be considered for the Energy Cost measurement, namely gNB-DU, gNB-CU-CP, gNB-CU-UP. In the end, we concluded that the essential part of a node’s Energy Consumption is due to the radio and therefore we agreed to consider only the Energy Cost in the gNB-DU. Based on that, we agreed to transfer the measured Energy Cost from gNB-DU to gNB-CU.

One question that arises is how the gNB-DU can measure its Energy Consumption. In TS 28.554 [1] different Energy Consumption measurements are provided including NF Energy Consumption and Energy Consumption of a gNB. However, there is no measurement to describe the Energy Consumption of a gNB-DU. It is therefore currently unclear how a gNB-DU can map its Energy Consumption to an Energy Cost measurement.

Proposal 1: Introduce a new measurement for the Energy Consumption of a gNB-DU.

If RAN3 confirms the need to introduce a new Energy Consumption measurement of a gNB-DU, RAN3 needs to send an LS to SA5 to request SA5 to introduce a new measurement in TS 28.554. We provide a draft LS to SA5 in [2].

Proposal 2: RAN3 to agree the LS to SA5 in [2].

F1AP signalling procedures are needed to transfer the Energy Cost from a gNB-DU to gNB-CU. These procedures would need to enable a gNB-CU to configure Energy Cost measurements from its gNB-DUs and to subsequently enable also the reporting of the Energy Cost measurement from a gNB-DU to gNB-CU. This topic was already discussed during Rel-18, where we identified two possible solutions for the way forward:

- Reuse existing signalling procedures e.g., Resource Status Reporting Initiation and Resource Status Reporting procedures over F1 interface

- Introduce new procedures to align F1AP signalling procedures with the Data Collection Reporting Initiation and Data Collection Reporting procedures already defined in XnAP.

In our view, a futureproof way forward is to introduce new procedures and align F1AP with the Data Collection Reporting Initiation and Data Collection Reporting procedures defined in Xn interface. This approach would also enable to transfer other information related to AI/ML, possibly even related to new use cases, in the future.

Proposal 3: Introduce new procedures for reporting of measured Energy Cost over F1 interface.

A corresponding F1AP CR is submitted to this meeting in [3].

# 3	Conclusion

In this paper we provide the following proposals:

Proposal 1: Introduce a new measurement for the Energy Consumption of a gNB-DU.

Proposal 2: RAN3 to agree to LS to SA5 in [2].

Proposal 3: Introduce new procedures for reporting of measured Energy Cost over F1 interface.

# References

[1]		3GPP TS 28.554, “5G end to end Key Performance Indicators (KPI)” (Rel.18)

[2]	R3-245607, “[Draft] LS on New Energy Consumption Measurement for gNB-DU”, Nokia, Deutsche Telekom, Jio, Telecom Italia

[3]		R3-245606, “ Procedures over F1 for the exchange of Energy Cost index”, Nokia, Deutsche Telekom, Jio, Telecom Italia