3GPP TSG-RAN WG3 Meeting #125bis	R3-245609

Hefei, China, 14 – 18 October, 2024

Agenda item:	11.4

Source:	Nokia

Title:	Discussion on the Rel-18 UE Performance measurements

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 125 we made the following agreements:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase.

In this contribution we provide our views regarding the definitions of the introduced UE Performance measurements in Rel-18. We think that the introduced UE Performance reporting should be consistent across use cases, that is for AI/ML slicing and AI/ML Mobility Optimization for NR-DC, enabling a common solution with respect to the entity that measures the different UE Performance measurements.

# Discussion

## 2.1	UE Performance Measurements

### 2.1.1	Packet Delay

TS 28.552 specifies the delay components for both DL and UL. The Average Packet Delay in DL is the sum of D1+ D2+ D3+ D4, which are defined as follows:

  - D1 (DL delay in over-the-air interface): This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface. It is measured at the gNB-DU. It is described in TS 28.552, clause 5.1.1.1.1.

  - D2 (DL delay on gNB-DU): This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. It is described in TS 28.552, clause 5.1.3.3.3.

  - D3 (DL delay on F1-U): This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.2.

  - D4 (DL delay in CU-UP): This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.1.

Similarly, Average Packet Delay in UL is the sum of D1+ D2.1+ D2.2+ D2.3+ D2.4, which are defined as follows:

  - D1 (UL PDCP packet average delay): This is the average PDCP delay in the UL. It is defined in TS 38.314, clause 4.3.1.1 and is measured by the gNB-CU-UP. This measurement is obtained according to the definition in clause 4.3.1.1 of TS 38.314 [8], named "UL PDCP Packet Average Delay per DRB per UE".

  - D2.1 (average over-the-air interface packet delay): This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2. It is measured at the gNB-DU.

  - D2.2 (average RLC packet delay): It measures RLC packet delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.3. It is measured at the gNB-DU.

  - D2.3 (average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured by the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.2.

  - D2.4 (average PDCP re-ordering delay in the UL): It measures PDCP re-ordering delay in the UL. It is measured at the gNB-CU-UP. It is defined in TS 38.314, clause 4.2.1.2.4.

In a split-architecture, some of the above delay components are measured in gNB-DU while some others are measured in gNB-CU-UP.  For example, for DL Packet Delay, D1 and D2 are measured at the gNB-DU while D3 and D4 are measured at gNB-CU-UP. In case of  UL Packet  Delay, D2.1 and D2.2 are measured in gNB-DU while D2.3, D2.4 are measured in gNB-CU-UP. It should also be noted that D1 is measured in the UE and gNB-CU-UP also needs this measurement to calculate the total UL packet delay.

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

In RAN3 #125, companies proposed two approaches to enable gNB-CU-CP to receive the overall Average Packet Delay measurements:

- gNB-CU-UP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-UP (over F1-U) and gNB-CU-UP forwards all the delay components (also measured by itself) to gNB-CU-CP (over E1). In this case, gNB-CU-UP aggregates all the delay components and sends them to gNB-CU-CP.

- gNB-CU-CP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-CP (over F1) and gNB-CU-UP sends the delay components measured by itself to gNB-CU-CP (over E1). In this case, gNB-CU-CP needs to aggregate the received delay components.

In our view, the gNB-CU-UP centric approach is the simplest. Even though Average Packet Delay comprises different individual components, some of which are being measured in gNB-CU-UP and some in the gNB-DU, the overall delay is eventually known by the gNB-CU-UP. As part of the QoS monitoring, an NG-RAN node can send to UPF in the UL PDU SESSION INFORMATION (PDU Type 1) frame the DL Delay Result and UL Delay Result IEs. DL Delay Result IE indicates the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and in the gNB-DU) and the delay over Uu interface in milliseconds for the involved QoS flow. Similarly, the UL Delay Result IE indicates the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow. Even though the purpose of sending the UL PDU SESSION INFORMATION is to send control information elements related to the PDU session from NG-RAN to UPF, the information is available to NG-RAN.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

### 2.1.2 Average Throughput UL/DL

The total throughput of a UE in the target node after a handover can be impacted by multiple factors, including congestion, transport delay and radio-related impacts within the target node. In a split architecture, these different types of impacts are visible at different nodes, such as in the gNB-CU-UP or in the gNB-DU. Hence, a throughput measurement in the gNB-CU-UP would be able to measure throughput performance due to congestion while a throughput measurement in the gNB-DU would measure UE throughput performance due to radio or scheduling issues.

We proceed to discuss the different notions of throughput measurements available.

Average UE Throughput UL: One way to measure Average UE throughput in the UL is described in TS 28.552, clause 5.1.1.3.3 where throughput represents the average UE throughput in uplink for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average UE Throughput DL: One way to measure Average UE Throughput in the DL is through clause 5.1.1.3.1 in TS 28.552 where throughput measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average buffered UE throughput DL: An alternative way to measure throughput is through clause 5.1.1.3.7 of TS 28.552 which provides the average downlink buffered UE throughput per DRB on NRCellCU. For standalone operation, the DRBs are mapped with the same 5QI while for EN-DC with the same QCI. It measures per UE and per bearer throughput independent of traffic patterns and packet size. It supports monitoring in Dual Connectivity scenario and in NSA option3a and 3x. This measurement is measured by the gNB-CU-UP.

In case of split architecture, there are two possible options to measure throughput, namely a) by measuring throughput in the gNB-DU (RLC level throughput) or b) in the gNB-CU-UP (PDCP level throughput). Below, we list a number of observations on these different options of throughput measurements:

- Throughput measurement at the gNB-DU is not end-to-end. It only reflects the effect of the lower layers in the measured UE Performance e.g., throughput after handover. As part of the handover decision, UE measures the different neighbours radio quality and a specific target cell is selected for handover considering the radio quality and other resource utilization aspects.  Hence, measuring the RLC level throughput after the handover provides somewhat redundant information as the handover decision itself is made based on the radio quality. On the contrary, measuring the overall throughput at PDCP layer will be more useful since it reflects the overall performance which includes gNB-CU-UP aspects, e.g., related to congestion, F1-U transport issues together with radio level issues. If the throughput UE Performance is then sent to the source node, the latter will be able to identify if a handover decision that was based on radio quality only, also was good in terms of end-to-end throughput.

Using gNB-DU level measurement alone as feedback can result in incorrect evaluation at source node. This is because the PDCP level throughput may be quite different from the RLC-related measurements. For example, RLC Level UE Performance can be good if the radio conditions are good, but at the same time the PDCP Level UE Performance may be bad e.g. due to congestion in gNB-CU-UP.  In another case, RLC Level UE Performance may be poor if the radio conditions are poor but PDCP level UE Performance may be good, that is there is no congestion in gNB-CU-UP. Hence, RLC level measurement alone is not sufficient at the source node for evaluating the impact of its AI/ML based action.  RLC level measurement can be used as additional information together with the PDCP level measurement.

- In NR-DC split-bearer, RLC level measurement is per-leg and can’t be aggregated to derive per-DRB measurement. To illustrate this with an example we consider the scenario where a UE is configured with one or more NR-DC split bearers after the handover to target node. If the throughout measurements are collected at RLC level, even though it is possible to collect the individual MCG and SCG leg level performance these measurements cannot be aggregated per-DRB since there is no common RLC SDU point for MCG and SCG legs. The RLC SDU point for MCG is located in MN while for SCG in SN node. Therefore, for MN, the information on how long and exactly in which time intervals the data is transmitted or kept in the buffer for SCG is hidden. The same holds for SN for MCG leg. Therefore, as neither MN nor SN knows the transmission/buffer status of data in the other leg, it is not possible to aggregate the MCG and SCG throughput per DRB. With PDCP level measurements, it is possible to collect per-bearer (QoS Flow) in all scenarios with or without NR-DC split bearers. A question that arises is whether then it is possible to send the per-leg (MCG/SCG) measurements directly to the source node without aggregation. However, the also source node will be missing required information to interpret the received measurements similarly as the target node (MN, SN). Therefore, such approach will not solve the problem.

Hence, it seems that PDCP level measurement of Average Throughput at gNB-CU-UP is the preferred option for measuring UL/DL Average Throughput.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

However, currently there exists no measurement that measures Average UE Throughput UL at the gNB-CU-UP. We, therefore, propose to send an LS to SA5, asking SA5 the feasibility to define a new measurement based on Average UE Throughput UL measured at gNB-CU-UP.

Proposal 3: Send an LS to SA5 to ask SA5 the feasibility of a new measurement based on Average UE Throughput in the UL, measured at gNB-CU-UP.

We provide a draft LS to SA5 in [1].

### 2.1.3 	DL Packet Loss

DL Packets may not be delivered to the UE due to two reasons. One reason is due to packet losses. Those include packets that are transmitted from the sending entity, e.g., gNB-CU-UP in case of DL PDCP SDUs, but are lost on F1 (split architecture) or Xn e.g., NR-DC split bearer or are not acknowledged from the receiving entity, e.g UE. Another reason is due to packets that are being discarded. Those include packets that could not be transmitted from the sending entity e.g., gNB-CU-UP in case of DL PDCP SDUs.

Observation 2: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

For Average Packet Loss DL, the following measurements are defined in TS 28.552/ TS 38.314.

- DL F1-U Packet Loss Rate (measured at DU of the DL packet loss on the F1-U interface) defined in TS 28.552, Clause 5.1.3.1.3: Packet Loss Rate measures the fraction of the PDCP SDU packets which are not successfully received at the gNB-DU. It is measured at gNB-DU.

- Packet Uu Loss Rate in the DL per DRB per UE defined in TS 38.314, Clause 4.2.1.5.1. This measurement measures packets that are lost at Uu transmission.

However, similarly to the case of throughput measurement, Packet Uu Loss Rate measurement at RLC level cannot be aggregated in case of NR-DC split bearer. This is because when the UE has an NR-DC split-bearer, there will be two independent Packet Uu loss rates in MCG and SCG legs. These individual measurements can’t be aggregated per-DRB level.

Different approaches could be considered to address this issue:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce Packet Loss Rate measurement at PDCP level

A measurement at gNB-CU-UP is preferable because it is end-to-end. On the other hand, a measurement at gNB-DU may be useful if the preference is to measure the packet losses in lower layers only.

Proposal 4: RAN3 to discuss which approach is preferrable:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce a new Packet Loss Rate measurement at PDCP level

## 2.2	Slice level Aspects

In RAN3 #125 we took an agreement to introduce finer granularity of UE Performance to support slice UE Performance, with the exact granularity to be defined in work item phase. Different granularities of finer granularity were discussed namely per QoS flow, per number of QoS flows, per DRB, per PDU session, to name a few.

In all scenarios, a given DRB maps to a single PDU session. QoS flows carried by a given DRB will all have the same UE performance. However, any aggregate of UE performance measurements at UE level can’t be used for the slicing use case when the UE is served by more than one slice. Aggregation of UE performance measurements for multiple PDU sessions belonging to the same slice would be theoretically possible, but would require additional aggregation effort and corresponding uncertainty in terms of interpretation of the aggregated result. UE performance reporting per PDU session would therefore correspond to a suitable granularity level for the slicing use case.

Proposal 5: To support slice level UE Performance PDU session is the preferred minimum granularity to be considered for  reporting.

Also for NR-DC, slices are mapped at PDU session level, i.e., each PDU session can belong to only one network slice. This also includes split bearer. In general, this points in the direction of PDCP level as observation point for UE performance indicators as also monitoring per slice requires to provide it per bearer and not per MCG and SCG leg.

Proposal 6: RAN3 should prioritize the gNB-CU-UP as an observation point for UE performance measurements.

## 2.3 	Granularity Level for Xn Reporting

In Rel-19, UE Performance reporting needs to support both AI/ML Network Slicing and AI/ML Mobility Optimization NR-DC use cases. The present discussion has shown that aggregation at UE level could provide misleading UE performance results, and can’t be used for the slicing use case when the UE is served by more than one slice. DRB level reporting is useful for both mentioned use cases.

However, UE performance is reported over Xn in the scenario of handover, and additionally may become reported for DC scenario in Rel-19. For the handover scenario it should be considered that a target node may decide to use a different QoS flow grouping for DRBs, within each PDU session, than the grouping used by the source node. In order to cover this case, the UE performance reporting over Xn should therefore be done per group of QoS flows corresponding, each of these groups corresponding to the DRB carrying these QoS flows in the target node.

Proposal 7: Support reporting of UE Performance measurements per group of QoS flows, each of these groups corresponding to the DRB carrying these QoS flows in the target node.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

Proposal 3: Send an LS to SA5 to ask SA5 the feasibility of a new measurement based on Average UE Throughput in the UL, measured at gNB-CU-UP.

Observation 2: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

Proposal 4: RAN3 to discuss which approach is preferrable:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce a new Packet Loss Rate measurement at PDCP level

Proposal 5: To support slice level UE Performance PDU session is the preferred minimum granularity to be considered for  reporting.

Proposal 6: RAN3 should prioritize the gNB-CU-UP as an observation point for UE performance measurements.

Proposal 7: Support reporting of UE Performance measurements per group of QoS flows, each of these groups corresponding to the DRB carrying these QoS flows in the target node.

# References

[1]	R3-245610	[Draft] LS on the Feasibility of new measurement that Averages UE Throughput in the UL, Nokia