3GPP TSG-RAN WG3 Meeting #125bis	R3-245612

Hefei, China, 14 – 18 October, 2024

Agenda item:	20.2

Source:	Nokia

Title:	Support for AI/ML Positioning

Document for:	Discussion and Decision

# 1	Introduction

The WID on AI/ML for Air Interface was revised in RP-242399 in RAN 105. One of the use cases in the WID is AI/ML Positioning enhancements that is described as follows in the revised WID:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

RAN3 #125bis is the first meeting where this work starts in RAN3.

During Rel-19, RAN1 has taken several agreements, some of which have RAN3 impacts. Specifically, we list below such agreements:

RAN1 #116

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

Note: details of the report are pending further discussion.

RAN1 #116-bis

For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB.

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

- Part A:

- channel measurement

- quality indicator of channel measurement

- time stamp of channel measurement

- Part B:

- ground truth label (or its approximation)

- quality indicator of label

- time stamp of label

For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least:

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

RAN1 #118

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

RAN2 at the same time has reached some high level agreements with respect to AI/ML positioning support, namely:

RAN2 #125bis

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs.

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

In this paper we provide our views setting the scope of AI/ML positioning in RAN3 and we provide some general principles that in our view should be respected. Finally, we provide our views on the AI/ML framework for AI/ML Positioning case 3a.

# 2	Discussion

## 2.1	Background

In the RAN1 work, 2 schemes are considered, namely AI/ML direct positioning and AI/ML assisted positioning. We summarize these schemes next. Figure 1 considers AI/ML Direct positioning and Figure 2 considers AI/ML Assisted Positioning.

Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Figure 1 AI/ML Direct Positioning

Case 2a: UE-assisted/LMF-based positioning with UE-side model

Case 3a: NG-RAN node assisted positioning with gNB-side model

Figure 2 AI/ML Assisted Positioning

From the above scenarios we observe that only case 3a (AI/ML Assisted Positioning) and case 3b (AI/ML Direct Positioning) have RAN3 impacts.

Proposal 1: RAN3 to consider only case 3a (AI/ML Assisted Positioning) and case 3b (AI/ML Direct Positioning).

The two schemes have different impacts to the RAN3 discussions. Case 3b corresponds to a new positioning method at the LMF. For this case, RAN1 has taken some agreements on new UE measurements to support this case. In our view, the standard impacts to RAN3 from case 3b pertain on how to send those measurements from RAN to the core network. On the other hand, case 3a is a different case where a gNB has AI/ML functionality and can provide LMF with estimates of  intermediate features. For this case, RAN3 needs to consider in which entity of the gNB, e.g., in the gNB-DU or in the gNB-CU, the AI/ML functionality is located.

## 2.2	General Principles

Positioning framework comprises a number of procedures that are agnostic to the exact positioning method to be utilized by the LMF. In our view this is a good principle to maintain in the AI/ML positioning discussions. In that sense, the current positioning architecture and framework shall be reused.

Proposal 2: AI/ML Air Interface work re-uses existing positioning architecture and framework.

The protocol used to transfer information between an LMF and an NG-RAN node is NRPPa. In RAN2 #125bis the following was agreed:

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

In our view, this is good way forward and we, therefore, support to validate this agreement in RAN3. If during the progress of the normative work it seems necessary, RAN3 can introduce new NRPPa procedures on a per need basis.

Proposal 3: NRPPa is used as a baseline for signalling between gNB and LMF for cases 3a and 3b, while new NRPPa procedures may be introduced if deemed necessary during the Rel-19 normative phase.

We also see useful to agree some high-level principles from TR 37.817, that were considered valid during Rel-18 NG-RAN AI/ML normative phase also now for the AI/ML Air Interface positioning work.

Proposal 4: Detailed AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of RAN3 scope.

Proposal 5: An AI/ML model used in a Model Inference function has to be initially trained, validated, and tested  before deployment.

Proposal 6: User data privacy and anonymization needs to be respected during AI/ML operation.

## 2.3	AI/ML Framework

For case 3b, where AI/ML functionality lies in the LMF, the AI/ML framework is up to SA2. SA2 has been already discussing solutions where Model Training can be in LMF, NWDAF/ MTLF or OAM, pending SA5.

Observation 1: AI/ML framework for case 3b is up to SA2.

However, case 3a is different since a gNB can provide assistance information to the LMF through reporting of LoS and NLoS, as well as timing information. A natural question that arises is where Model Inference and Model Training take place in split architecture. In Rel-17, RAN3 discussed a functional framework for the Rel-18 NG-RAN AI/ML use cases, illustrated in Figure 3 and described in TR 37.817. Even though this framework was not part of the normative work, it provided guidance on the different steps of the AI/ML workflow.

Figure 3 Functional Framework for RAN intelligence.

This functional framework is general and may be considered also for the AI/ML positioning discussions for case 3a.

Proposal 7: The functional framework from TR 37.817 can be used as a guideline for the AI/ML positioning framework pertaining to case 3a.

Therefore, similarly to the Rel-18 NG-RAN AI/ML discussions we think that two possible locations for training of an AI/ML Model for Positioning case 3a is OAM and gNB itself. Naturally, a gNB is a good candidate to train an AI/ML Model for those intermediate features, since it has access to raw measurements sent by the UEs. In addition, training in the OAM could also be supported if OAM receives those measurements so that an ML Model can also be trained offline. For RAN3 purposes, Model Inference is executed in the gNB. In case of monolithic gNB architecture we propose the following:

Proposal 8: In case of monolithic architecture, consider the following scenarios for AI/ML Positioning use case 3a:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB;

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note that if we follow the existing functional framework we considered in Rel-18 NG-RAN AI/ML discussions we can also reuse the Model Deployment/Update functionality that SA5 has introduced to support the deployment of an ML Model from OAM to RAN.

In case of split architecture, we can consider the following list of deployment scenarios:

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB-CU.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB-DU.

- AI/ML Model Training is located in gNB-CU and AI/ML Model Inference is located in the gNB-DU.

In our view, option 3 has limitations, without obvious benefits, since it will increase the load over F1 interface both for transferring input data for training in the gNB-CU and also for the Model Deployment/Update over F1 interface. Therefore, in our view this option should be eliminated.

Proposal 9: AI/ML Model Training and AI/ML Model Inference within a gNB in split architecture are collocated e.g., in the gNB-DU or in gNB-CU.

We summarize some analysis on the advantages and disadvantages of options 1 and 2 in the table below:

| Case 3a (gNB-side model) | AI/ML Model Training/Inference in gNB-CU | AI/ML Model Training/Inference in gNB-DU |
| --- | --- | --- |
| Advantages | - Centralized control and coordination in the CU | - Input data is already available in DU - Minimizes data transfer - Follows legacy positioning framework |
| Disadvantages | - Input data needs to be sent to CU | - Computationally challenging for DU |

Proposal 10: RAN3 to discuss different Model Training and Model Inference location scenarios for AI/ML positioning use case 3a in case of split architecture, namely whether they are located in the gNB-CU or gNB-DU.

Another aspect that needs to be addressed is related to Model monitoring. In AI/ML assisted positioning, Model monitoring may be based on estimated intermediate parameters corresponding to model output for AI/ML assisted positioning   According to the agreement from RAN2 #125bis, RAN2 assumes that gNB or LMF could perform performance monitoring for case 3a. Along the same lines, also RAN1 #118 agreed that both options, an Option A, where an NG-RAN node performs monitoring metric calculation of its own model and an Option B, where LMF performs monitoring metric calculation for the model located in the NG-RAN node are feasible.

In TR 38.843, Model monitoring is described as a report/feedback under two possible cases, namely:

- Model Inference and Model Monitoring are at the same entity

- Model Inference and Model Monitoring are not at the same entity

Note that Model Monitoring in TR 38.843 corresponds to Model Performance monitoring in RAN3 discussions which is the process enabling the monitoring of the performance of an AI/ML model and which is initiated by Model Performance Feedback.

Proposal 11: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

For AI/ML positioning case 3a we believe that the best location for Model Performance monitoring is the LMF. In case 3a since the Model Outputs due to inference are only estimated intermediate parameters, the assessment of performance of the model would require use of the estimated intermediate parameters to perform location estimate to compare against the ground truth for the actual location. The gNB cannot perform location estimate but the LMF has access to measurements from the gNB as well as gNB assistance information through estimations of intermediate parameters. This would correspond to the case described in TR 38.843 where Model Inference and Model Performance monitoring are not in the same entity. The LMF can monitor the performance of its positioning estimates by e.g., monitoring an error based on the defined metrics and determining the error evolution.

Proposal 12: Model Performance monitoring related to the estimated intermediate parameters of the gNB such as LoS, NLoS and timing information, can be performed by the LMF.

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: RAN3 to consider only case 3a (AI/ML Assisted Positioning) and case 3b (AI/ML Direct Positioning).

Proposal 2: AI/ML Air Interface work re-uses existing positioning architecture and framework.

Proposal 3: NRPPa is used as a baseline for signalling between gNB and LMF for cases 3a and 3b, while new NRPPa procedures may be introduced if deemed necessary during the Rel-19 normative phase.

Proposal 4: Detailed AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of RAN3 scope.

Proposal 5: An AI/ML model used in a Model Inference function has to be initially trained, validated, and tested  before deployment.

Proposal 6: User data privacy and anonymization needs to be respected during AI/ML operation.

Observation 1: AI/ML framework for case 3b is up to SA2.

Proposal 7: The functional framework from TR 37.817 can be used as a guideline for the AI/ML positioning framework pertaining to case 3a.

Proposal 8: In case of monolithic architecture, consider the following scenarios for AI/ML Positioning use case 3a:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB;

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Proposal 9: AI/ML Model Training and AI/ML Model Inference within a gNB in split architecture are collocated e.g., in the gNB-DU or in gNB-CU.

Proposal 10: RAN3 to discuss different Model Training and Model Inference location scenarios for AI/ML positioning use case 3a in case of split architecture, namely whether they are located in the gNB-CU or gNB-DU.

Proposal 11: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Proposal 12: Model Performance monitoring related to the estimated intermediate parameters of the gNB such as LoS, NLoS and timing information, can be performed by the LMF.