3GPP TSG-RAN WG3 Meeting #125bis	R3-245636

Hefei, China, 14th – 18th October 2024

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:			(TP for TS 38.300) Support of 5G Femto

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on additional topological enhancements for NR was agreed at RAN #105 [1]. One of the objectives in this WID is to specify overall architecture and access control for NR Femto is necessary as follows:

… The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed. …

In this contribution, we will discuss the impacts on RAN architecture aspects for NR Femto including the access control, and provide our view on it.

2.	Discussion

In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, it is also agreed to support the optional deployment of a NR Femto GW (i.e., Option 2) in a normative phase. From stage 2 point of view, the specification impacts to support Option 2 are already known from 4G. In other words, the current texts about HeNB GW in TS 36.300 can be used as a baseline for NR Femto GW.

Proposal 1: It is proposed to reuse the current texts about HeNB GW in TS 36.300 as a baseline for NR Femto GW in Option 2.

During the study on NR Femto [2], it is concluded that the direct connection (i.e., Option 1) of an NR Femto node to the 5GC via the NG interface, is already possible. Our understanding is that the description about NR Femto GW can cover Option 1. Hence, there is no critical specification impact to support Option 1.

Observation 1: Option 1 has no critical specification impact.

According to Clause 4.6 in TS 36.300, the HeNB architecture considering HeNB GW is captured. Regarding the NR Femto architecture, we think that section 5.2.1.2 of TR 38.799 can be captured into TS 38.300. One important thing is that in 4G, the S1-U interface from the HeNB may be terminated at the HeNB GW, or a direct logical U-Plane connection between HeNB and S-GW may be used. However, during the study, the NR Femto GW is only used as a concentrator for the NG-C interface. There is no agreement on that NR Femto GW can serve as the concentration of UP connection.

Proposal 2: The NR Femto should connect directly to the UPF without the help of NR Femto GW.

Also, during the study, it is concluded that the open, hybrid and closed access mode can be supported for NG Femto, thus implying that the Xn-based handover involving NR Femto node is allowed as follows:

| Source | Target | Notes |
| --- | --- | --- |
| gNB or any NR Femto node | open access NR Femto node |  |
| gNB, or any NR Femto node | hybrid access NR Femto node |  |
| hybrid access NR Femto node or closed access NR Femto node | closed access NR Femto node | Only applies for same CAG ID and PLMN, and if the UE is a member of the CAG cell. |
| Any NR Femto node | gNB |  |

Since RAN3 agrees to only support direct connection of an NR Femto node to other NR Femto nodes / gNBs via the Xn interface, there is no need to capture the corresponding description on Xn GW (which may be evolved from X2 GW).

Regarding the functional split, there are some stage 3 related descriptions in TS 36.300. We think that these texts can be captured later after the Stage 3 discussion on NR Femto.

According to the objectives of NR Femto WID, the stage 2 descriptions on access control for NR Femto should be captured. For now, we think that current descriptions on access control for PNI-NPN in Clause 16.7.4 of TS 38.300 can be reused without any modifications. Therefore, in order to avoid unnecessary duplication, it is better to just add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300. In order to show that three access modes (i.e., open, hybrid, closed) equivalent to those in 4G can be also supported by existing CAG mechanism, we propose to capture the corresponding texts in Clause 5.3 of TR 38.799 into stage 2 specification.

Proposal 3: For access control, it is proposed to add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300.

Proposal 4: For access control, it is proposed to capture the corresponding descriptions on three access modes (i.e., open, hybrid, closed) in TR 38.799 into stage 2 spec.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 5: It is proposed to agree the corresponding TPs in Appendix.

3.	Conclusion

In this contribution, we focused on the impacts on RAN architecture aspects for NR Femto including the access control, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to reuse the current texts about HeNB GW in TS 36.300 as a baseline for NR Femto GW in Option 2.

Proposal 2: The NR Femto should connect directly to the UPF without the help of NR Femto GW.

Proposal 3: For access control, it is proposed to add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300.

Proposal 4: For access control, it is proposed to capture the corresponding descriptions on three access modes (i.e., open, hybrid, closed) in TR 38.799 into stage 2 spec.

Proposal 5: It is proposed to agree the corresponding TPs in Appendix.

4.	Reference

- RP-242395, “New WID on additional topological enhancements for NR,” NTT DOCOMO, INC., AT&T, September, 2024.

- TR 38.799, “Study on additional topological enhancements for NR (Release 19),” v19.0.0.

5.	Appendix: TP for TS 38.300

This appendix provides the Text proposal for TS 38.300 based on the proposals of this contribution.

----------------Start of the First Change---------------

## 16.xx	NR Femto

### 16.xx.1	General

This clause describes the functionalities to provide NR access at home or at enterprise premises as specified in TS 23.501 [3].

### 16.xx.2	Architecture

Figure 16.xx.2-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 16.xx.2-1: NR Femto Architecture

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. A direct logical U-Plane connection between NR Femto node and UPF is used.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto node shall be the same as those supported by an gNB (with possible exceptions e.g. NNSF) and the procedures run between a NR Femto node and the 5GC shall be the same as those between an gNB and the 5GC.

Xn-based HO involving NR Femto nodes is allowed as shown in Table 16.xx.2-1.

Table 16.xx.2-1: Xn-based HO support

| Source | Target | Notes |
| --- | --- | --- |
| gNB or any NR Femto node | open access NR Femto node |  |
| gNB, or any NR Femto node | hybrid access NR Femto node |  |
| hybrid access NR Femto node or closed access NR Femto node | closed access NR Femto node | Only applies for same CAG ID and PLMN, and if the UE is a member of the CAG cell. |
| Any NR Femto node | gNB |  |

This version of the specification supports Xn-connectivity between NR Femto node and gNB or between NR Femto nodes, independent of whether any of the involved NR Femto nodes is connected to a NR Femto GW.

### 16.xx.3	Functional Split

A NR Femto node hosts the same functions as an gNB as described in clause 16.xx.2, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	A NR Femto node shall only connect to a single NR Femto GW at one time, namely no NG Flex function shall be used at the NR Femto node:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

-	The TAC and PLMN ID used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE registration is hosted by the NR Femto GW instead of the NR Femto node.

-	NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

The NR Femto GW hosts the following functions:

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femto nodes and the NR Femto GW and between the NR Femto GW and the AMF.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

A list of CAG IDs may be included in the PAGING message. If included, the NR Femto GW may use the list of CAG IDs for paging optimisation.

In addition to functions specified in clause 16.xx.2, the AMF hosts the following functions:

-	Access control for UEs that are members of Closed Access Groups (CAG):

-	CAG information signalling to the NG-RAN in case of registration/handover and in case of the change of CAG information.

-	Supervising the NG-RAN action after the change in the CAG information of a UE.

### 16.xx.4	Access control

Existing access control mechanisms (e.g. CAG control) as described in clause 16.7.4 can be used to manage the UE(s)'s access to the NR Femto node. The open, hybrid and closed access mode can be also supported for the access control of the UEs served by NR Femto node as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList IE in the SIB1, but without the cellReservedForOtherUse IE. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

-----------------End of the First Change---------------