3GPP TSG-RAN WG3 Meeting #R3-125-bis		        R3-245648

Hefei, CH, Oct 14-18, 2024

Agenda Item:	11.2

Source: 	Rakuten

Title:  	Mobility Optimization with AI/ML based Network Slicing

Document for:	Discussion and decision

# Introduction

Following are the agreements from RAN3#123-bis meeting:

AI/ML-based network slicing model in NG-RAN node can generate following information as output:

-	Resource management within RRM policy (Internal output)

-	Slice aware mobility decisions (Internal output)

Following are the notes and agreements from RAN3#124 meeting:

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

In Rel-19, predicted information from CN to NG-RAN is not considered.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

Following are the notes and agreements from RAN3#125 meeting:

In Rel-19, there is no consensus to exchange measured or predicted UE Traffic between neighbours.

Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase.

# Discussion

AI/ML-based network slicing model in a gNB can generate the following information as output [1]:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)

-	Slice aware mobility decisions (used by gNB internally)

We propose that the above information is generated from the AI/ML training entity along with timing information so that the validity duration of the prediction is known at the gNB level.

Proposal 1: Predicted radio resource status per slice and Predicted slice available capacity be provided together with timing information

We discuss a solution for addressing the requirements for advanced mobility mechanisms like CHO and LTM with AIML-based network slicing.

Layer1/Layer 2 triggered mobility (LTM) was introduced in Rel-18 and can offer improvements in handover latency and interruption time compared to Layer 3 based mobility. LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signaled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB has previously prepared and configured to the UE through RRC signaling. Then the UE switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce the mobility latency.

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE is configured with one or more CHO candidate cells and starts evaluating the execution condition(s) upon receiving the CHO configuration and stops evaluating the execution condition(s) once a handover is executed.

The basic principle behind mobility solutions like CHO and LTM are

- One or more candidate cells are prepared in advance and the candidate cells configuration is sent to the UE by the source gNB.

- The HO is executed based on UE-based approach (execution criteria like in CHO and C-LTM) or network-based approach (LTM cell switch command sent by gNB-DU to the UE).

- There could be an unpredictable “lead time” i.e. the duration between candidate cell preparation and HO execution.

Given the above background information, it is clear that there is a need to special handling of advanced mobility solutions like LTM and CHO in co-existence with AI/ML based Network slicing. We discuss two methods below for handling such mechanisms.

Method 1: Exchange of measured/predicted resource status between source and candidate cells

Based on the predicted radio resource status per slice output from the AIML entity, each gNB is aware of the predicted radio resource availability per slice. Any two NG-RAN nodes which have a source cell<->candidate cell relationship exchange this information (at the per slice per cell level) received from the AI/ML entity on a periodic basis over the Xn interface. This exchange will indicate to the source gNB a measure of predicted resource availability at the candidate cells. From 38.473 and 38.423 (F1 and Xn Specs), the “RESOURCE STATUS UPDATE” procedure can be configured for periodic information reporting between CU-DU over F1 OR any two neighbouring NG-RAN nodes over Xn.

Method 2: Exchange of absorption capacity per slice per cell between source and candidate cells

We define

- Measured throughput absorption capacity as a measure of the average bitrate capacity that the NG-RAN node may admit and serve at the time of reporting.

- Predicted throughput absorption capacity as a measure of the average bitrate capacity that the NG-RAN node may be able to admit and serve at a given future time instance.

The following information could be included in both F1 (Intra gNB-CU CHO and (C-) LTM) and Xn (Inter-gNB-CU CHO and (C-)LTM):

- Measured throughput absorption capacity

  - UL and DL GBR Measured throughput absorption capacity per slice per cel

  - UL and DL non-GBR Measured throughput absorption capacity per slice per cell

- Predicted throughput absorption capacity

  - UL and DL GBR Predicted throughput absorption capacity per slice per cell

  - UL and DL non-GBR Predicted throughput absorption capacity per slice per cell

This can be used to prevent the source gNB from sending HO requests to target gNBs/gNB-DUs which cannot admit a UE and all of its PDU sessions and DRBs. Use of this method enhances the existing mechanism by reducing potential signalling overhead. The measured/predicted UE throughput per slice info can also be used to reject a request from the target node after the HO request has been sent. The above proposed methods are applicable for the legacy L3-based mobility and also for other mobility solutions like DAPS.

Proposal 2: RAN3 discusses method 1 and method 2 and agrees a way forward for handling the advanced mobility solutions like CHO and LTM with AIML based Network Slicing.

# Conclusion

We have the following proposals:

Proposal 1: Predicted radio resource status per slice and Predicted slice available capacity be provided together with timing information

Proposal 2: RAN3 discusses method 1 and method 2 and agrees a way forward for handling the advanced mobility solutions like CHO and LTM with AIML based Network Slicing.

# Reference

[1]. TR38.743 Study on enhancement of AI/ML for NG-RAN

[2] TS38.473

[3] TS38.423