3GPP TSG-RAN WG3 #125-bis	R3-245654

# Hefei, China, 14th – 18th October 2024

Agenda Item:	10.3.1

Source:	Ericsson

Title: 	SON/MDT enhancements for NTN

Document for:	Discussion

# 1	Introduction

RAN plenary decided during RAN#103 to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON enhancements for Non-terrestrial networks.

During previous meeting, the following agreements were taken:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

Additionally, the following ways forward were also discussed:

FFS on whether it matters if RLF happens within the duration or after the duration period.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when RLF CHO is executed

Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

This paper builds on the previous submissions by discussing SON for NTN on a range of different topics such as RLF enhancements, SHR enhancements and MDT area scope related enhancements.

# 2	Discussion

### 2.1 MDT related enhancements for NTN

Using the existing MDT framework in an NTN network requires further considerations and enhancements to cope with the specificities of NTN. One of the key differences between the NTN and the TN networks is the fact that, compared to the TN, in NTN the mapping between the coverage of an NTN cell and the geographical area covered by that cell changes over the time. Indeed, as a satellite follows its orbit the coverage of the satellite cells swipes the surface of the earth meaning that, at two different points in time the same geographical area may be covered by two different NTN cells.  In this context it may be difficult to define an Area Scope for MDT that enables measuring the NTN service coverage in a given geographical area. It is therefore proposed to extend the definition of the MDT Area Scope to enable collection of immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

In the previous meetings, some companies proposed that we should add mapped cell ID to the Area Scope of MDT to support NTN networks. This is not very optimal as the mapped cell ID still translates to large geographical areas. On the other hand, enabling collection of MDT in smaller geographical areas and/or enabling finer resolution when defining the geographical areas is more useful as described below.

Proposal 2: Don’t include mapped cell ID for logged MDT in Area Scope of MDT.

Instead of mapped cell IDs in the Area Scope, it is better to include geographical areas with smaller granularity (e.g. GNSS coordinates). These will allow an operator to collect MDT more precisely, for example around a lake, mountain, instead of the whole NTN cell which are big. The exact structure and formulation of these areas of smaller granularity can be discussed in the upcoming meetings.

Proposal 3: Support geographical areas with smaller granularity in the Area Scope of MDT for both logged and immediate MDT.

The Area Scope of MDT needs specific enhancements related to indicate to the UE, that the MDT has to be collected in NTN networks. To do so, we need to add an NTN wide indicator in the Area Scope of MDT. This will allow the network to indicate to the UE to collect MDT only in NTN. There is currently no method in the specifications to indicate that MDT must be collected in a specific type of network, NTN in this case. This will allow an operator to build a coverage map of NTN network and detect problems specific to that type of RAT.

Other options such as indicating the CGI of the cell can be used, however they have an upper limit on the number of identifiers and as such is unsuitable for this scenario.

Proposal 4: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Another aspect of NTN that requires further consideration with respect to MDT is the reporting of UE MDT measurements. Indeed, compared to TN, reporting of UE measurements over the NTN uplink comes with a much higher energy cost on the UE side. This cost is the natural consequence of the fact that for the NTN uplink the distance between the transmitter (the UE on the surface of the earth) and the receiver (the satellite high in orbit) is an order of magnitude higher compared to the distance between the transmitter and the receiver in case of TN uplink (e,g.,  hundreds/thousands of km for NTN vs hundreds/ thousands of meters for TN). In this context and whenever possible, it is preferable to avoid wasting UE’s battery on sending the MDT reports while the UE is under the service coverage of a NTN and to differ these reports for the times when the UE moves back under the coverage of a TN.

Proposal 5: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

### 2.2 RLF related enhancements for NTN

Current RLF framework has been standardized with terrestrial networks in mind. When a UE declares RLF when in an NTN cell, there are currently no mechanisms to log NTN specific information in the RLF report. Thus, the network, upon receiving this RLF report, cannot fix any specific issues that a NTN network might have. Certain addition of NTN related information in the RLF report, for e.g. ephemeris information, NTN related causes such as GNSS failures, can help the network determine the RLF cause. This is critical to ensure a better non-terrestrial network functioning.

Some might argue that RLF falls more under RAN2 work, however it is good to keep in mind that RAN2 has very little time to discuss SON related topics during rel-19. Additionally, RAN3 is the leading group for the WI so we think it is prudent to start discussion early and report our outcomes to RAN2.

Observation 1: RAN2 has limited time to discuss SON related topics in rel-19 and RAN3 is the leading group for the WI.

Proposal 6: RAN3 to agree on what additions are needed in the RLF Report and to send an LS to RAN2 for final decision.

One of these additions would be related to the satellite switch with re-sync procedure, as defined in rel-18. It would be beneficial for the network to understand that this procedure was performed and failed. This could be done by e.g. reusing the “hof” codepoint as connection failure type in the RLF Report, and by adding a new indication (explicit or implicit) that the mobility procedure as failed. A new a new value for the lastHO-Type field (e.g. satSwitchWithResync) could also be added.

Proposal 7: RAN3 to suggest to RAN2 to enhance the RLF Report with indication that a satellite switch with re-sync procedure was performed and failed.

During the previous meeting, there were discussions on also addition of absolute time when the RLF occurred into the RLF report. Currently the RLF report already supports a time variable that is the TimeSinceFailure-r16 that records the time since failure occurred. The value of this variable goes up to 172800s i.e. 48 hours. One issue with reusing this variable for NTN especially for too late CHO execution, is that the time window is in ms whereas the granularity of TimeSinceFailure is in seconds. Therefore, it is beneficial to have the absolute time in the RLF report.

If the above seems too stringent, it would also be possible to have a relative time instead, which would be the time between T1 and the failure (could be negative or positive).

Proposal 8: RAN3 to suggest to RAN2 to include absolute time when the RLF occurs in the RLF report, or relative time between failure and T1.

## 2.3 RACH-less HO related enhancements for NTN

During the previous meeting, there was an FFS agreed related to the enhancements for RACH-less HO. This should be agreed as explained below.

“Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.”

RACH-less handover was standardized in rel-18 for Non-terrestrial networks that was further applicable to other Wis such as mobile IAB among others. RACH-less HO for non-terrestrial networks needs to be optimized as SON enhancements for RACH-less HO were not considered previously. RACH-less HO optimizations are also being discussed in other Work Items, therefore we propose the following.

Proposal 9: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 10: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

One of the differences between NTN and LTM RACH-less access is the fallback mechanism defined for NTN. This can be described as followed:

- UE is configured with RACH-less, including a TA value, and an RSRP threshold;

- At the time of HO, if RSRP of the target cell is lower than the configured threshold, UE will perform Random-Access procedure

It is useful for the network (i.e. source node) to understand that RACH-less was not performed while configured, in order to optimize HO failure, time, resources, etc…

Observation 2: In case of mobility failure or near-failure, it is beneficial for the source node to understand that RACH-less was configured but that fallback to Random-Access procedure was performed instead.

For the RLF Report, and for the failure case, the DL RSRP included in RA_info could be used. In that case, the dlPathlossRSRP IE (used for 2-step RA) should also cover NTN, and only be included in case of fallback. This would only need very small changes in RRC specification.

Proposal 11: For NTN mobility failures, dlPathlossRSRP IE within the RA_Info is included into the RLF Report only in case of RACH-less to Random-Access procedure fallback.

For the SHR, and for the near failure case, it can be discussed if a new SHR trigger is needed, or if T304 trigger is reused. But in any case, the SHR needs to be forwarded to the source node for analysis.

Proposal 12: For NTN mobility near-failures, SHR needs to be forwarded to the source node.

Looking at the fallback mechanism in more details, when uplink grant for RACH-less handover is configured, the UE will initiate RACH-less HO or Random-Access procedure for HO by checking if there is SSB corresponding to the configured uplink grant with SS-RSRP above a RSRP threshold is available. Thus, the information of the measured RSRP for the SSB beam(s) for the configured grant can be used for the NW to optimize the configured uplink grant for RACH-less handover configuration. It is therefore proposed to include these information in SON reports sent to the network.

Proposal 13:  UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

# 3	Summary

In this discussion paper, NTN SON/MDT enhancements were discussed and the following observations and proposals were made:

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 2: Don’t include mapped cell ID for logged MDT in Area Scope of MDT.

Proposal 3: Support geographical areas with smaller granularity in the Area Scope of MDT for both logged and immediate MDT.

Proposal 4: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Proposal 5: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

Observation 1: RAN2 has limited time to discuss SON related topics in rel-19 and RAN3 is the leading group for the WI.

Proposal 6: RAN3 to agree on what additions are needed in the RLF Report and to send an LS to RAN2 for final decision.

Proposal 7: RAN3 to suggest to RAN2 to enhance the RLF Report with indication that a satellite switch with re-sync procedure was performed and failed.

Proposal 8: RAN3 to suggest to RAN2 to include absolute time when the RLF occurs in the RLF report, or relative time between failure and T1.

Proposal 9: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 10: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

Observation 2: In case of mobility failure or near-failure, it is beneficial for the source node to understand that RACH-less was configured but that fallback to Random-Access procedure was performed instead.

Proposal 11: For NTN mobility failures, dlPathlossRSRP IE within the RA_Info is included into the RLF Report only in case of RACH-less to Random-Access procedure fallback.

Proposal 12: For NTN mobility near-failures, SHR needs to be forwarded to the source node.

Proposal 13:  UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

# 4	References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.