3GPP TSG-RAN WG3 Meeting #125-bis				                         R3-245668

Hefei, China, 14th - 18th Oct 2024

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		Further discussion on on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

- This paper provides further discussion on on-demand SIB1.

2	Discussion

- At the RAN#105 meeting, it was agreed to consider only Case 2 in Rel-19, and the revised objectives are shown below:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Observation: Only Case 2 needs to be supported in Rel-19.

At RAN3#125 meeting, the following agreement is reached for case 2.

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Based on the above agreement, we provide further proposals:

Proposal 1: Reuse the NG-RAN node configuration update procedure to send UL WUS configuration from the NES cell to Cell A over Xn.

Proposal 2: Reuse DU configuration update procedure to send UL WUS configuration from NES DU to CU.

For the gNB serving Cell A, after receives UL WUS configuration from other NES gNBs, its CU shall transmit these configuration to its DU. We suggest to reuse CU configuration update procedure to send the UL WUS configuration from CU to DU for broadcasting.

Proposal 3: Reuse CU configuration update procedure to send the UL WUS configuration from CU to DU for broadcasting.

There are following issues is not clear now.

Issue 1: what strategy is used to determine the list of CELL A?

For the gNB serving NES cells, It is not clear what strategy is used to determine the list of CELL A to assist with broadcasting UL WUS configurations for the NES cell. Based on OAM pre-configuration? or the capability exchange between neighbour cells?

We think that a static OAM pre-configuration based on network deployment could be one approach. However, considering that many legacy gNBs may not support this feature, and other gNBs might also be unable to support it due to implementation. From our perspective, we prefer to exchange the UL WUS broadcasting capability between neighbour cells.

Proposal 4: Exchange the UL WUS broadcasting capability between neighbour cells.

Issue 2: which node decide to activate/deactivate on-demand SIB1?

It is unclear whether the CU or DU makes the decision on the activation or deactivation of the OD-SIB1 for NES cells. We believe that the CU, having knowledge of the provisioning capabilities of neighboring gNBs (cells), for example, through OAM pre-configuration or a dynamic capability exchange procedure, as well as an overview of the coverage, load, and NES state of neighboring cells, is better suited to make this decision. In addition, the CU is also responsible for the RRM function, and the on-demand SIB1 can block access for legacy UEs. From RRM perspective, the CU is also better suited to make the decision.

However, there are two approaches: one is for the CU to indicate the OD-SIB1 (de)activation state of the cell to the DU, and the other is for the CU to provide the allowed cells to the DU, then the DU finally determines the OD-SIB1 (de)activation state of the cell. From our perspective, we prefer the later approach.

Proposal 5: CU provides the allowed cells for OD-SIB1 to the DU, then the DU finally determines the OD-SIB1 (de)activation state of the cell.

Issue 3: whether to support the dynamic OD-SIB1 activation/deactivation?

In the discussion of the last meeting, upon the reception of the UL WUS configuration, it is FFS if Cell A broadcasts it right away or not. We believe this depends on whether a dynamic OD-SIB1 activation/deactivation strategy or a static strategy is supported.

The static strategy assumes that the OD-SIB1 state of the NES remains relatively unchanged over the long term, meaning that sending/updating (or releasing) the UL WUS configuration from the NES cell implicitly indicates the OD-SIB1 activation/deactivation state.

In contrast, a dynamic strategy implies that even if Cell A receives the UL WUS configuration, it does not need to always broadcast it. The NES cell will dynamically change its OD-SIB1 activation/deactivation state and explicitly indicate to Cell A whether to broadcast or stop broadcasting the UL WUS configuration. From our perspective, we prefer to support dynamic OD-SIB1 activation/deactivation indication through the new procedure like UL WUS provision request/cancel.

Proposal 6: Introduce a new UL WUS provision request/cancel procedure to support dynamic OD-SIB1 activation/deactivation of NES cell.

3 Conclusion

Based on the above discussion, we provide the following observation and proposals.

Observation: Only Case 2 needs to be supported in Rel-19.

Proposal 1: Reuse the NG-RAN node configuration update procedure to send UL WUS configuration from the NES cell to Cell A over Xn.

Proposal 2: Reuse DU configuration update procedure to send UL WUS configuration from NES DU to CU.

Proposal 3: Reuse CU configuration update procedure to send the UL WUS configuration from CU to DU for broadcasting.

Proposal 4: Exchange the UL WUS broadcasting capability between neighbour cells.

Proposal 5: CU provides the allowed cells for OD-SIB1 to the DU, then the DU finally determines the OD-SIB1 (de)activation state of the cell.

Proposal 6: Introduce a new UL WUS provision request/cancel procedure to support dynamic OD-SIB1 activation/deactivation of NES cell.