3GPP TSG-RAN WG3 Meeting #125-bis	R3-245676

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Response to R3-245201

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the previous three RAN3 meetings (RAN3 #123-bis, RAN3 #124, RAN3 #125), we proposed a solution for Rel-19 Inter-CU LTM [1], [2], [3], in which the RRC and PDCP anchor do not change with UE mobility, i.e., the source gNB-CU continues to serve as an anchor as the UE moves across more than one gNB coverage. In our contributions [1], [2], [3], we explained the motivation for considering this solution outlining its many benefits. We continued the discussion in this meeting with three contributions [4], [5], [6], discussing in some detail various technical aspects of the solution, some of the main procedures in LTM preparation and LTM execution, and an impact analysis of the solution where the anchor gNB changes as the UE moves out of coverage of a gNB.

In this contribution, we provide clarifications and responses to questions raised by other companies during offline discussions and in the contribution [7] that was submitted in this meeting.

# 2 Alignment with the Rel-19 WID

As detailed in [4-6], inter-gNB/inter-CU LTM with no switch of RRC/PDCP anchor does not require support of additional functionality by the UE beyond Rel-18 LTM, which is not true for inter-gNB LTM with switch of RRC/PDCP anchor.

Observation 1: Rel-18 LTM features are sufficient for the UE to perform inter-gNB LTM without switching the PDCP/RRC anchor. This is not the case if the PDCP/RRC anchor needs to be switched (i.e., in case of Anchor switching based solution).

As [6] explains, inter-gNB LTM with no switch of RRC/PDCP anchor is transparent to the gNB-DU serving the UE. This implies all R18 Intra-CU LTM procedures can be reused as is including LTM candidate cell preparation, Early TA RACH configuration, TA Information Transfer, LTM CSI resource configuration, LTM Cell Switch Notification, etc.

Observation 2: Inter-gNB LTM without switching the PDCP/RRC anchor is transparent to the gNB-DU serving the UE. All Rel-18 F1 LTM procedures can be reused.

R19 Inter-CU LTM objective indicates that subsequent LTM mobility has to aim to avoid RRC Reconfiguration during cell switching procedure and PDCP/RRC switching based solution requires RRC Reconfiguration during subsequent Inter-CU LTM cell switch and is not aligned with WID objective.

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

Furthermore, [4] shows how subsequent LTM is supported in a similar manner to Rel-18. On the other hand, subsequent inter-gNB LTM with switch of RRC/PDCP anchor introduces at least three extra complexities compared to subsequent inter-gNB LTM with fixed PDCP anchor:

- Complexity 1: security needs to be handled and prepared among the gNBs and the CN throughout the subsequent mobility lifetime. This is not needed if the PDCP anchor is fixed.

- Complexity 2:  all candidate gNBs need to keep updated on the current serving gNB (PDCP anchor) and involves lot signaling across configured gNBs. This is not needed if the PDCP anchor does not change.

- Complexity 3: Anchor switch-based solution causes lot of signaling overhead during ping-pong scenario between 2 neighbor gNBs.

Observation 3: Security handling during subsequent mobility becomes an issue only if the PDCP anchor is switched during inter-gNB LTM.

Observation 4: Notifying all candidate gNBs about any change of the current serving gNB becomes an issue only if the PDCP/RRC anchor is switched during inter-gNB LTM and this involves lot of signaling among candidate gNBs and RRC Reconfiguration towards UE, which is not aligned with WID objective of “aiming to avoid RRC Reconfiguration between cell switches as per Rel-18 LTM”.

Observation 5: Supporting subsequent inter-gNB LTM is easier if the PDCP/RRC anchor remains fixed rather than varying with each inter-gNB cell switch.

Based on the observations above, inter-gNB LTM with fixed RRC/PDCP anchor solution is more aligned with the scope of the Rel-19 WID compared to inter-gNB LTM with changing RRC/PDCP anchor.

Observation 6: Inter-gNB LTM with fixed RRC/PDCP anchor aligns more closely with Rel-19 WID compared to inter-gNB LTM with changing RRC/PDCP anchor solution.

# 3 NG-RAN architecture aspects

Contribution [7] makes an incorrect observation based on the assumption that inter-gNB F1 interfaces are necessary, as depicted in Figure 1 of the referenced document, and states that the PDCP anchor solution violates the current NG-RAN architecture:

“We believe that such abovementioned facts are a violation to the current NG-RAN architecture.”

Figure 1: Inter-gNB LTM with fixed PDCP/RRC anchor is transparent to gNB-DUs. No inter-gNB F1 interface is needed.

Figure 1 illustrates the scenario of two UEs:

- UE1 is initially connected to gNB1-DU and performs inter-gNB LTM cell switch to gNB2-DU but remains RRC-connected to gNB1-CU

- UE2 has always been connected to gNB2-DU and RRC-connected to gNB2-CU.

As illustrated in Figure 1, each of gNB1-DU and gNB2-DU establish a single F1 connection to gNB1-CU and gNB2-CU respectively. In the scenario of Figure 1, gNB2-DU is not aware that UE1 and UE2 are RRC-connected to different gNB-CUs. Instead, gNB2-DU forwards all signaling of UE1 and UE2 to gNB2-CU.

On the other hand, it is gNB2-CU who figures out that RRC signaling of UE1 shall be forwarded to gNB1-CU, whereas RRC signaling of UE2 terminates at gNB2-CU. This is feasible based on information exchange during LTM preparation b/w the two gNB-CUs, where gNB1-CU informs gNB2-CU during LTM preparation phase that UE1 will stay anchored at gNB1-CU in case an inter-gNB cell switch to gNB2-DU occurs in the future.

Observation 7: As in the current 5G RAN architecture, a gNB-DU establishes a single F1 connection to one gNB-CU and forwards signaling of all its served UEs to that gNB-CU:

- The gNB-DU is unaware of which, if any, of its served UEs are RRC-connected to a different gNB-CU than its controlling gNB-CU.

- The gNB-CU that controls the gNB-DU determines whether RRC signaling received from the gNB-DU should be retained or forwarded to another gNB-CU.

Therefore, no cross-gNB F1-C interface is needed to support inter-gNB LTM with fixed RRC/PDCP anchor.

Observation 8: Inter-gNB LTM with fixed RRC/PDCP anchor is compliant with the current NG-RAN architecture principles. No cross-gNB F1-C interface is needed between the source gNB-CU and target gNB-DU after LTM cell switch.

“Observation 2: The PDCP anchor based solution requires that a gNB-CU-UP serves UEs in the NR cells in another gNB that it does not support.”

Contribution [7] also makes the following observation:

In the GNB-CU-UP E1 SETUP REQUEST message, a gNB-CU-UP provides to the gNB-CU-CP its supported cells in NR CGI Support List IE. It is optional for the gNB-CU-UP to provide this information to the gNB-CU-CP, and it is optional for the gNB-CU-CP to consider this information during bearer context establishment, as the E1-AP specification TS 37.483 states: “If the NR CGI Support List or the Extended NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment”. This information is configured to the gNB-CU-UP by the OAM, and the OAM configuration can be extended.

Observation 9: Regarding the information of the NR cells that the source gNB-CU-UP serves, this list of cells can be extended by OAM configuration, and it is not mandatory for the source gNB-CU-CP to consider this information during bearer context establishment.

# 4 Signalling and standards impact

Contribution [7] makes the following observation:

“Observation 3: Supporting the PDCP anchor based solution would have a significant impact on the standard and require a substantial workload.”

In this section we show that the standard impact of inter-gNB LTM with fixed RRC/PDCP anchor is minimal and is less complex than RRC/PDCP anchor switch-based solution.

Reuse of Xn handover signalling

To support inter-gNB LTM with fixed RRC/PDCP anchor, several information in legacy Xn signaling need to be provided by the source gNB to the candidate gNB. E.g.:

- Inter node RRC HandoverPreparationInformation message, which is carried in the UE Context Information IE in the Xn Handover Request message. This is needed to notify the candidate gNB, e.g., of the UE capabilities and the source cell configuration to assist the candidate gNB to prepare the UE’s candidate lower layer configuration.

- DRB QoS + tunnel info for data forwarding, which is shared over legacy Xn signaling in several contexts such as NR-DC or SDT with partial context transfer. As we have discussed in our contribution [6], during LTM preparation, the source gNB-CU determines the QoS flow to DRB mapping and transmits a DRB to be Setup List IE in Handover Request message to the candidate gNB-CU, which is forwarded to the candidate gNB-DU by the candidate gNB-CU over F1. The candidate gNB-DU determines the list of accepted DRBs.

To support inter-gNB LTM with fixed RRC/PDCP anchor, several information in legacy Xn signaling need to be returned by the candidate gNB to the source gNB. E.g.:

- Lower layer configuration (CellGroupConfig), which can be wrapped into an RRCReconfiguration message similar to the legacy RRC HandoverCommand-IE in the Xn Handover Request ACK message

- Tunnel info for data forwarding, which is shared over legacy Xn signaling in several contexts such as NR-DC or SDT

Observation 10: Legacy Xn HO messages and IEs can be reused during LTM preparation to support inter-gNB LTM with fixed PDCP anchor.

Handling of NAS messages

As discussed in [4], NAS messages are carried as containers within RRC. Transport of RRC messages is transparent to NAS.

Observation 11: Exchange of NAS messages b/w the CN and the UE is transparent to the signalling path b/w the RRC-terminating gNB and the UE.

Signalling to support LTM configuration modification and LTM candidate cell cancellation functions

[7] states the following:

“new signalling/procedures are foreseen to support the interaction between the UE’s anchor gNB-CU and the candidate gNB-DU in another node to support the LTM configuration modification, LTM candidate cell cancellation functions, etc.”

Procedures such as modification or cancellation of LTM configuration will need to be developed anyway, whether inter-gNB LTM is performed with or without switch of PDCP anchor.

Observation 12: Procedures to modify or cancel LTM configurations will need to be developed for all inter-gNB LTM solution variants (fixed or varying PDCP/RRC anchor).

On the other hand, procedures to modify or cancel LTM candidate configurations require that the candidate gNB knows the current serving PDCP/RRC anchor. This is readily available in the case of fixed PDCP/RRC anchor. This becomes problematic in case the PDCP/RRC anchor changes since all candidate gNBs must be notified of each anchor switch after each inter-gNB cell switch, which involves lot of signalling among candidate gNBs and new anchor gNB.

Observation 13: Procedures to modify or cancel LTM configurations require that the candidate gNB knows the current RRC/PDCP anchor at all times since only the current RRC/PDCP anchor can execute the RRC change on the UE. This is much simpler if the RRC/PDCP anchor is fixed rather than switching RRC/PDCP anchor.

Need for anchor relocation

[7] argues that anchor relocation needs to be supported anyway, so support of fixed RRC/PDCP anchor is not needed. However, it should be noted that the fixed RRC/PDCP anchor and the varying RRC/PDCP anchor address complementary use cases, which means for various deployments either solution variant may be more favorable. This implies inter-gNB LTM with fixed RRC/PDCP anchor should still be supported as baseline.

Observation 14: Fixing or switching the RRC/PDCP anchor during inter-gNB LTM addresses complementary use cases.  Anchor RRC/PDCP based solution shall be supported as baseline in specification work and Anchor switching based solution serves as complementary to Anchor based solution.

Support for UE associated F1-AP and/or NG-AP procedures when the anchor gNB-CU communicates with the target gNB-DU after LTM cell switch and handling of positioning messages

[7] states that “in order for the anchor gNB-CU to communicate with the target gNB-DU after the UE’s cell switch, all the UE associated F1AP procedures and all the UE associated NGAP procedures impacting on gNB-DU have to be replicated into XnAP. This may lead to not only functional redundant and huge standard workload.”

The above statement is not correct, and the current specification already supports the analogous exchange of many UE-associated F1AP information over XnAP and NGAP. For example, UE associated messages received over NGAP which result in RRC configuration (e.g., MDT Configuration, QoE configuration) to a UE can be supported via already existing RRC Transfer in Xn and F1, thereby the anchor CU can send the MDT/QoE RRC configuration to the RLC-terminating DU.

Other messages that might have impact on the DU (listed below) are already supported over Xn and F1, thereby the anchor CU can still use the existing Xn and F1 messages to send those with the RLC-terminating DU. For example,

- Trace messages (e.g., Trace Start) is present in Xn and F1. Thereby, the anchor CU (upon receiving trace start from AMF) can still use the Xn and F1 Trace messages to send the trace to the RLC-terminating DU

- SON messages (e.g., Access and Mobility Indication, RACH Indication) is supported over both Xn and F1. Thereby, the anchor CU can still use the Xn and F1 messages to forward the SON reports to the RLC-terminating DU

- Other non-UE associated messages (e.g., MLB, ES) are all under the responsibility of the non-anchor CU (See section 6 for more details) and no impacts to standards are needed.

If there is anything missing and that needs to be added at stage-3 level, we are open to discuss. But from our analysis, most things are already supported.

Furthermore, the main benefits of the PDCP anchor solution, viz., no need for security key update, no PDCP re-establishment at LTM cell switch, and no need of path switch at LTM cell switch, etc. outweigh the signalling overhead cost to support a missing procedure, if any needed.

Observation 15: If the UE associated messages received over NGAP (e.g., MDT/QoE configurations) result in a RRC configuration towards a UE, this can be supported via already existing RRC Transfer in Xn and F1, thereby the anchor CU can send the MDT/QoE RRC configuration to the RLC-terminating DU.

Observation 16: Most messages that might have impact on the DU (e.g., Trace Start, Access and Mobility Indication, RACH Indication) are already supported over Xn and F1, thereby the anchor CU can still use the existing Xn and F1 messages to send those to the RLC-terminating DU. If there is anything missing and that needs to be added, we are open to discuss.

Observation 17: The current specifications already support signaling of most UE-associated NGAP messages over XnAP and F1AP as well.  Anchor CU can use the existing Xn/F1 messages to communicate with the RLC-terminating DU and no redundancy over Xn/F1 or “NO huge” standard workload is expected.

[7] states that: “it requires the anchor gNB-CU to translate NRPPa messages into F1AP messages and duplicate them on XnAP, which is also redundant and heavy on workload”.

The handling of positioning messages that affect the gNB-DU, e.g., TRP Information exchange, Positioning Activation, Positioning Deactivation, can be done the same way as discussed above for the UE associated NG-AP procedures that have impact on the gNB-DU.

Observation 18: The handling of positioning messages that affect the gNB-DU, e.g., TRP Information exchange, Positioning Activation, Positioning Deactivation, can be done the same way as the UE associated NG-AP procedures that have impact on the gNB-DU. No significant standard workload is foreseen.

# 5 Performance impact

[7] makes the following observation:

“Observation 4: The PDCP anchor based solution will introduce extra delay on both control plan and user plane on transmissions from/to the UE after cell switch. Which may lower down the user experience and performance and impact on the support of latency sensitive URLLC services.”

As discussed in our contribution [3], Xn latency can be quite minimal since such connections are carried over fiber and different operators have different transport network topologies, which is completely under operator control.

We also note that similar delays over Xn exist in MR-DC for MN terminated bearers with SCG RLC or for SN terminated bearers with MCG RLC, and since there are no restrictions or issues regarding supporting latency sensitive application traffic using such bearers, there should arise no issues regarding supporting latency sensitive application traffic after LTM cell switch, for the PDCP anchor solution. For L3 measurement report based LTM cell switch requires additional signalling hop between CU-CP and DU. One can argue that this will also have latency impact and but still considered to be supported. (UE reported L3 RRC Measurements reports will be received by CU-CP, and it has to be forwarded back to DU to make any Cell Switch decision).

Observation 19: The PDCP/RRC anchor solution introduces negligible delay on the control plane and the user plane for transmissions to and from the UE after an LTM cell switch and is dependent on specific operator transport network topology (Operators have full control over transport network topology design).

Observation 20: For the PDCP/RRC anchor solution, there should be no issues supporting latency-sensitive application traffic after an LTM cell switch. Similar delays over Xn exist in MR-DC for MN terminated SCG bearers and SN terminated MCG bearers, and for such bearers, there are no issues supporting latency-sensitive application traffic. Similar delay argument applies to L3 RRC Measurement Report triggered LTM cell swich but still it is supported because latency between CU-CP and DU transport is negligible.

[7] states that: “…the extra latency does not significantly impact latency-sensitive services by the example of MN terminated bearers with SCG RLC or SN terminated bearers with MN RLC in case of dual connectivity. However, this analogy is not correct. As dual connectivity is designed for user throughput boosting and transmission reliability. Apparently, the latency sensitive services can be transmitted via MCG/SCG only bearers simultaneously.” For URLLC, it is well known that PDCP packet duplication using DC is supported. Therefore, it is not true that latency sensitive services are carried over MCG/SCG only bearers.

# 6 Impact on RRM functions that are placed at the gNB-CU

In the below table, we provided responses to the arguments raised in [7] regarding the impact on RRM functions that are placed at the gNB-CU.

| Arguments in [7] | Response |
| --- | --- |
| The PDCP anchor based solution will impact negatively on the high layer RRM functions which are placed at the gNB-CU, like MRO, MLB, overload control, energy saving, etc. The main reason is that the RRC protocol and the corresponding high layer RRM functions for the UE are placed at the anchor gNB-CU, while the target gNB-DU hosting the real radio resources of the target cell serving the UE is controlled and managed by its controller gNB-CU. But the controller gNB-CU cannot control anything allocated to the UE in the target gNB-DU that is migrated into the target cell by the PDCP anchor based LTM solution. | This argument is not true. Even with the anchor based LTM solution, the non-anchor CU (or controller-CU as you refer it) still controls its served cells e.g., (de)activate its served cells, perform load balancing and can do this via existing procedures, To be clearer: - The non-anchor CU is still responsible for all “cell associated” functionalities (e.g., load balancing, cell (de) activation) of its served cells - The non-anchor CU is also responsible for the LTM configurations (e.g. preparing/cancelling/modifying) the LTM candidates and provide the LTM configurations to the anchor CU. - The anchor CU is only responsible for terminating the RRC signalling of the UE, and sending the RRC configurations (including the configurations of LTM candidate cells) to the UE Observation 21: As in legacy, the non-anchor gNB-CU has full control of RRM functions such as MRO, MLB, ES, etc. |
| For example, in section 8.2.7 in TS 38.473, it specifies that the F1-C interface support gNB-DU overload control function by the gNB-DU Status Indication function. Once received the overload indication from the gNB-DU, the gNB-CU may react to reduce the load in the gNB-DU, by avoiding handover more UEs into the overloaded gNB-DU, handover out some UEs to its neighbours, etc. But it cannot do anything to the UEs which is controlled by the other gNB-CU. Due to such distinction, it may also disable the gNB-CU to perform its load balancing strategies, cell switch off strategies, etc. | Again, your statement here is not correct. Please see below explanations. Mobility Load Balancing / Overload As mentioned above, the non-anchor CU is still responsible for load balancing of its served cells even if a UE is anchored at another CU. So, existing gNB-DU Status Indication can still be used by the non-anchor CU. Upon receiving such overload indication, the non-anchor CU can for instance initiate cancellation of the LTM candidate cell towards the anchor CU. In response, the anchor CU can handover the UE/update the LTM candidate configuration of the UE/etc. Observation 22: For MLB or upon receiving an overload indication from the gNB-DU, the non-anchor CU may initiate LTM modification/cancel procedure towards the anchor CU, which can update the UE’s LTM configuration, handover the UE, etc. Cell (de)activation Similar to load balancing, the non-anchor CU is still responsible for cell (de) activation of its served cells even if a UE is anchored at another CU. Therefore, the non-anchor CU can still perform appropriate cell switch off strategies, update LTM candidate cells based on the cell (de) activation status and inform the anchor CU using LTM modification/cancel procedures. Observation 23: For ES, the non-anchor CU may decide to deactivate a candidate cell and then initiate LTM cancel procedure towards the anchor CU. The anchor CU cancels the respective LTM candidate configuration on the UE and may handover the UE, following which the non-anchor CU can deactivate the LTM candidate cell as per legacy. |
| Particularly, in MRO, normally the RLF report will be delivered to the UE’s last serving node indicated by the NR CGI in the report for root analysis. In the scenario here, the RLF report will be forwarded to the controller gNB-CU rather than the anchor gNB-CU. This may mislead the MRO algorithm in the controller gNB-CU, because it is the anchor gNB-CU who made the handover decision for the UE. | This statement is not true. Existing RLF Report forwarding mechanisms still apply (and no difference in the way CGI is encoded as well). For example, consider a scenario where RLF occurred during LTM cell switch from cell 1 served by DU1 to cell 2 served by DU2, and the anchor is kept the same at CU1. As in legacy, RLF Report will include the CGI: - CGI of source cell = Cell 1 ID + CU1 ID - CGI of target cell = Cell 2 ID + CU2 ID RLF Report will still be forwarded from the retrieving node to UE’s last serving node as indicated by the CGI of source cell, i.e., CU1. In summary, no impacts to MRO. Observation 24: No impact to MRO during inter-gNB LTM with fixed PDCP anchor. RLF reports are always forwarded to the right gNB-CU based on the CGI of the source cell. |

# 7 Impact on OAM NRM

The observation in [7] on this issue is:

“Observation 6:  SA5 needs to be involved to evaluate the impact on the current NRM of the gNB for the PDCP anchor based solution, for example, managing the Cell Relations of a cell by a gNB-CU not controlling the cell.”

Specifically, the following is mentioned in [7]

The IOC of NRCellRelation represents a neighbour cell relation from a source cell to a target cell. And the source cell can be a NRCellCU instance.

Based on above interpretation, we may conclude that one gNB-CU-CP can only manage the neighbour cell relations of which the source cell is under the control of it.

While the PDCP anchor based solution may need one gNB-CU-CP to manage and maintain the cell relationship for cells that are not controlled by the gNB-CU-CP which seems contradicting to the current NRM in SA5 specification.

The above statement is not true. As in legacy, every CU is responsible for managing its own cell relations, even with the PDCP/RRC anchor based solution. There is no need for anchor CU to manage the neighbour cell relations of the cells controlled by non-anchor CU.

Also, over Xn, NG-RAN1 can already exchange the served cell information and neighbour cell information to NG-RAN2 over Xn Setup and NG-RAN Config Update procedures, that can be used by NG-RAN2 for several purposes e.g., operating its served cells and/or for knowing the neighbors of NG-RAN1. Therefore, no impact is foreseen with the PDCP/RRC anchor based solution and existing information over Xn can be reused for cell relations.

Observation 25: As in legacy, every CU is responsible for managing its own cell relations, even with the PDCP/RRC anchor based solution. There is no need for anchor CU to manage the cell relations of the controlling CU.

Observation 26: Neighbour cell information is already exchanged over Xn and can be reused by the anchor CU to know the neighbours of controlling CU. No new impact is foreseen with the PDCP/RRC anchor based solution for maintaining/knowing cell relations.

Observation 27: There is no impact to NRM of the gNB (and SA5 need not be involved) for the PDCP/RRC based anchor-based solution, for example, for managing the Cell Relations.

# 8 Impact on functions in the core network

[7] makes the following observation:

“Observation 7:  Evaluation should be done in SA2, CT1 and CT4 to prove if any impact on core network functions when the AMF receives a NR CGI in the ULI which does not belong to the reporting gNB.”

There are several precedents where a gNB reports ULI of a cell not served by that gNB. One example is DC in which the MN reports ULI associated with the PSCell of the SN. Another example is mIAB in which the F1-terminating donor reports ULI of the mIAB-MT served by a separate RRC-terminating donor. In a similar manner, the PDCP/RRC anchor can report the ULI of the cell served by the non-anchor gNB.

Observation 28: NR-DC and mIAB set precedents for reporting ULI by one gNB for cells served by a different gNB. Similarly, the PDCP-terminating gNB may report ULI of LTM cells served by the RLC-terminating gNB.

# 9 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations.

Alignment with the Rel-19 WID

Observation 1: Rel-18 LTM features are sufficient for the UE to perform inter-gNB LTM without switching the PDCP/RRC anchor. This is not the case if the PDCP/RRC anchor needs to be switched (i.e., in case of Anchor switching based solution).

Observation 2: Inter-gNB LTM without switching the PDCP/RRC anchor is transparent to the gNB-DU serving the UE. All Rel-18 F1 LTM procedures can be reused.

Observation 3: Security handling during subsequent mobility becomes an issue only if the PDCP anchor is switched during inter-gNB LTM.

Observation 4: Notifying all candidate gNBs about any change of the current serving gNB becomes an issue only if the PDCP/RRC anchor is switched during inter-gNB LTM and this involves lot of signaling among candidate gNBs and RRC Reconfiguration towards UE, which is not aligned with WID objective of “aiming to avoid RRC Reconfiguration between cell switches as per Rel-18 LTM”.

Observation 5: Supporting subsequent inter-gNB LTM is easier if the PDCP/RRC anchor remains fixed rather than varying with each inter-gNB cell switch.

Observation 6: Inter-gNB LTM with fixed RRC/PDCP anchor aligns more closely with Rel-19 WID compared to inter-gNB LTM with changing RRC/PDCP anchor solution.

NG-RAN architecture aspects

Observation 7: As in the current 5G RAN architecture, a gNB-DU establishes a single F1 connection to one gNB-CU and forwards signaling of all its served UEs to that gNB-CU:

- The gNB-DU is unaware of which, if any, of its served UEs are RRC-connected to a different gNB-CU than its controlling gNB-CU.

- The gNB-CU that controls the gNB-DU determines whether RRC signaling received from the gNB-DU should be retained or forwarded to another gNB-CU.

Observation 8: Inter-gNB LTM with fixed RRC/PDCP anchor is compliant with the current NG-RAN architecture principles. No cross-gNB F1-C interface is needed between the source gNB-CU and target gNB-DU after LTM cell switch.

Observation 9: Regarding the information of the NR cells that the source gNB-CU-UP serves, this list of cells can be extended by OAM configuration, and it is not mandatory for the source gNB-CU-CP to consider this information during bearer context establishment.

Reuse of Xn handover signalling

Observation 10: Legacy Xn HO messages and IEs can be reused during LTM preparation to support inter-gNB LTM with fixed PDCP anchor.

Handling of NAS messages and positioning messages

Observation 11: Exchange of NAS messages b/w the CN and the UE is transparent to the signalling path b/w the RRC-terminating gNB and the UE.

Signalling to support LTM configuration modification and LTM candidate cell cancellation functions

Observation 12: Procedures to modify or cancel LTM configurations will need to be developed for all inter-gNB LTM solution variants (fixed or varying PDCP/RRC anchor).

Observation 13: Procedures to modify or cancel LTM configurations require that the candidate gNB knows the current RRC/PDCP anchor at all times since only the current RRC/PDCP anchor can execute the RRC change on the UE. This is much simpler if the RRC/PDCP anchor is fixed rather than switching RRC/PDCP anchor.

Need for anchor relocation

Observation 14: Fixing or switching the RRC/PDCP anchor during inter-gNB LTM addresses complementary use cases.  Anchor RRC/PDCP based solution shall be supported as baseline in specification work and Anchor switching based solution serves as complementary to Anchor based solution.

Support for UE associated F1-AP and/or NG-AP procedures when the anchor gNB-CU communicates with the target gNB-DU after LTM cell switch

Observation 15: If the UE associated messages received over NGAP (e.g., MDT/QoE configurations) result in a RRC configuration towards a UE, this can be supported via already existing RRC Transfer in Xn and F1, thereby the anchor CU can send the MDT/QoE RRC configuration to the RLC-terminating DU.

Observation 16: Most messages that might have impact on the DU (e.g., Trace Start, Access and Mobility Indication, RACH Indication) are already supported over Xn and F1, thereby the anchor CU can still use the existing Xn and F1 messages to send those to the RLC-terminating DU. If there is anything missing and that needs to be added, we are open to discuss.

Observation 17: The current specifications already support signaling of most UE-associated NGAP messages over XnAP and F1AP as well.  Anchor CU can use the existing Xn/F1 messages to communicate with the RLC-terminating DU and no redundancy over Xn/F1 or “NO huge” standard workload is expected.

Observation 18: The handling of positioning messages that affect the gNB-DU, e.g., TRP Information exchange, Positioning Activation, Positioning Deactivation, can be done the same way as the UE associated NG-AP procedures that have impact on the gNB-DU. No significant standard workload is foreseen.

Performance impact

Observation 19: The PDCP/RRC anchor solution introduces negligible delay on the control plane and the user plane for transmissions to and from the UE after an LTM cell switch and is dependent on specific operator transport network topology (Operators have full control over transport network topology design).

Observation 20: For the PDCP/RRC anchor solution, there should be no issues supporting latency-sensitive application traffic after an LTM cell switch. Similar delays over Xn exist in MR-DC for MN terminated SCG bearers and SN terminated MCG bearers, and for such bearers, there are no issues supporting latency-sensitive application traffic. Similar delay argument applies to L3 RRC Measurement Report triggered LTM cell swich but still it is supported because latency between CU-CP and DU transport is negligible.

Impact on RRM functions that are placed at the gNB-CU

Observation 21: As in legacy, the non-anchor gNB-CU has full control of RRM functions such as MRO, MLB, ES, etc.

Observation 22: For MLB or upon receiving an overload indication from the gNB-DU, the non-anchor CU may initiate LTM modification/cancel procedure towards the anchor CU, which can update the UE’s LTM configuration, handover the UE, etc.

Observation 23: For ES, the non-anchor CU may decide to deactivate a candidate cell and then initiate LTM cancel procedure towards the anchor CU. The anchor CU cancels the respective LTM candidate configuration on the UE and may handover the UE, following which the non-anchor CU can deactivate the LTM candidate cell as per legacy.

Observation 24: No impact to MRO during inter-gNB LTM with fixed PDCP anchor. RLF reports are always forwarded to the right gNB-CU based on the CGI of the source cell.

Impact on OAM NRM

Observation 25: As in legacy, every CU is responsible for managing its own cell relations, even with the PDCP/RRC anchor based solution. There is no need for anchor CU to manage the cell relations of the controlling CU.

Observation 26: Neighbour cell information is already exchanged over Xn and can be reused by the anchor CU to know the neighbours of controlling CU. No new impact is foreseen with the PDCP/RRC anchor based solution for maintaining/knowing cell relations.

Observation 27: There is no impact to NRM of the gNB (and SA5 need not be involved) for the PDCP/RRC based anchor-based solution, for example, for managing the Cell Relations.

Impact on functions in the core network

Observation 28: NR-DC and mIAB set precedents for reporting ULI by one gNB for cells served by a different gNB. Similarly, the PDCP-terminating gNB may report ULI of LTM cells served by the RLC-terminating gNB.

# References

[1] R3-241532, RAN3 #123-bis, Design considerations to support inter-gNB LTM HO, Qualcomm Incorporated.

[2] R3-243522, RAN3 #124, Inter-gNB LTM and keep PDCP anchor, Qualcomm Incorporated, NTT DOCOMO.

[3] R3-244228, RAN3 #125, Inter-gNB LTM with no change of RRC/PDCP anchor, Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti AirTel (India), Sony.

[4] R3-245314, RAN3 #125-bis, Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes, Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India).

[5] R3-245315, RAN3 #125-bis, Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM, Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India).

[6] R3-245313, RAN3 #125-bis, Signalling enhancements for inter-CU LTM HO, Qualcomm Incorporated, Vodafone, Bharti Airtel (India).

[7] R3-245201, RAN3 #125-bis, Discussion on the PDCP anchor based solution, Huawei, Ericsson, CATT, China Telecom, CMCC, Samsung.

[8] R3-245585, RAN3 #125-bis, Discussion on inter-CU LTM non-DC case, NTT DOCOMO.