3GPP TSG-RAN WG3 Meeting #125bis	R3-245685

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	8.3

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # 3_SDTNetworkBasedSolution

Document for:	Approval

- Introduction

CB: # 3_SDTNetworkBasedSolution

- Check the issues raised online towards network based solution

(moderator - ZTE)

Summary of offline disc R3-245685

- For the Chairman’s Notes

No consensus on whether MAC reset is needed for network-based solution is FFS.

No issue on AMF redirecting NG connections (i.e., NG connection from old gNB to new gNB), reuse the legacy procedure. Call flow to be added

No issue for DL data loss by data forwarding

Scenario Limitation

Not suitable for UL data/signalling case

Not suitable for DL MBS configuration.

To be continue…

- Discussion-First round

  - Background

The following is abstracted from online discussion.

Any issue towards network based solution? - When CN establish the new UE context, how to release the old UE context? - DL Data loss issue? Nok: The CN will release the old UE context as specified in SA2 ZTE: How RAN node can aware the relationship between the old UE context and new UE context HW: There should be no UE and CN impact for network based solution, the issues raised in ZTE’s paper need to be further checked CATT: Legacy NAS recovery can be used. DL data forwarding needs to be checked. LG: Data loss issue to be checked QC, Lenovo: There is no answer in RAN2 whether MAC CE reset is always needed or not. Considering the complexity, such kind of solution is not pressured in R18. Do not see too much benefits compared with the complexity.

  - RAN2 progress

The following is from RAN2’s agreement and incoming LS.

In the RAN2 #127, RAN2 agreed to send a reply LS R2-2407816/R3-245012 to RAN3, as below.

R2-2407816 Reply LS on SDT signalling optimization for partial context transfer 1. Overall Description: RAN2 would like to thank RAN3 for the reply LS on SDT signalling optimization for partial context transfer in R2-2404123 (R3-242198). With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that: - If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost. - If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

Meanwhile, for this topic, RAN2 also made the following agreement in their Chairman’s note.

Agreements on the LS in addition to what is already agreed in the LS from last meeting

1	Acknowledge that there is an issue for the cases that there is some data in the buffer which causes dy-synch issues.  RAN2 discussed how to solve the issue and identified two possible ways to handle this: potentially a MAC reset may be needed or NW implementation can handle it.   If a MAC reset is needed, this would be a non-backward compatible change (i.e. not desirable), and some companies are not sure whether NW implementation can handle this issue.

Observation 1: RAN2 agrees that UE will not reset MAC for network-based solution.

Observation 2: RAN2 are not sure whether network implementation can handle this without MAC reset.

Observation 3: RAN2 acknowledge that there is an issue for the cases that there is some data in the buffer which causes dy-synch issues.

Question 1: Whether it is feasible and how does RAN3 resolve the de-synch issues, in case that MAC reset is not performed at UE, excluding split gNB deployment?

| Companies | Yes/No | Comments |
| --- | --- | --- |

No consensus on whether MAC reset is needed for network-based solution is FFS. To be continue…

  - Issues raised from R3-245677

In R3-245677, it provides an overall procedure for the network-based solution.

Figure 1. Overall procedure for Network-based solution

[TS 38.300] Figure 9.2.1.3-1: UE triggered transition from RRC_IDLE to RRC_CONNECTED 1. The UE requests to setup a new connection from RRC_IDLE. 2/2a. The gNB completes the RRC setup procedure. NOTE: The scenario where the gNB rejects the request is described below. 3. The first NAS message from the UE, piggybacked in RRCSetupComplete, is sent to AMF. 4/4a/5/5a. Additional NAS messages may be exchanged between UE and AMF, see TS 23.502 [22]. 6. The AMF prepares the UE context data (including PDU session context, the Security Key, UE Radio Capability and UE Security Capabilities, etc.) and sends it to the gNB. 7/7a. The gNB activates the AS security with the UE. 8/8a. The gNB performs the reconfiguration to setup SRB2 and DRBs for UE, or SRB2 and optionally DRBs for IAB-MT. 9. The gNB informs the AMF that the setup procedure is completed. NOTE 1: RRC messages in step 1 and 2 use SRB0, all the subsequent messages use SRB1. Messages in steps 7/7a are integrity protected. From step 8 on, all the messages are integrity protected and ciphered. NOTE 2: For signalling only connection, step 8 is skipped since SRB2 and DRBs are not setup.

For the network-based solution, we do not want to modify both UE and CN behaviour, so that, after receiving RRCSetup (step: 8b in Figure 1 and step 2 in Figure 9.2.1.3-1) message from gNB, it shall send RRCSetupComplete message (step 2a) to the gNB including NAS PDU. Then, the gNB shall send NG: Initial UE context message to AMF including NAS PDU.

Observation 1: After receiving NAS-PDU (including e.g., Service request), the AMF shall establish a new NG connection between new gNB (i.e., receiving gNB) and CN, and new UE context for the UE.

Observation 2: Meanwhile, the AMF has already established the old NG connection between old gNB (i.e., last serving gNB) and old UE context for the same UE.

Question 2: Whether any issue with AMF redirecting NG connections (i.e., NG connection from old gNB to new gNB)?

| Companies | Yes/No | Comments |
| --- | --- | --- |

OK, no issue reuse the legacy procedure. Call flow to be added

If AMF triggers to release the old connection and old UE context, how RAN guarantee that DL data buffered at old gNB will not be lost?

- After receiving NG: Initial UE context Setup message, the new gNB shall establish the same DRBs based on the stored UE context.

- The new DRB’s SN (sequence number) shall reuse the old DRB’s SN other than SN=0.

Question 3: Whether and how RAN avoid DL data loss?

| Companies | Yes/No | Comments |
| --- | --- | --- |

OK, no issue for DL data loss

Question 4: How to UL data loss?

FFS.

  - Issues raised from R3-245046

The following is abstracted from R3-245046 and R3-245677

- Scenario Limitation Not suitable for UL data/signalling case Not suitable for DL MBS configuration. - NGAP procedure/signalling involved Introduce additional NGAP signalling, to discard the current INACTIVE AS context and AS security context and to establish AS context and AS security context again Reuse the legacy NGAP signalling, but introduce more latency. - Impact on network’s behaviour and procedure/signalling - Extend network behaviour: use the old UE context to establish the same DRBs. - Extend network behaviour: XnAP Retrieve UE context response message instead of Retrieve UE context Failure message - Extend network behaviour: F1AP UE context modification message instead of UE context setup message - Enhance XnAP Retrieve UE context response message, to end ongoing SDT - Enhance F1AP UE context modification message, to release SDT bearer and to establish non-SDT bearer - Impact on RRCSetup without MAC reset Resolve MAC de-synch issue

- Conclusion, Recommendations

- References