3GPP TSG-RAN WG3 Meeting #125bis	R3-245827

Hefei, China, 14th – 18th October 2024

Agenda Item:	16.3

Source:	Nokia, ZTE

Title:	[TP for TR 38.769] TP for Locating Ambient IoT Devices

Document for:	Discussions & Approval

# 2	Text Proposal for TR 38.769 v1.0.0

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

### 6.8.2	Topology 1

For support of AIoT device location, the AIoT RAN node may report an identifier of the reader in the Inventory Report message to the AIoT CN and the AIoT CN knows the location of the identified reader.

Editor’s note 3: Definition of reader and format of the identifier of a reader is FFS.

Editor’s note 4: Report in further other messages is FFS.

The AIoT CN knows the location of a reader supported by an AIoT RAN node by one of the following methods:

(a) the AIoT CN is configured by O&M about the readers supported by an AIoT RAN node and their location.

(b) the AIoT CN receives the list of readers supported by an AIoT RAN node and their location in an XXAP Setup/Configuration Update message received beforehand.

Editor’s note 5: Which method is used is FFS.

### 6.8.3	Topology 2

For support of AIoT device location, information is reported to the AIoT CN in the Inventory Report to enable the AIoT CN to determine the location of the AIoT_enabled UE Reader to which the AIoT device responded.

Editor’s note 6: Report of such information in further other messages is FFS.

The information enabling the AIoT CN to determine the location of the AIoT_enabled UE Reader to which the AIoT device responded to the inventory may be one or more of the following:

(a) the AIoT_enabled UE Reader knows its location (e.g. if AIoT_enabled UE has GNSS info) and this is reported to the AIoT CN in the Inventory Report.

(b) an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN has determined the location of the identified UE using legacy LCS procedure(s).

(c) an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN has determined the location of the identified UE via User location information reported by the AIoT_enabled gNB (e.g. in almost all UL NGAP messages).

Editor’s note 7: Selection of one or more of (a), (b), (c) is FFS.

Editor’s note 8: Introduction of additional solutions FFS.

Editor’s Note 9: Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 10: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” for purpose (b) needs further study.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>